3GPP TSG-RAN WG4 Meeting #116bis	R4-2513437

# Prague, CZ, 13 - 17 Oct, 2025

Title: 	Discussion on NTN testing for NGSO demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	6.6.3.3

Document for:	Discussion

Background

During RAN4#116 meeting, Way Forward [1] for NTN_testing_NGSO_channel_model_and_demod was approved. In this contribution, we share our view on NTN testing for NGSO demodulation performance requirements.

Discussion

IoT NTN demodulation requirement with applying time-varying channel model

- agreement - RAN4 can take the residual frequency offset as 200Hz and time offset error as to check the demodulation requirement impact with applying time-varying doppler shift and propagation delay channel model for IoT NTN - For Category M1 - Table 7.24A.2-1: Te_NTN_M1 Timing Error Limit CE Mode; Te_NTN_M1_. A; 41*TS (=1.33 us). Note 1: TS is the basic timing unit defined in TS 36.211 4.1. Note 2: This requirement applies regardless of the downlink carrier bandwidth.. - For NB-IoT - Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS (=3.16us). Note 1: TS is the basic timing unit defined in TS 36.211 4.1. - Further discuss whether to add the additional margin based on simulation results in the next meeting

To align with NR NTN PDSCH, we propose to add 0.5 dB margin for IoT NTN PDSCH requirements.

Add 0.5 dB margin for IoT NTN PDSCH requirements.

Sampling frequency offset impact for testing

- agreement - Interesting companies including UE and TE vendors are encouraged to provide analysis and evaluation for the impact of sampling frequency offset in related to both the Doppler frequency and its rate of change (i.e., Doppler drift) caused by the time-varying Doppler shift model for NGSO. - FFS how and where to capture the impact of sapling frequency offset if identified

From our understanding, the sampling frequency offset impact is up to implementation and can be covered by the 0.5 dB margin added for the time-varying Doppler shift model for NGSO. It is no need to add more margin.

The sampling frequency offset impact is up to implementation and can be covered by the 0.5 dB margin added for the time-varying Doppler shift model for NGSO. It is no need to add more margin.

Test applicability of requirement with applying time-varying channel model

- WF - Option 1: - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration. - Option 2 - For Rel-19 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18. - For Rel-17/Rel-18 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18. - Other options are not precluded

From our understanding, the new NGSO requirements should applied only for Rel-19 UE. If UE can pass new NGSO requirements applying the new channel model, then it can skip the corresponding NGSO requirements instead of all NGSO requirements defined in Rel-17/Rel-18. Because only two test cases are selected from the NGSO requirements defined in Rel-17/Rel-18 for new NGSO requirements definition with timing varying channel model and additional 0.5dB margin added, the others are still applicable for testing with NTN-TDL channel model without additional 0.5dB margin added.

The new NGSO requirements should applied only for Rel-19 UE. If UE can pass new NGSO requirements applying the timing varying new channel model, then it can skip the corresponding NGSO requirements defined in Rel-17/Rel-18. All other test cases defined in Rel-17/18 are only applicable for testing under NTN-TDL channel model.

Some company proposed to test all NTN requirements including Rel-17/18/19 under timing varying channel model, but the Rel-17/18 NGSO requirements are defined under NTN-TDL channel modeling, it means that will requires RAN4 to re-simulate all Rel-17/18 test cases under timing varying channel model, this will be very heavy workload for RAN4 considering the parallel discussion for NR Rel-19/Rel-20 and 6GR study for demodulation requirements. It is due to the workload concerns and consideration the key motivation for this Rel-19 NGSO testing WI is to test that UE can properly pre-compensate the frequency offset shift and timing offset shift, the selected RF frequency error test, RRM UL timing test and limited number of demodulation test cases are enough to verify the UE’s pre-compensation on the frequency offset shift and timing offset shift under timing varying channel model.

Observation 1: RAN4 needs to re-simulate all Rel-17/18 demodulation requirements with the timing varying channel model that will be very heavy workload for RAN4, with also consideration of the parallel discussion for NR Rel-19/Rel-20 and 6GR study for demodulation requirements.

Observation 2: The selected RF frequency error tests, RRM UL timing tests and limited number of demodulation test cases are enough to verify the UE’s pre-compensation on the frequency offset shift and timing offset shift under timing varying channel model as per the discussion on this WI [2].

Now it is in the late stage of Rel-19, some NTN UEs already finished some Rel-17/18 NTN testing, it means that both UE and TE vendors already support the NTN testing with NTN-TDL channel emulated, there are no technical feasibility issues to conduct the NGSO testing under NTN-TDL channel model.

Not allow Rel-17/18 UE to test the NGSO requirements under timing varying channel model, considering the heavy workload to re-simulate all test cases for RAN4 demodulation requirements, the selected RF/RRM/Demodulation tests can well meet the test purpose and no technical feasibility issues to conduct the NGSO testing under NTN-TDL as per the Rel-17/18 NTN NGSO testing in the market.

Test feasibility

As per the test procedure defined in RAN5 test specification TS 38.521-5 section 8.2.1.2.2.1.1_1.3:

5. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 8.2.1.2.2.1.1_1.3.3. 8.2.1.2.2.1.1_1.3.2 Test procedure 1. UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means. 2 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508[12] table 5.6.2.1-2 for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in clause 8.2.1.2.2.1.1_1.3.3. 3. Deactivate UE prediction of satellite trajectory by any preconfigured means. 4. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables Table 8.2.1.2.2.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. 5. Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 as appropriate. 6. Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to [Annex G]. 4. Repeat steps from 1 to 3 for each subtest in Table 8.2.1.2.2.1.1-3 as appropriate.

The demodulation requirements are defined with the starting elevation angle 30°assumed to verify the case of the largest delay shift.

From the test step yellow highlighted, we can know that UE firstly needs to enter into the RRC_CONNECTED state no later than elevation angle 30°, and the test needs to be repeated for many times. If the timing varying channel model is applied, it means that UE needs to perform the initial access procedure and ensures to enter into the RRC_CONNECTED state no later than elevation angle 30°for each subtest, the satellite needs to disappear suddenly and appears again with the Doppler shift and delay shift jumping during the testing. This will result in complex test setup, long test time and serious UE power consumption, this test feasibility and complexity need to be carefully studied.

Observation 3: Complex test setup should ensure UE to enter into the RRC_CONNECTED state no later than elevation angle 30°for each subtest to correctly test the UE performance under the largest delay shift at elevation angle 30°as defined in RAN4.

Further discussion is needed on how to ensure the correct testing under timing varying channel model considering that UE needs to perform the initial access procedure and ensures UE to enter into the RRC_CONNECTED state no later than elevation angle 30°for each subtest during the testing.

Proposal

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Add 0.5 dB margin for IoT NTN PDSCH requirements.

- The sampling frequency offset impact is up to implementation and can be covered by the 0.5 dB margin added for the time-varying Doppler shift model for NGSO. It is no need to add more margin.

- The new NGSO requirements should applied only for Rel-19 UE. If UE can pass new NGSO requirements applying the timing varying new channel model, then it can skip the corresponding NGSO requirements defined in Rel-17/Rel-18. All other test cases defined in Rel-17/18 are only applicable for testing under NTN-TDL channel model.

RAN4 needs to re-simulate all Rel-17/18 demodulation requirements with the timing varying channel model that will be very heavy workload for RAN4, with also consideration of the parallel discussion for NR Rel-19/Rel-20 and 6GR study for demodulation requirements.

The selected RF frequency error test, RRM UL timing test and limited number of demodulation test cases are enough to verify the UE’s pre-compensation on the frequency offset shift and timing offset shift under timing varying channel model as per the discussion on this WI [2].

Not allow Rel-17/18 UE to test the NGSO requirements under timing varying channel model, considering the heavy workload to re-simulate all test cases for RAN4 demodulation requirements, the selected RF/RRM/Demodulation tests can well meet the test purpose and no technical feasibility issues to conduct the NGSO testing under NTN-TDL as per the Rel-17/18 NTN NGSO testing in the market.

Complex test setup should ensure UE to enter into the RRC_CONNECTED state no later than elevation angle 30°for each subtest to correctly test the UE performance under the largest delay shift at elevation angle 30°as defined in RAN4.

- Further discussion is needed on how to ensure the correct testing under timing varying channel model considering that UE needs to perform the initial access procedure and ensures UE to enter into the RRC_CONNECTED state no later than elevation angle 30°for each subtest during the testing.

Reference

R4-2512525, Way Forward for [116][323] NTN_testing_NGSO_channel_model_and_demod, RAN4#116, Samsung

RP-252076, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, RAN#109