3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513487

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on Event triggered L1 measurement reporting

Source: 	Huawei, HiSilicon

Agenda item:	4.23.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on Event triggered L1 measurement reporting.

Discussion

One maintenance issue in [2] is to be discussed in the following.

- Issue 2-4: Impact on DL synchronization Candidate options: - Option 1: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) Recommended WF: - Discuss the option.

As per the proposal, the potential changes on LTM cell switch delay are proposed as below [3]

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - the UE has sent an event-triggered L1 report within 160 ms before receiving the cell switch command, or

The new condition above is a case that although the L1 measurement period of the SSB associated to target TCI state is larger than 160 ms after the TCI state activation MAC-CE is received, the L1 event triggered report is sent within 160ms before receiving cell switch command. As we know, the following Ax events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events are specified in RAN2:

-	 Event A1beam: Beam of serving cell becomes better than absolute threshold;

-	 Event A2beam: Beam of serving cell becomes worse than absolute threshold;

-	 Event A3beam: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	 Event A4beam: Beam of candidate cell becomes better than absolute threshold;

-	 Event A5beam: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Taking Event A3/A5 as an example, the triggered report instance is not always the moment when the L1 measurement on the candidate cell is executed. UE can perform Ax event evaluation when every latest measurement result (on candidate cell or on serving cell) is acquired. Therefore the event trigger report within 160ms before cell switch command may be triggered by the new measurement result from serving cell, and it doesn’t represent that the fine timing tracking on candidate cell is performed within 160ms. Therefore we don’t think it is reasonable to add the proposed condition in [3] as one condition for Tfirst-RS and TRS-proc = 0.

Proposal 1: No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command doesn’t represent that the fine timing tracking by L1 measurement on candidate cell is performed within 160ms.

Conclusion

The contribution provides analysis on Event triggered L1 measurement reporting. The following proposal is proposed:

Proposal 1: No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command doesn’t represent that the fine timing tracking by L1 measurement on candidate cell is performed within 160ms.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2509066, Topic summary for [116][223] NR_Mob_Ph4_Part1, Apple

- R4-2509161, DraftCR for LTM event-triggered L1 reporting, Nokia