3GPP TSG-RAN WG4 Meeting #116bis	R4-2513491

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on performance requirements of Conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	6.17.2.3

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

The contribution provides preliminary discussion on the performance requirements for R19 mobility.

Discussion

As per the WID [1], the scope of CLTM is confined to the scenario where the UE is in non-DC. So RAN4 don’t define test cases for conditional PSCell switch.

Proposal 1: Define test cases in non-DC and don’t define test cases for conditional PSCell switch.

According to core requirements of CLTM, both intra-frequency, inter-frequency target cell have requirements. And both RACH and RACH-less schemes are included as well. Therefore the below proposal are proposed.

Proposal 2: For conditional LTM, the following test case category are to be specified for FR1-FR1 and FR2-FR2:

- RACH based Intra-frequency conditional PCell switch

- RACH based Inter-frequency conditional PCell switch

- RACH less Intra-frequency conditional PCell switch

- RACH less Inter-frequency conditional PCell switch

Regarding subsequent CLTM, the similar tests can be specified as well.

Proposal 3: For subsequent conditional LTM, the following test case category are to be specified:

- RACH based Intra-frequency subsequent conditional PCell switch

- RACH based Inter-frequency subsequent conditional PCell switch

- RACH less Intra-frequency subsequent conditional PCell switch

- RACH less Inter-frequency subsequent conditional PCell switch

Given the fact that the subsequent CLTM includes a complete CLTM procedure, we suggest that the UE who pass subsequent CLTM tests can skip CLTM tests.

Proposal 4: UE who pass subsequent CLTM tests can skip CLTM tests.

Conclusion

The contribution provides analysis on performance part. The following proposals are proposed:

Proposal 1: Define test cases in non-DC and don’t define test cases for conditional PSCell switch.

Proposal 2: For conditional LTM, the following test case category are to be specified for FR1-FR1 and FR2-FR2:

- RACH based Intra-frequency conditional PCell switch

- RACH based Inter-frequency conditional PCell switch

- RACH less Intra-frequency conditional PCell switch

- RACH less Inter-frequency conditional PCell switch

Proposal 3: For subsequent conditional LTM, the following test case category are to be specified:

- RACH based Intra-frequency subsequent conditional PCell switch

- RACH based Inter-frequency subsequent conditional PCell switch

- RACH less Intra-frequency subsequent conditional PCell switch

- RACH less Inter-frequency subsequent conditional PCell switch

Proposal 4: UE who pass subsequent CLTM tests can skip CLTM tests.

Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4