3GPP TSG-RAN WG4 Meeting #116bis	R4-2513502

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	On LP-WUR maintenance issues

Agenda Item:	4.22.3

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The initial discussion for this topic was initially discussed at RAN4 110bis meeting. The WI was officially closed at RAN4 116 meeting and the topic summary and WF can be found at [5].

Discussion

Issue 1-5-4: LR based RRM when stationary or low-mobility criteria is configured for power saving feature

- Proposals

  - P1: In core maintenance phase, RAN4 to discuss the case when low mobility criteria is configured for UE power saving but not met in the LP-WUR based RRM case#1 and case#3. (Apple)

  - P2: RAN4 to discuss the UE behaviour when UE detects to meet Rel-16 low mobility criteria and Rel-19 RRM relaxation criteria. (Ericsson)

Recommendations:

At RAN2 131 meeting there is no conclusion on whether the low-mobility criteria will be used for LP-WUR or not. From RAN4 requirement perspective if RAN2 determines to use low-mobility criteria, then it is similar to Rel-16 UE power saving WI where two criteria are designed and RAN4 needs to define corresponding requirement for each of them and also may need define requirement when two criteria are both satisfied. At RAN2 131 meeting, the following agreement was achieved:

RAN2 assumes UE low mobility criterion is not included for Rel-19 LP-WUS RRM relaxation/offloading mode.

Based on RAN2 agreement, there is no need to have discussion on the impact of low mobility criteria.

- There is no need to have discussion on the impact of low mobility criteria.

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met

- Proposals:

  - P1: When UE exits the serving cell offloading case, the following approaches can be considered for UE behavior: (oppo)

    - Option 1: UE follow relaxed measurement and meet the corresponding requirements.

    - Option 2: UE follow legacy measurement and meet the corresponding requirements.

    - Option 3: Up to UE implementation and no requirements apply.

  - P2-1: When a UE leaves fully offloading state due to serving cell quality, the MR will consecutive measure a few (5) SSBs or consecutively measures SSBs within a transition period (100ms), then the MR will follow either relaxed measurement requirement or legacy measurement requirement (vivo)

  - P2-2: RAN4 to define requirements on MR wake-up delay for cases where UE determines LR exit condition is met. The wake-up delay for cases where UE detects LP-WUS is re-used (Huawei)

  - P2-3: After MR wake up, MR follows SSB periodicity for serving and neighbouring cell evaluation.(Nokia)

This issue was discussed at previous RAN4 meeting and in the ad hoc meeting [6], the following agreements were achieved:

Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

FFS on the measurement periodicity after wake up.

FFS on whether the wake up delay apply in the scenario when exiting from case 3.

Regarding the whether to reused the wake up delay when the UE exits from case 3, i.e., the following issue:

Issue 1-5-x: MR wake up delay after exiting case 3 due to offloading conditions cannot be met

The exiting from case 3 to legacy state can be triggered by either when MR condition is not satisfied or when LR condition is not satisfied, providing LR condition is configured, based on the following RAN2 131 meeting agreement:

The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

If the exit from case 3 to legacy is triggered by LR, considering the DRX cycle with a scaling factor 16, there could a few case where the exiting happens between two MR measurement occasions and it is also likely that the MR goes into deep sleep mode between two MR measurement occasions. Since the MR wake up delay has already been used in the scenario when exiting from case 1 and exiting from LP-WUS monitoring, it is reasonable to apply the MR wake up delay when exiting from case 3 to align the UE behaviour. After the MR wake up delay, the MR can simply follow the legacy measurement requirements.

- When exiting from case 3 due to exiting conditions of RRM relaxation is met, MR reuse the existing wake up delay, after the MR wake up delay, the MR can simply follow the legacy measurement requirements.

Issue 1-1-2: Threshold for switch between different cases

- Proposals

  - P1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). (Apple)

To our understanding, based on RAN2 discussion, the procedure when exits from offloading mode is to wake up MR, then it is up to MR’s measurement to determine whether to start neighbor cell measurement or not. There is no corresponding RAN2 agreement on using LR measurement to check the criteria for neighbor cell measurement.

It is also agreed that the offloading threshold is higher than that of threshold for neighbor cell measurement hence there is no necessity to check neighbor cell measurement criteria during offloading mode.

- No need to further discussion using LR measurement to check neighbour cell measurement criteria during offloading mode or exit offloading mode.

Issue 2-1-7: On interference in simulation

- Proposals

  - P3: For the case {M, L}={1,6} when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case (vivo Nokia)

  - P4: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences. (Huawei)

Background:

For M=1, L=6, the following set is the only case with fully opposite sequence:

- [1 0 1 0 1 0]

- [0 1 0 1 0 1]

This issue is discussed at RAN4 116 meeting and there is no consensus. Since even for {M,L}= {1,6}, there is only two sequence with fully opposite value and there are multiple sets of {M,L}, it is suggested no further action on this issue.

- No need action on the opposite sequence when {M,L}= {1,6}.

Issue 1-1-17: LP-WUR operation with SDT

- Proposals

  - P1: SDT requirements apply also for UE supporting LP-WUS.

  - P1-1: The UE supporting SDT and LP-WUS uses LR-RSRP measurements for SDT condition evaluation when the evaluation is started while the UE is in MR offloading. (Nokia)

Recommendations:

For the SDT, current in the TS38.133 the following requirements are available:

5.5.2	Requirements on UE synchronization for small data transmissions

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA value as specified in subclause 7.1.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell, then the UE shall drop the small data transmission.

5.5.3	TA validation requirements

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT.

Based on above requirements, if any conditions are not met the UE simply drop the small data transmission. Hence there is no impact on LP-WUR requirements.

- No impact on LP-WUR requirements for LP-WUR operation with SDT.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- There is no need to have discussion on the impact of low mobility criteria.

- When exiting from case 3 due to exiting conditions of RRM relaxation is met, MR reuse the existing wake up delay, after the MR wake up delay, the MR can simply follow the legacy measurement requirements.

- No need to further discussion using LR measurement to check neighbour cell measurement criteria during offloading mode or exit offloading mode.

- No need action on the opposite sequence when {M,L}= {1,6}.

- No impact on LP-WUR requirements for LP-WUR operation with SDT.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2512139, WF on RRM requirements for NR_LPWUS, RAN4 116, vivo

[5] R4- 2509065,	Topic summary for [116][222] NR_LPWUS, RAN4 116, vivo

[6] R4-2512138,	Ad-hoc minutes for NR_LPWUS