3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513537

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Views on UE RF requirements for NR_LBCA_Sw_enh

Source: 	Huawei, HiSilicon

Agenda item:	7.8.3

Document for:	Discussion

# Introduction

TSG RAN#109 approved a new WI of “Enhancement of low NR band carrier aggregation via switching”.  “NR_LBCA_Sw_enh”[1]. This paper provides views on UE RF impact of dual downlink scenario.

# Discussion

## Tx requirements

### Time mask

The below Figure 1 is an excerpt of spectrum arrangement for two example BCs from TR 38.768.

Figure 1: An excerpt of frequency arrangements for example BCs from TR38.768

In R20, n29/n67 (SDL) and n12/n28 DL are concurrently used during Case 3 operation, so-called dual DL. That is the only difference compared to R19 in terms of UE RF architecture. In this sense, we don’t see any reasons to change time mask for R20.

Proposal 1: No new time mask for R20 LB-LB CA SW is needed on top of that for R19 LB CA SW.

### Switching period

Switching period was one of the items extensively discussed in R19 LB CA SW. In the end, Rel-19 defined 35μs, 70μs, and 140μs for switching between Case 1 and Case 2 as an optional UE capability. The below Figure 2 shows possible implementations for the corresponding switching periods.

Figure 2: Possible implementations according to needed switching periods

As can be seen in Figure 2, although Rx filter design for R20 is different from that for R20, essentially, the needed architectures for the respective switching periods stay.

Observation 1: There is no factors to shorten switching period in R20 compared to that for R19 in terms of UE RF point of view. Required Rx filter design for R20 CA_n12-n29/CA_n28-n67 is different from that for R20, but it doesn’t give any reasons to shorten the switching period.

In addition, as clearly described in the WID, it is encouraged to “reuse” R19 requirements as much as possible.

Observation 2: R20 WID clearly says that reuse R19 requirements as much as possible.

Proposal 2: Switching periods for R20 shall be the same as those for R19, i.e., 35μs, 70μs, and 140μs for switching between Case 1 and Case 3 are defined as an optional UE capability.

## Rx requirements

### Rx general

First of all, before going into deep details, we believe that how UE to UE and BS to BS co-existence issues (SDL BS transmission to FDD BS reception), is addressed shall be clarified as discussed in our companion paper of [2]. If the conclusion is e.g., setting a guard band between SDL spectrum and FDD UL spectrum, or only part of FDD or SDL is used, the conclusion may impact on UE RF discussion, specifically filter design.

Observation 3:  How UE to UE or BS to BS co-existence issue (SDL BS transmission to FDD BS reception), is addressed shall be clarified before going into deep detail discussion on Rx requirements as discussed in our companion paper of [R4-2513536].

Observation 4: For CA_n28-n67, if an operator holds only lower portion of n28, 5 MHz guard band is available so that it may resolve the issue, though still feasibility study is needed, e.g., whether or not 5 MHz is enough for BS filter to have sufficient attenuation, if it is feasible to keep 50 MHz(738-788 MHz) pass-band as UE Rx filter in 700 MHz frequency range, etc.

In addition, as can be seen from Figure 1, the relative frequency ration of n67+n28 DL is around 8.4%. The feasibility study is needed if full frequency range is expected to be used.

Observation 5: For CA_n28-n67, if full-range of spectrum is used, UE Rx filter to cover both n67 + n28 DL(738-803MHz) wouldn’t be feasible.

Proposal 3:  RAN4 should first clarify how UE to UE and BS to BS co-existence issues are resolved and if there is any restriction such as only lower portion of n28 is used, or only some portion of n29 is used etc., before going into the deep detail.

Proposal 4: Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of Proposal 3.

### REFSENS

For CA_n12-n29, current REFSNS for n29 may not be stringent even if we consider some special attention to eliminate n12 UL blocker from other UEs. Thus, we may be able to keep the current REFSENS or may need only limited amount of relaxation depending on further study.

Observation 6: For CA_n12-n29, RAN4 may be able to keep the current REFSENS or may need only limited amount of relaxation depending on further study.

For CA_n28-n67, the discussion is affected by the conclusion on if there is any restriction in terms of spectrum usage for LB CA operation with dual DL.

Observation 7: For CA_n28-n67, the discussion is affected by the conclusion on if there is any restriction in terms of spectrum usage for LB CA operation with dual DL, e.g., fullband n28 is expected to be used or only lower portion of n28 is used.

### Blocking

Since in dual DL operation, Rx filter combined with both FDD DL (or portion of FDD DL) and SDL (or portion of SDL) is implemented, the locations of blockers for blocking requirements shall be shift depending on the assumed Rx filter frequency range.

Proposal 5: Locations of blockers for blocking requirements shall be shift depending on assumed actual Rx filter frequency range.

# Conclusions

In this paper we provided our views on UE RF aspects for R20 LB CA SW.

Proposal 1: No new time mask for R20 LB-LB CA SW is needed on top of that for R19 LB CA SW.

Observation 1: There is no factors to shorten switching period in R20 compared to that for R19 in terms of UE RF point of view. Required Rx filter design for R20 CA_n12-n29/CA_n28-n67 is different from that for R20, but it doesn’t give any reasons to shorten the switching period.

In addition, as clearly described in the WID, it is encouraged to “reuse” R19 requirements as much as possible.

Observation 2: R20 WID clearly says that reuse R19 requirements as much as possible.

Proposal 2: Switching periods for R20 shall be the same as those for R19, i.e., 35μs, 70μs, and 140μs for switching between Case 1 and Case 3 are defined as an optional UE capability.

Observation 3:  How UE to UE or BS to BS co-existence issue (SDL BS transmission to FDD BS reception), is addressed shall be clarified before going into deep detail discussion on Rx requirements as discussed in our companion paper of [R4-2513536].

Observation 4: For CA_n28-n67, if an operator holds only lower portion of n28, 5 MHz guard band is available so that it may resolve the issue, though still feasibility study is needed, e.g., whether or not 5 MHz is enough for BS filter to have sufficient attenuation, if it is feasible to keep 50 MHz(738-788 MHz) pass-band as UE Rx filter in 700 MHz frequency range, etc.

Observation 5: For CA_n28-n67, if full-range of spectrum is used, UE Rx filter to cover both n67 + n28 DL(738-803MHz) wouldn’t be feasible.

Proposal 3:  RAN4 should first clarify how UE to UE and BS to BS co-existence issues are resolved and if there is any restriction such as only lower portion of n28 is used, or only some portion of n29 is used etc., before going into the deep detail.

Proposal 4: Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of Proposal 3.

Observation 6: For CA_n12-n29, RAN4 may be able to keep the current REFSENS or may need only limited amount of relaxation depending on further study.

Observation 7: For CA_n28-n67, the discussion is affected by the conclusion on if there is any restriction in terms of spectrum usage for LB CA operation with dual DL, e.g., fullband n28 is expected to be used or only lower portion of n28 is used.

Proposal 5: Locations of blockers for blocking requirements shall be shift depending on assumed actual Rx filter frequency range.

# Reference

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)

- R4-2513536, General views on NR_LBCA_Sw_enh, Huawei, HiSilicon