3GPP TSG-RAN WG4 Meeting #116bis                                             				R4-2513560

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.6.4.1

Source:		Eutelsat, THALES

Title:		HD-FDD implications for Ku and Ka band VSAT RF

Document for:		Discussion/ Decision

# Introduction

At RAN#109 a new WID [1] with the following scope was agreed:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

# Overview

This contribution considers the impact on TS 38.101-5 [2] from the introduction of HD-FDD VSAT in Rel-20. The emphasis of this contribution is on Phase 1. Phase 2 aspects are also briefly considered and commonalities with Phase 1 work identified.

# Discussion

## HD-FDD

- HD-FDD VSAT can either receive from the serving SAN or a transmit to the serving SAN at any one time, not both.

- The preferred implementation of a HD-FDD VSAT is an electronically steered flat panel antenna with a single physical Tx/ Rx aperture (as opposed to separate panels for Tx and Rx). Therefore, it is assumed that a T/R switch functionality will be used. Such a switch functionality can be implemented by either T/R switches at the front-end; or alternatively by using duplexers at the front-end with ON/OFF switching capabilities at the Rx/Tx path.

Observation 1: In any HD-FDD implementation VSAT transmission to the SAN is ‘off’ when the VSAT is in receive thus avoiding the transmitter chain adversely affecting receiver sensitivity.

Proposal 1: The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with a Tx/Rx switch function.

The present specification TS 38.101-5, section 6.3.3 specifies a ‘Transmit ON/OFF time mask’.

Proposal 2: RAN4 to investigate if the existing transmit ON/OFF time mask can be reused or, if necessary, specify a T/R and, if different, a R/T switching time.

- At present the 3GPP specifications limit the applicability of FR1-NTN in bands above 10 GHz to the Ku band; Ka band only supports FR2-NTN numerology which is outside the scope of Phase 1 of the WID agreed at RAN#109.

- The HD-FDD limitation only applies to the VSAT UE; the SAN is unaffected (i.e., full duplex).

- We (will) fully support the Phase 2 work but recognise this will follow Phase 1 as this may have some impact on RAN1 specifications; in particular to add support for 120 kHz SCS operation.

Observation 2: In Phase 1 work, according to the WID, RAN4 will focus on Ku band VSAT UE using FR1-NTN numerology; Ku band using FR2-NTN numerology and Ka band (FR2-NTN numerology) will follow upon completion of Phase 1.

Observation 3: The SAN RF specification [3] is unaffected by the introduction of HD-FDD VSAT,

- The starting point for HD-FDD RF parameters should be the full duplex parameters already specified for Ku band FR1-NTN numerology. Note that the reference sensitivity specified in [1] section 10.3 “OTA reference sensitivity level” permits the vendor to define the minimum elevation angle at which the requirement is met:

“] the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75”

Thus, unless the HD-FDD VSAT is unable to meet sensitivity within this broad range, no change is necessary.

- It is expected that HD-FDD operation will enable smaller size flat panel antennas to be feasible (see [x]).

Proposal 3: RAN4 should start by reviewing which RF parameters, if any, of existing VSAT types 2, 3, 5, 6 with electronically steered antenna need to be modified.

- The target size of 20 cm x 20 cm for an Ultra Small HD-FDD antenna (the example in the WID) will result in some RF performance compromises (e.g., receiver sensitivity) which will necessitate creating a new VSAT type (“Type 7”) in [2].

Proposal 4: Create a new VSAT Type 7 in the specification [2] with relaxed RF performance compared with existing (larger) antenna types.

Considerations for future work on HD-FDD include:

- Although not yet specified by 3GPP, full band VSAT UE with their uplink operating in the Ku band 12.75 – 13.25 GHz, or any other band where the Downlink/ Uplink duplex gap is not sufficient to permit RF filtering, could benefit from HD-FDD.

- Any changes to RF specifications required to support HD-FDD for Ku band using FR1-NTN numerology are likely to also apply to Ku band FR2-NTN and Ka band (FR2-NTN).

Observation 4: HD-FDD and/or Ultra-Small antenna may also apply in future bands.

Proposal 5: The specification structure should aim to permit reuse of the HD-FDD capability and/or Ultra Small antenna in future 5G NR or 6G bands.

Proposal 6: Specification revisions for both VSAT UE TS 38.101-5 and SAN TS 38.108 could be further considered.

Proposal 7: RAN4 may further consider to update at least the following requirements for HD-FDD VSAT UE compared with FDD VSAT UE: EIRP (could be increased, at least EIRPmax), EIS (could be decreased), tolerances, reference measurement channels, EVM evaluation and frequency evaluation methodology.

Proposal 8: RAN4 may further discuss to update at least the following requirements for SAN RF (under HD-FDD VSAT UE work, compared with FDD VSAT UE use case): reference measurement channels, EVM evaluation and frequency evaluation methodology.

# Conclusions

An analysis of the impact of HD-FDD on the operation of VSAT UE above 10 GHz has been discussed and the following observations are made and proposals for future work are offered.

Observation 1: In any HD-FDD implementation VSAT transmission to the SAN is ‘off’ when the VSAT is in receive thus avoiding the transmitter chain adversely affecting receiver sensitivity.

Proposal 1: The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with a Tx/Rx switch function.

Proposal 2: RAN4 to investigate if the existing transmit ON/OFF time mask can be reused or, if necessary, specify a T/R and, if different, a R/T switching time.

Proposal 3: RAN4 should start by reviewing which RF parameters, if any, of existing VSAT types 2, 3, 5, 6 with electronically steered antenna need to be modified.

Proposal 4: Create a new VSAT Type 7 in the specification [2] with relaxed RF performance compared with existing (larger) antenna types.

Observation 4: HD-FDD and/or Ultra-Small antenna may also apply in future bands.

Proposal 5: The specification structure should aim to permit reuse of the HD-FDD capability and/or Ultra Small antenna in future 5G NR or 6G bands.

Proposal 6: Specification revisions for both VSAT UE TS 38.101-5 and SAN TS 38.108 could be further considered.

Proposal 7: RAN4 may further consider to update at least the following requirements for HD-FDD VSAT UE compared with FDD VSAT UE: EIRP (could be increased, at least EIRPmax), EIS (could be decreased), tolerances, reference measurement channels, EVM evaluation and frequency evaluation methodology.

Proposal 8: RAN4 may further discuss to update at least the following requirements for SAN RF (under HD-FDD VSAT UE work, compared with FDD VSAT UE use case): reference measurement channels, EVM evaluation and frequency evaluation methodology.

# References

[1]	 RP-252962 New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN#109, Sept. 2025

[2] TS 38.101-5	NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements, v19.1.0, June 2025

[3] TS 38.108 	NR; Satellite Access Node radio transmission and reception, v19.1.0, June 2025