3GPP TSG-RAN WG4 Meeting#116bis 	R4-2513572

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussions on OTA test methods for NTN Ka and Ku bands

Agenda Item:	7.3.3

Document for:	Approval

- Introduction

Objectives for this core part work item are as follows

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

  - Specify the required quiet zone to cover VSAT device type based on industry needs

    - 60cm QZ as a starting point

  - Specify preliminary MU assessment for test methodologies

During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic.

3GPP RAN Plenary approved the new Rel-20 OTA WI [1] in the most recent RAN#109 meeting held in Beijing, Sep 2025. The objective for core part WI is focusing on test methodologies development for NTN VSAT to support Ka and Ku bands:

In this paper, we share our initial views on potential test methods enhancement to support NTN VSAT Ka and Ku bands.

- Discussion

NTN Ka and Ku band cover the frequencies between 10-20GHz. To define the enhanced test method to verify Ka/Ku core requirements, it should be clearly stated that RAN4 should focus on developing OTA test methodology to cover all the specified RF/RRM/Demodulation requirements.

Proposal 1: RAN4 should define enhanced test methodologies to cover all NTN Ka and Ku bands requirements testing, includes RF, RRM and Demodulation.

Extending the specified FR2 test methodologies (DFF/IFF/NFTF) to cover Ka and Ku bands can be a starting point for further discussions, there are some challenges:

- FR2 IFF Test system improvement

Technically, the most promising way is to extend the applicability of FR2 IFF method (24~40GHz) down to 8~10GHz by using advanced reflector and larger chambers.

CATR solution as baseline: CATR systems use a parabolic reflector to create a collimated (planar) wavefront in a relatively short distance, simulating a far-field condition within a compact chamber, this can be a premier solution for 8-24 GHz testing due to the minimized issue of high path loss by reflector and Large, high-quality quiet zone in the chamber. However, the low-frequency range will increase the manufacturing difficulty of reflector, which would be a key challenge of this method. Further analysis on feasibility and quality of Quiet Zone is required.

- FR1/FR2 DFF Test system improvement

Current FR1 AC system works well below 7GHz, it is difficult to extend the system capability to cover Ka ranges due to the limitation of measurement antenna and other RF elements:

- Current measurement antenna can support up to 10~12GHz, FFS higher frequency

- RF elements, e.g., amplifiers, should be updated

- Absorber may also be updated

- New MU assessment is required

- The complexity of the test system depends on how large QZ is needed

Current FR2 AC system also has some limitations:

- There are commercially available test systems to support 700-20GHz OTA testing, but only for R&D purpose. The QZ quality may not meet standard requirements

- Absorber may also be updated to meet the MU

- The chamber size depends on QZ

In short, the measurement antenna to support Ka and Ku band would be feasible. But the challenge of the system enhancement is also highly dependent on how large the QZ is and whether the min measurement distance can be relaxed (if DFF is used).

Proposal 2: RAN4 should study the feasibility and complexity of extending FR1 AC system and FR2 IFF/DFF system to cover NTN Ka/Ku frequency range.

Proposal 3: RAN4 should specify a reasonable QZ for NTN VSAT testing to minimize the burden of test system upgrade.

Regarding linear polarization or circular polarization measurements, this has also been discussed in Rel-19 FR1 NTN OTA. RAN4 confirmed that from test system perspective, it is feasible to update the test setup to support CP measurements. However, due to complexity of test system update and unclear measurement gain of this feature, RAN4 finally decided to stick on traditional linear polarization-based approach for FR1 NTN OTA measurements.

But for above 10GHz NTN, anyway system update is required, RAN4 should target to support the CP-based measurements for NTN.

Proposal 4: RAN4 should target to specify NTN OTA test methodologies supporting circular polarization measurements.

- Conclusion

We have the following proposal for Rel-20 OTA test method enhancement:

Proposal 1: RAN4 should define enhanced test methodologies to cover all NTN Ka and Ku bands requirements testing, includes RF, RRM and Demodulation.

Proposal 2: RAN4 should study the feasibility and complexity of extending FR1 AC system and FR2 IFF/DFF system to cover NTN Ka/Ku frequency range.

Proposal 3: RAN4 should specify a reasonable QZ for NTN VSAT testing to minimize the burden of test system upgrade.

Proposal 4: RAN4 should target to specify NTN OTA test methodologies supporting circular polarization measurements.

- References

- RP-252903, WID: Enhancement of UE OTA test method and requirements for NR, RAN VC (vivo), RAN meeting #109, Sep 2025.