3GPP TSG-RAN WG4 Meeting #116-bis												R4-2513580

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:			7.3.3

Source:	Huawei, HiSilicon

Title:	on NTN Ka and Ku band OTA tests

Document for:	approval

# Introduction

The WID [1] of OTA tests has been agreed during RAN#109 as shown below. This contribution discusses the associated tests in more details.

- Define enhanced test methodologies to cover NTN Ka and Ku bands as well as VSAT device type

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

  - Specify the required quiet zone to cover VSAT device type based on industry needs

    - 60cm QZ as a starting point

  - Specify preliminary MU assessment for test methodologies

# Discussion

Ku and Ka bands have frequency range of 12-18GHz and 26.5-40GHz, respectively. Such frequency ranges make existing FR2 OTA test methodologies more suitable than those for FR1. Various test methodologies such as DFF, IFF, NFTF are available for FR2 frequency range. A VSAT size of 20cm x 20cm and possibly larger was mentioned during RAN#109 discussion and the associated far field distance would be 8 and 16 meters at 15GHz and 30 GHz, respectively. If a quiet of zone 60cm is considered as the maximum dimension of a VSAT, the far field distance would be 36 and 72 meters at 15GHz and 30 GHz, respectively. This implies that the Direct Far Field (DFF) may not be suitable for Ku and Ka band VSAT OTA measurement due to practical considerations such as chamber sizes.

Proposal 1: remove DFF from consideration as the far field distances required by a quiet zone of 60cm at Ku and Ka bands are too large to implement in anechoic chambers.

It is possible to use criteria such as measurement uncertainty, chamber size, or cost, etc to decide either IFF or NFTF to be the baseline method. However, such an exercise may not be the best use of meeting time. As a result, test procedures for both IFF and NFTF should be developed under this work item.

Proposal 2: develop test procedures for both IFF and NFTF.

A VSAT of 20cm x 20cm is more likely to form beams at Ku and Ka bands. Then some existing FR2 metrics should be considered within a given field of view as VSAT is used to provide coverage in the upper hemisphere. A field of view of up to 60 degrees, with 0 degree and 90 degree representing zenith and horizon, respectively, should be sufficient to cover most satellite constellations and the corresponding performance metrics are defined within this field of view.

Proposal 3: define the field of view for VSAT to be up to [60] degrees in upper hemisphere.

Metrics such as maximum TRP and EIRP are for regulatory compliance and should be included in VSAT OTA measurement.

Proposal 4: include maximum TRP and EIRP within a defined field of view for regulatory compliance.

For orbits other than geo-stationary, VSAT needs to track satellites moving across the sky. Such tracking capability should be tested by defining satellite angular speeds for various orbits as seen from VSAT. For this type of tests, the link antenna in the chamber should be placed in the upper hemisphere because VSAT would track signals from the link antenna in order to form beams. From an implementation point of view, it would be easier to keep the position of the link antenna fixed and rotate the platform where VSAT is mounted. The speed of platform rotation should correspond to the defined satellite angular speed as seen from VSAT. The platform rotation should resemble satellite trajectory as seen from VSAT and would be in two dimensions, namely q- and f-directions in a Cartesian coordinate. In reality, VSAT would be able to move its beams in both q- and f-directions.

Typically NTN satellites use Low Earth Orbit (LEO) and Medium Earth orbit (MEO). Instead of testing performances under LEO and MEO separately, the model for angular speed could be the average of those under LEO and MEO in order to reduce test time.

Proposal 5: develop a combined LEO and MEO model of angular speed as seen from VSAT within a defined field of view.

The transmit and receive metrics for VSAT can be average of EIRP and sensitivity measurement across a defined field of view at given grid points under the angular speed model. Details of sensitivity measurement under a given angular speed need to be discussed because sensitivity measurement may involve value search, for example, to reach 95% of maximum throughput and could take longer time to complete than that allowed by an angular speed model.

Proposal 6: define transmit and receive metrics as the average of EIRP and sensitivity measurement across a defined field of view at given grid points under angular speed model.

NFTF systems may have two or more different configurations. One configuration could consist of an array of near field probes and can take near field measurement at required grid points all at once. Another configuration would employ a single probe and measure all near field grid points one at a time.

Observation 1: NFTF using a single probe to scan the near field may not be suitable for angular speed model

Another discussion point is that the link antenna placed at the zenith may cause blockage in both IFF and NFTF test systems. For example, if the link antenna is placed in front of or through a hole at compact range antenna, it would cause blockage and degrade the quality of plane wave in the quiet zone.

A similar situation could occur in NFTF. Possible options are to

- replace a measurement probe in the near field probe array by the link antenna

- embed the link antenna in the near field measurement probe array provided there is enough space among the measurement probes to accommodate the link antenna.

- use a measurement probe for measurement in uplink and link antenna in downlink assuming the probe antenna can support both uplink and downlink frequency ranges simultaneously, followed by duplexer, etc.

For example, the effect of the blockages described above could be accounted for in quality of quiet zone.

Observation 2: potential blockage caused by link antenna in both IFF and NFTF needs solutions or to be accounted for.

A common issue for both Ku and Ka bands and VSAT devices is whether tests should use circular or linear polarisation, because most communication satellites use circular polarisation while current test methodologies defined in 3GPP employ linear polarisation. If linear polarisation is used in test systems, devices or VSAT with circular polarisation would be unduly penalised even though those devices would have better performance in real NTN networks. In addition, there would be no possibility to use test systems with linear polarisation to assess the performance of circular polarisation such as axial ratio. The reason for test systems to use circular polarisation is to drive devices to improve their performances in real NTN network operation. While test systems using linear polarisation clearly fail to play such a role.

Proposal 7: NTN OTA tests should use circular polarisation.

The use of circular polarisation (CP) for NTN device measurement was discussed in release 19 and a number of companies expressed reluctance to use circular polarisation as all the current OTA measurement systems use linear polarisation. If such reluctance still persists in this work item, an alternative is to use measurement under linear polarisation (LP) to derive characteristics of circular polarisation. Obviously, this is not an ideal solution and does not portrait 3GPP as a technology leading standard development organisation.

Both reference [2] and [3] demonstrate that CP performances such as total power and axial ratio can be derived by making 3 or 4 LP measurements. The method in [2] uses two pairs of orthogonal measurements with a comment that one of the measurements can be derived from the other three, viz. three measurements would be sufficient. However, it claims that four measurements improve accuracy. Fig.1 in [2] shows a generic diagram of elliptic polarisation. The derived phase difference between two orthogonal electric fields, major axis inclined angle and axial ratio are given in equations (10), (11) and (12) in [2], respectively.

[3] adopts essentially the same approach as [2], but with 3 LP measurements in any directions. Namely orthogonality among LP measurements is not required. Fig.5 in [3] provides the layout of an elliptic polarisation, together with the angles of 3 LP measurements. In [3], the derived left and right handed circular polarisations are given equations (44), (45), axial ratio is in equation (45) and major axis inclined angle is in equation (60).

Neither [2] nor [3] can determine the handedness of CP due to the sign uncertainty in determining the phase difference between orthogonal electric fields in CP.

The derivations in [2] and [3] are for CP transmission. The same procedure can equally be applied to receive measurement.

Proposal 8: If the use of circular polarisation is not agreeable for VSAT measurement, adopt the method in reference [3] in R4-2513580 to derive circular polarisation performance metrics and axial ratio using 3 linear polarisation (LP) measurements with two out of three LP measurements orthogonal to each other to align with E and E.

# Conclusion

This contribution provides the following proposals and observations.

Proposal 1: remove DFF from consideration as the far field distances required by a quiet zone of 60cm at Ku and Ka bands are too large to implement in anechoic chambers.

Proposal 2: develop test procedures for both IFF and NFTF.

Proposal 3: define the field of view for VSAT to be up to [60] degrees in upper hemisphere.

Proposal 4: include maximum TRP and EIRP within a defined field of view for regulatory compliance.

Proposal 5: develop a combined LEO and MEO model of angular speed as seen from VSAT within a defined field of view.

Proposal 6: define transmit and receive metrics as the average of EIRP and sensitivity measurement across a defined field of view at given grid points under the angular speed model.

Observation 1: NFTF using a single probe to scan the near field may not be suitable for angular speed model

Observation 2: potential blockage caused by link antenna in both IFF and NFTF needs solutions or to be accounted for.

Proposal 7: NTN OTA tests should use circular polarisation.

Proposal 8: If the use of circular polarisation is not agreeable for VSAT measurement, adopt the method in reference [3] in R4-2513580 to derive circular polarisation performance metrics and axial ratio using 3 linear polarisation (LP) measurements with two out of three LP measurements orthogonal to each other to align with E and E.

# Reference

- RP-252903 WID: Enhancement of UE OTA test method and requirements for NR

- Improved Measurement Method of Circularly-Polarized Antennas Based on Linear-Component Amplitudes, D. Wang, et al, Open Journal of Antennas and Propagation, Vol.5, pp.36-45, March 2017, DOI: https://doi.org/10.4236/ojapr.2017.51004

- Determining Antenna Polarization Using Amplitude-Only Measurements With Linear-Polarized Antenna in Three Positions, R. Zentner, et al, IEEE Access, 2024, Vol.12, pp.136542-136564, DOI: 10.1109/ACCESS.2024.3463477