3GPP TSG-RAN WG4 Meeting #116bis	            				 R4-2513587

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.11.7

Source: 	Apple

Title: 			 Demodulation and/or CSI reporting requirements for CSI prediction

Document for:	Discussion

Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved. This contribution provides discussion on demodulation requirements for CSI prediction.

Discussion

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

According to the revised WID RP-243244, CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 requirements. In previous meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. Reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook.

For CSI prediction, it was agreed that AI/ML based performance requirement should not be worse than the legacy, and FFS which legacy requirement/feature are relevant.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

During RAN4#115, the following agreement was reached:

Issue 1-1: Requirement considerations

Agreement:

The performance of AI based CSI prediction should not be worse than R16 etypeII.

- The exact test procedure and configurations are FFS

- The impact of different timeline between AI and R16 etype II should be also carefully analysed

Issue 1-3: Doppler handling

Agreement:

  - De-prioritize 100Hz Doppler in R19

    - Consider 100Hz Doppler in the future release if needed

In the context of AI/ML-based CSI prediction, defining meaningful and fair performance requirements is fundamentally more complex than in legacy PMI testing. Unlike traditional PMI reporting, where the algorithm behavior is deterministic and independent of deployment scenarios, the performance of AI/ML models is scenario-dependent, tightly linked to the training dataset and inference conditions. Factors such as antenna configuration, UE speed (Doppler), observation window length, and prediction window length directly affect prediction accuracy and generalization. This poses a challenge for RAN4, as the current legacy testing framework assumes a single PMI requirement is sufficient to represent conformance across all conditions.

While non-static channel conditions (e.g., changing SNR or fading profiles over time) offer a more realistic and comprehensive test of model generalization, they introduce significant complexity and ambiguity, especially for a group like RAN4 that has not previously handled time-varying test conditions. Defining the appropriate dynamics, reference models, and reproducible baselines would require extensive new work and agreement, which is not feasible within the limited timeline of Rel-19.

Therefore, to ensure AI/ML model robustness and performance while remaining within practical constraints, it is essential that RAN4 focuses on well-defined static test conditions for Rel-19. However, unlike legacy PMI tests, multiple static test cases must be defined, each reflecting a different scenario dimension. This allows the test framework to reflect real-world variability in AI/ML model behavior without requiring non-static (time-varying) setups, thereby ensuring a balance between test coverage and feasibility.

AI/ML-based CSI prediction introduces a dynamic behavior that diverges from the deterministic operation of legacy PMI reporting. Unlike traditional methods, the performance of an AI model can vary significantly over time due to changes in deployment environments, UE behavior, and data drift. This creates a need for a runtime performance monitoring mechanism to detect and react to model degradation or misalignment. Without such monitoring, a malfunctioning or outdated model could lead to persistent suboptimal performance, such as inaccurate precoder selection, reduced throughput, or even link failures, without the network being aware.

To address this, RAN1 has introduced the concept of Type 3 UE-assisted performance monitoring, which allows the UE to internally assess the quality of the AI/ML model's predictions. A promising metric identified for this purpose is the Squared Generalized Cosine Similarity (SGCS) between the predicted CSI and either the actual measured CSI or the CSI-RS-implied channel. However, there is currently no standardized procedure in RAN4 for how this metric should be computed, aggregated (e.g., as a CDF), or reported. Additionally, there is no guidance on how the network should interpret these values, nor how such reports should interact with fallback or reconfiguration mechanisms.

Test Assumptions for CSI prediction

In the last meeting, in order to facilitate the AI/ML model evaluation and alignment, the initial simulation assumption was agreed. We would like to provide our view for further proceeding the AI/ML CSI prediction evaluation in step 2 and step 3.

Channel Estimation and related SNR point(range) for simulation calibration

In the evaluation of AI/ML-based CSI prediction, the accuracy of the predicted CSI is highly sensitive to the quality of the input channel estimates and the associated SNR conditions. While initial evaluations have often assumed ideal DL channel estimation for calibration and intermediate metric comparison, this assumption does not reflect realistic deployment conditions. Practical channel estimation methods introduce noise and estimation errors that directly impact the CSI fed into the AI/ML model, thereby degrading model performance.

Additionally, the performance of CSI prediction models is heavily dependent on the SNR range over which the training and testing datasets are collected. In low SNR conditions, noise dominates the channel, distorting spatial correlation properties and making accurate prediction ineffective. Conversely, training across an excessively wide SNR range may dilute model performance in practical deployment scenarios. Thus, a standardized approach to define the baseline channel estimation method and to select appropriate SNR ranges for model training and simulation calibration is essential.

Moreover, legacy methods such as PMI-based throughput measurements already use SNR thresholds, e.g., the SNR at which 90% throughput is achieved with following PMI. Aligning the AI/ML CSI prediction evaluation with such existing baselines provides a concrete reference while enabling model comparison across implementations.

RAN would need to address the following issues for simulation alignment and performance requirements:

- Whether a baseline practical channel estimation method (e.g., as used in demodulation tests) should be standardized for AI/ML model input preparation, or left to company-specific implementations.

- How to determine and agree on the SNR operating point and range used for AI/ML training and testing. One proposal is to center this range around the SNR at which 90% of maximum throughput is achieved with following PMI, using ±X dB margins. [R4-2507737 ]

- Whether AI models should be trained using data from specific SNR points or a mixed SNR dataset spanning a defined range, especially when practical estimation (rather than ideal) is applied.

- Implications of SNR-dependent training on model robustness and generalization, especially under variable real-world radio conditions.

We would like to propose an alternative way to train the model with a range of SNR values. We propose to introduce curriculum-based SNR training as a structured alternative to the conventional ±X dB training window for AI/ML-based CSI prediction. While the ±X dB method, where training data are collected uniformly within a fixed range around a central SNR,  provides a simple and implementable approach, it treats all SNR levels equally from the beginning, potentially hindering training stability and convergence. As an enhancement, we propose curriculum training to better align the learning process with the model’s capacity to generalize over varying SNR conditions.

Curriculum training in AI refers to a machine learning strategy where a model is trained on data that is presented in a progressively increasing order of difficulty. This approach is inspired by how humans learn, starting with simpler concepts and gradually building up to more complex ones. By starting with easier examples, the model can build a foundational understanding before tackling more challenging aspects of the task, potentially leading to faster learning and better generalization.

Both approaches start from a common reference point, the SNR anchor (SNR_anchor), which is defined as the SNR where the throughput using predicted PMI achieves 90% of the maximum throughput (SNR_ue_AI/ML). The ±X dB method defines the training SNR range as [SNR_anchor – X, SNR_anchor + X], with X typically set between 3–5 dB. While this approach ensures relevance to the test operating point, it may expose the model too early to noisy samples where CSI correlation is degraded.

The curriculum-based training strategy improves on this by staging the training over multiple SNR phases:

- Phase 1: Start with high-SNR samples (e.g., SNR ≥ SNR_anchor + 3 dB) where CSI structure is clear and prediction is stable.

- Phase 2: Expand to include moderate-SNR samples (e.g., SNR ∈ [SNR_anchor – 3 dB, SNR_anchor + 3 dB]) to introduce more realistic variation.

- Phase 3: Optionally include lower-SNR data (e.g., SNR_anchor – 5 dB) if SGCS or PMI throughput remains meaningful, enabling robustness to real-world degradations.

This staged training approach helps the model first learn the temporal and spatial correlation in cleaner CSI before generalizing to noisier and more variable environments. It reduces the risk of early overfitting to noise, improves convergence, and ensures better alignment between training and test-time performance when practical channel estimation is applied. Therefore, while the ±X dB method remains a useful baseline, RAN4 is invited to investigate curriculum-based SNR training as a candidate approach for standardizing AI/ML CSI prediction model for simulation alignment and performance requirements evaluations

Curriculum-Based SNR Training for AI/ML CSI Prediction

RAN4 is invited to perform CSI-prediction evaluation by combining clear test assumptions with a staged SNR training approach that improves model robustness to noise

- Baseline test conditions – Standardize practical channel estimation and SNR operating ranges to define the conditions under which performance requirements (e.g., SGCS thresholds, throughput ratios) will later be applied.

- Curriculum SNR training – Begin with high-SNR samples where CSI structure is clean, then progressively expand to moderate and lower SNR ranges to strengthen robustness.

- Improved convergence and generalization – Enable the model to learn stable spatial-temporal patterns first, reducing overfitting to noise and improving performance under realistic conditions.

Generalization Issues for CSI prediction

In order to investigate the robustness of the models under diverse conditions, RAN4 may consider the following variability dimensions that can impact model generalization:

Doppler Profile / Channel Dynamics

- Different UE speeds and mobility profiles affect temporal correlation.

- A model trained at 30 Hz Doppler may fail under 60–100 Hz unless adapted.

- R4-2508080 agreed to de-prioritize 100 Hz Doppler in R19 but acknowledge the future need for robustness.

Antenna Port Configuration

- CSI prediction can depend on port density (e.g., 8 vs 32), antenna topology (ULA, URA), and codebook alignment.

- Training on one antenna virtualization configuration may not generalize across others

Environmental Scenario / Deployment Topology

- Channel fading patterns in rural macro differ from urban canyon scenarios.

- Model trained on one channel model may underperform when deployed in another unless generalization is tested.

Generalization Issues across Antenna Port Configurations

In AI/ML-based CSI prediction, models are trained to learn how the wireless channel evolves over time based on CSI input observed at the UE. While the temporal dynamics of the physical channel are governed by environmental mobility, the structure of the CSI input is shaped by how the gNB virtualizes and exposes CSI ports, which may vary significantly between vendors and deployments. Even with similar physical antenna panels, gNBs may implement different virtualization schemes for CSI-RS ports, including varying mappings of physical antenna elements to CSI ports, different port densities, and distinct port indexing patterns.

This variation creates a generalization challenge for AI/ML models trained on CSI data from a specific gNB. A model trained on one virtualization layout may learn spatial correlation patterns, angular structures, or port ordering relationships that do not hold when the same UE connects to a gNB with a different virtualization configuration. As a result, even if the physical propagation environment is similar, the AI/ML model may misinterpret the CSI structure and produce suboptimal predictions. For instance, in one deployment the CSI port grid may correspond to a full 2D angular domain (e.g., 4×8), while another gNB might expose a reduced or reordered set of ports (e.g., sparse 1×32 with azimuth-only emphasis), resulting in different channel feature representations.

Moreover, since CSI prediction often feeds into PMI selection using Rel-18 eType II codebooks which assume a spatially structured beamspace  a mismatch in port layout between training and deployment may distort how the model projects CSI onto beam indices. To address this, models can be designed with layout-awareness, or trained on CSI datasets generated from multiple gNB virtualization schemes.

Doppler Effects

In AI/ML-based CSI prediction, the model aims to learn how the wireless channel evolves over time based on past CSI observations. While the prediction task is fundamentally governed by the physics of mobility and scattering, a key challenge arises when the Doppler characteristics of the environment differ between the training and inference. The Doppler spread, which is proportional to user equipment (UE) speed and carrier frequency, governs the rate of channel variation. Therefore, a model trained on data from a specific Doppler regime (e.g., low-mobility users at 30 Hz Doppler) may underperform when deployed in a scenario with faster-moving users (e.g., 60 Hz or 100 Hz Doppler), due to a mismatch in the temporal dynamics it has learned to expect.

From a learning perspective, Doppler spread influences the temporal correlation between successive CSI snapshots. In low-Doppler conditions, the channel varies slowly and exhibits high temporal coherence, allowing the model to make relatively straightforward extrapolations into the future. In contrast, in high-Doppler environments, the temporal correlation degrades more rapidly, and the CSI changes more erratically across time. A model trained only on low-Doppler data may overfit to smooth or slowly evolving patterns and fail to generalize when the input exhibits rapid fluctuations that were not present in the training data. This leads to reduced prediction accuracy and potential degradation in downstream tasks such as PMI selection.

Moreover, different Doppler conditions may not only affect the speed of channel variation but also alter the effective time window over which CSI prediction remains feasible. For example, a fixed prediction horizon of Δ = 10 ms may correspond to very different correlation levels depending on whether the Doppler is 30 Hz or 80 Hz. This discrepancy introduces a hidden assumption in the training data distribution, where the model learns to associate a given time step with a certain amount of channel change. When that assumption no longer holds (i.e., in deployment with higher Doppler), the model’s internal representation of time progression becomes invalid, leading to poor generalization.

To address this, AI/ML models need to be trained and evaluated across a range of Doppler profiles, reflecting realistic UE mobility scenarios. Alternatively, models can incorporate explicit Doppler-awareness through input features (e.g., UE speed or estimated Doppler spread), dynamic prediction horizons, or adaptive recurrence mechanisms. Without such consideration, models may be brittle to variations in user speed or frequency band. Therefore, RAN4 is invited to consider Doppler generalization as a critical axis in CSI prediction testing and potentially define test cases that span representative Doppler spreads (e.g., 30 Hz, 60 Hz, 80Hz) to ensure robustness in real-world deployments.

Study Generalization of AI/ML CSI Prediction Across Deployment Variations

RAN4 is invited to evaluate how well AI/ML CSI prediction models generalize when training and deployment conditions differ, as this determines the scope of performance verification.

- Doppler spread / UE mobility – Examine robustness of prediction under varying mobility levels and channel dynamics.

- SNR range

- Impact on testing – Use findings to decide how many and which test cases are needed, allowing a more efficient but still reliable conformance framework.

# Conclusion

This contribution provides discussion on CSI prediction. The proposals captured are:

- Curriculum-Based SNR Training for AI/ML CSI Prediction

RAN4 is invited to perform CSI-prediction evaluation by combining clear test assumptions with a staged SNR training approach that improves model robustness to noise

- Baseline test conditions – Standardize practical channel estimation and SNR operating ranges to define the conditions under which performance requirements (e.g., SGCS thresholds, throughput ratios) will later be applied.

- Curriculum SNR training – Begin with high-SNR samples where CSI structure is clean, then progressively expand to moderate and lower SNR ranges to strengthen robustness.

- Improved convergence and generalization – Enable the model to learn stable spatial-temporal patterns first, reducing overfitting to noise and improving performance under realistic conditions.

Study Generalization of AI/ML CSI Prediction Across Deployment Variations

RAN4 is invited to evaluate how well AI/ML CSI prediction models generalize when training and deployment conditions differ, as this determines the scope of performance verification.

- Doppler spread / UE mobility – Examine robustness of prediction under varying mobility levels and channel dynamics.

- SNR range

- Impact on testing – Use findings to decide how many and which test cases are needed, allowing a more efficient but still reliable conformance framework.

# Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface