# 3GPP TSG-RAN WG4 Meeting #116bis	                                                R4-2513620

# Prague, CZ, 13 – 17 October, 2025

Agenda Item:	6.15.2.2

Source: 	Apple

Title:	Discussion on RRM performance requirements for Adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

RAN4 RRM performance requirements for Adaptation of common signal/channel transmission was initially discussed in the previous meeting. Summary can be found in [1]. In this contribution we provide further discussion on the open issues. After discussion, corresponding proposals are provided.

# Discussion

Issue 3-3-1: Test case list

Agreement

RAN4 to at least define the following SSB adaptation test case.

  - L3 intra-frequency measurement

RAN4 to discuss whether the following SSB adaptation test case list can be defined.

- L1-RSRP

- L1-SINR

- CBD

We don’t think RAN4 needs to define new test cases to cover all L1-RSRP, L1-SINR and CBD. Note that only periodicity can be changed in this procedure. the only thing that needs to be verified is whether UE can timely follow the new periodicity. From this aspect, those procedures are the same. Therefore, testing L1-RSRP is enough.

Proposal 1: adding L1-RSRP in test case list is enough. No need to consider L1-SINR, CBD and any other L1 procedures.

Issue 3-3-2: General principle

RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed.

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

In general we think all aspects above make sense.

Proposal 2: we are fine with the following proposals:

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

# Conclusion

In this contribution, we provide discussion on performance part of Adaptation of common signal/channel transmission. After discussion the following conclusions are provided:

Proposal 1: adding L1-RSRP in test case list is enough. No need to consider L1-SINR, CBD and any other L1 procedures.

Proposal 2: we are fine with the following proposals:

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

# Reference

- R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ercisson