3GPP TSG-RAN WG4 Meeting #116-bis												R4-2513631

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:			6.9.2.1

Source:	Huawei, HiSilicon

Title:	on TRP TRS tests

Document for:	approval

# Introduction

This contribution discusses post processing steps and ratio between size 1 and size 2 devices in the working procedure as captured in WF from RAN4 #116 [1].

# Discussion

Option 2 in step 6-g of the working procedure [1] as highlighted below removes 1 best and 1 worst performing data point from the data pool for this band. Data points from a measurement pool should not be removed unless they are proven to be faulty. Such action would alter the measurement pool artificially and should be precluded.

Proposal 1: remove option 2 under step 6-g from the working procedure as measurement data should not be removed unless proven faulty.

  - FFS whether RAN4 will introduce a methodology to alter the device data pool according to the following options:

    - Option 1: Use the device data pool as is

    - Option 2: Assuming the total number of devices in a given band is > [50], remove 1 best and 1 worst-performing data point from the data pool for this band

    - Option 3: Further discuss a mechanism to identify and re-measure faulty devices; possible mechanism options are as follows:

      - Option 3a: define a threshold under which the device is considered a faulty sample

      - Option 3b: Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

      - Option 3c: Analyse the CDF curve to identify deviations of the lower tail and therefore a faulty sample of the device

      - Other options are not precluded

    - Other options are not precluded

The WF [2] in a previous meeting agrees a single set of requirements for devices of size 1 and size 2 for non-defined bands. This means that requirements derived from measurement data of size 1 equally applies to device of size 2, and vice versa. However, current working procedure [1] step 5-a-iv-6 imposes a ratio of numbers of devices with size 1 and size 2. Constraint using such a ratio is completely unnecessary due to the agreement [2] of size 1 and size devices to have the same requirements.

Proposal 2: remove step 5-a-iv-6 in the current working procedure as imposing a ratio between numbers of size 1 and size 2 devices is against the previous agreement in R4-2508782.

- Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: target to be =TBD%

        - If for any band with the number of measured data points < TBD this threshold is not met, RAN4 can request the test labs to measure up to TBD additional devices to shift the ratio closer to the target value

# Conclusion

This contribution provides the following proposals.

Proposal 1: remove option 2 under step 6-g from the working procedure as measurement data should not be removed unless proven faulty.

Proposal 2: remove step 5-a-iv-6 in the current working procedure as imposing a ratio between numbers of size 1 and size 2 devices is against the previous agreement in R4-2508782.

# Reference

- R4-2512693 Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3

- R4-2508782 Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3