3GPP TSG-RAN WG4 Meeting #116BIS							   R4-2513759

Prague, Czech, 13th – 17th Oct, 2025

Agenda Item: 	7.1.3

Source: 	MediaTek Inc.

Title: 	Coexistence evaluation and detail parameters on TN FWA 4x26dBm device in Rel-20

Document for: 	Discussion & Decision

# Introduction

In the last RAN plenary # 109 Meeting, RAN approved the UE RF enhancements for NR FR1, Phase 5 WI [1] as following work scope. In this paper we propose our view how to coexistence evaluation to derive ACLR/ACS for UE side and initial discuss the detail coexistence deployments scenarios and RF parameters for TN FWA high power UEs.

- 4x26dBm (up to 32dBm) high Power UE for FWA only

- 2x26dBm PC1.5 UE in FDD band for both HHUE and FWA

- Feasibility study from 4Q 2026 as following two sub-topics (start 4Q 2026)

  - PC1.5 1Tx (1x29dBm) for TDD bands for FWA UE

  - PC1 2Tx (2x29dBm) for TDD bands for FWA UE

# Coexistence evaluation for 4x26dBm FWA UE

## Coexistence study for High power 4x26dBm (up to 32dBm) FWA in TN

RAN#109 Plenary approved the following Objectives for TN high power related WID in Rel-20 [1].

High power UE (HPUE) for NR single carrier operation

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

- Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, n104

- Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- This work is to start in 2026 Q4

Based on RAN plenary agreements and the current RAN4 specification status, RAN4 need to evaluate coexistence study how the up to 32dBm TN FWA UE can protect the legacy system in 5G example operating bands.

Because only NTN PC1 (up to 31dBm) HP FWA UE’s RF requirements are specified including UE ACLR/ACS in TS38.101-5, still RAN4 do not define the TN 4x26dBm (up to 32dBm) FWA UE in TS38.101-1 until now. Therefore, we proposed as follow.

Proposal1: RAN4 needs to coexistence study for 4x26dBm FWA UE to derive both UE/gNB ACLR/ACS requirements to protect legacy 5G system in a 5G example operating band, e.g. n77.

## TN NR Deployment Parameters

In Table 1, we share the detail deployment parameters for TN UE as follow

Table 1. Simulation assumptions of TN NR deployment parameters

|  | NR |
| --- | --- |
| Carrier frequency in GHz | n77 (3.3~4.2GHz) |
| Size of each nominal channel BW in MHz | 100 |
| Transmission bandwidth in MHz | N/A |
| Environment | Deployment scenario in Urban Macro and Dense Urban in Table 2 |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | Deployment scenario in Urban Macro and Dense Urban in Table 2 |
| Coexistence scenarios | - 1) Aggressor: PC1BIS FWA Victim : legacy NR UE - 2) Aggressor: PC1BIS FWA Victim : legacy NR gNB |
| DL subcarrier spacing | 30 kHz |
| UL | DFT-s-OFDM |
| DL power control | No |
| UL power control | TR 36.942 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | 1 |
| Number of scheduled UE per cell (UL) | 3 |
| UE antenna height in meter | 1.5m |
| UE antenna gain in dBi | 0 |
| Building penetration loss | In pathloss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 5 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901 |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 |

| Link-level performance model | Throughtput-SINR mapping |
| --- | --- |
| UE distribution | Uniform |
| Evaluation metrics | NR: Average throughput loss and 5%-ile throughput loss should be less than 5%. |

Table 2: Dense Urban Deployment-related parameters of TN (n77)

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 65.03 m |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 450 m is assumed. |  |  |  |

Table 3: Urban Macro Deployment-related parameters of TN (n77)

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 450 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20/0% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

To derive adjacent channel coexistence evaluation, RAN4 shall consider the NR general deployment parameters in among Table 1,2,3 for FWA 4x26dBm UE.

## UE RF parameters for 4x26dBm FWA device

FWA antenna capability

We consider in the Table 4 below the need for at least the following FWA device to be considered as mandatory capability to evaluate coexistence in 5G NR operating band, e.g., n77.

Table 4. Examples of possible 5G-A FWA capabilities for around 4GHz

| Device types | Level of constraints | Example Capabilities Envelope |  |  |
| --- | --- | --- | --- | --- |
|  |  | Duplex | Antennas | power class (dBm) |
| FWA form factor | Low | 5G TDD | 4T/4R | 4x26 dBm (up to 32dBm) |

Proposal 2: RAN4 to consider the FWA antenna capability with 4Tx/4Rx for up to 32dBm FWA UE in 5G NR example bands, e.g., n77.

FWA Noise figure & power dynamic range

According to TR 38.921, a noise figure of 9 dB is used for HHUE as the baseline assumption, and 13 dB is used as the optional assumption for the co-existence evaluation for the 6425 – 7125 MHz and 10 - 10.5 GHz bands.

Therefore, RAN4 can consider 9 dB Noise figure for coexistence evaluation.

For the power dynamic ranges of FWA, RAN4 can consider existing agreements in TR38.921 from -33dBm to max. output power (up to 32dBm). Hence, the power dynamic range of FWA device can be decided as 65 dB for 100 MHz channel bandwidth.

Proposal 3: RAN4 consider 9 dB Noise figure for FWA as same in TR38.921. Also, the power dynamic range can be decided as 65 dB (from -33dBm to 32 dBm) for 100 MHz channel bandwidth.

UE antenna characteristics

In TR38.922, RAN4 already agreed that the UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming. So same assumption will be considered for HP FWA device.

ACLR and ACS

Before the derive actual ACLR/ACS in n77 based on coexistence evaluation, RAN4 can assume the existing ACLR and ACS to verify the coexistence evaluation by SLS simulation.

Table 5. UE/BS ACLR/ACS assumptions for 4x26dBm FWA device for around 4GHz

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 45 dB |
| UE | ACLR | 37/34/31 dB |
|  | ACS | 33 |

Proposal 4: For the UE/BS ACLR/ACS, Table 5 will be assumed for coexistence evaluation. If UE parameters are not mentioned in the paper, then it will consider the UE RF requirements for single carrier in TS38.101-1.

## BS parameters

BS antenna array size for urban macro

In TR38.922, RAN4 defined the BS antenna characteristics as shown in Table 6. Hence, BS the antenna characteristics will be assumed for coexistence evaluation of FWA 4x26dBm Device in around 4GHz frequency range.

Also, the significant path loss at higher frequencies necessitates a greater number of antennas to maintain adequate signal strength and coverage. From the observation, RAN4 can assumed at least 192 antenna elements (AE) and up to 256 AE for BS antennas are assumed to guaranteed which can be address a particularly problematic for cell-edge users.

Proposal 5: RAN4 assume the BS antenna characteristics in Table 6 as baseline. Furthermore, RAN4 can update the number of BS antenna elements at least 192 antenna elements (AE) and up to 256 AE to address a particularly problematic for cell-edge users.

Table 6: IMT parameters relevant for 1710 to 4990 MHz [3]

| Parameter | Macro Rural | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB | 30 dB |
| j3dB | 90 deg. | 90 deg. | 90 deg. | 90 deg. |
| q3dB | 65 deg. | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | 0.7l m | N/A |
| qsubtilt | 3 deg. | 3 deg. | 3 deg. | N/A |
| M | 4 | 4 | 4 | 8 |
| N | 8 | 8 | 8 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 2.1l m | 0.7l m |
| qetilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| jescan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. |
| r | 1 | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 46 dBm | 37 dBm |
| qmech | 3 deg. | 6 deg. | 6 deg. | N/A |

# Summary

In this paper, we provide our views on the coexistence evaluation necessity and related coexistence detail scenarios and parameters for FWA 4x26dBm (up to 32dBm) device. Based on the above analysis, we propose as following proposals in paper.

Proposal1: RAN4 needs to coexistence study for 4x26dBm FWA UE to derive both UE/gNB ACLR/ACS requirements to protect legacy 5G system in a 5G example operating band, e.g. n77.

Proposal 2: RAN4 to consider the FWA antenna capability with 4Tx/4Rx for up to 32dBm FWA UE in 5G NR example bands, e.g., n77.

Proposal 3: RAN4 consider 9 dB Noise figure for FWA as same in TR38.921. Also, the power dynamic range can be decided as 65 dB (from -33dBm to 32 dBm) for 100 MHz channel bandwidth.

Proposal 4: For the UE/BS ACLR/ACS, Table 5 will be assumed for coexistence evaluation. If UE parameters are not mentioned in the paper, then it will consider the UE RF requirements for single carrier in TS38.101-1.

Proposal 5: RAN4 assume the BS antenna characteristics in Table 6 as baseline. Furthermore, RAN4 can update the number of BS antenna elements at least 192 antenna elements (AE) and up to 256 AE to address a particularly problematic for cell-edge users.

# References

- [1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5,” Moderator (vivo)

- [2] TR38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”

- [3] TR38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”

- [4] TR38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects