3GPP TSG-RAN WG4 Meeting #116bis		R4-2513797

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:		6.12.2.2

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 MIMO Part2

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of performance part of this WI are described in [1] as:

- Specify necessary performance requirements for the enhancements listed above.

In last RAN4#116 meeting, RAN4 finalized the core part discussion on RRM core requirements. In last RAN4#116 meeting, the open issues of RRM performance part of Rel-19 MIMO Phase 5 are captured in [2]. In this contribution, we provide our view of the RRM performance part of Rel-19 MIMO Part2.

- Discussion

In last RAN4#116 meeting, RAN4 finalized the RRM core requirements.

In this meeting, we want to finalize the test case list.

Test cases

- Test cases of CJTC reporting

In last RAN4#116 meeting, the open issues of test scope are captured in in [2]:

#### Issue 2-2-3: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Nokia; Ericsson; MTK. Whether to define TCs to verify CJTC-Dd, measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-f，measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-P， (To be verify?); N; N; N. Whether to define TCs to verify scheduling restriction; Y (if core req.). Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy; Y; Y. end

RAN4 introduced the core requirements for delay and frequency offset, UE shall measure and report the measurement during measurement period and pass accuracy test.

For the TCs design for CJTC reporting, RAN4 can add TCs to verify the measurement period and accuracy in the same test.

For the scheduling restriction and measurement restriction, RAN4 specified the core requirements for mixed numerology. We prefer to add one test case for scheduling restriction. For measurement restriction, it’s fine for no test case.

Proposal 1: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

Proposal 2: Test case list for CJTC reporting:

TC1: FR1 CJTC-Dd measurement period and accuracy

TC2: FR1 CJTC-F measurement period and accuracy

TC3: FR1 CJTC-Dd scheduling restriction

- Asymmetric DL sTRP/UL mTRP scenarios

In last RAN4#116 meeting, the open issues of test scope are captured in in [2]:

#### Issue 2-3-1: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Huawei; Nokia; MTK. Whether to define TCs to verify timing requirements?; Y; Y; Y; Y; Y. Whether to define TCs to verify TCI state switching requirements?; Y; N. FR1 and/or FR2?; both. Whether to cover Scenario#1 and Scenario#2?; Both for FR1. Scenario#2 for FR2; Both; Both; Scenarios#1; both; Scenarios#1. End

In this objective, two TAs are enhanced for the UL-TRPs for core requirements. For the TCI state switching, the legacy requirements are reused, no need to add new test cases.

Proposal 3: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

Proposal 4: Test case list for Asymmetric DL sTRP/UL mTRP scenarios:

TC1: Scenario#1 uplink transmission timing two TAG FR2

TC2: Scenario#2 uplink transmission timing two TAG FR2

TC3: Scenario#1 uplink transmission timing two TAG FR1

TC4: Scenario#1 uplink transmission timing two TAG EN-DC FR1

TC5: Scenario#1 uplink transmission timing two TAG EN-DC FR2

- Conclusion

In this contribution, we analyze the potential impacts on RRM performance requirements for Rel-19 MIMO part2 and provide our proposals:

Proposal 1: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

Proposal 2: Test case list for CJTC reporting:

TC1: FR1 CJTC-Dd measurement period and accuracy

TC2: FR1 CJTC-F measurement period and accuracy

TC3: FR1 CJTC-Dd scheduling restriction

Proposal 3: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

Proposal 4: Test case list for Asymmetric DL sTRP/UL mTRP scenarios:

TC1: Scenario#1 uplink transmission timing two TAG FR2

TC2: Scenario#2 uplink transmission timing two TAG FR2

TC3: Scenario#1 uplink transmission timing two TAG FR1

TC4: Scenario#1 uplink transmission timing two TAG EN-DC FR1

TC5: Scenario#1 uplink transmission timing two TAG EN-DC FR2

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung