3GPP TSG-RAN WG4 Meeting #116bis		R4-2513800

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:		6.13.3.2

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 SBFD operation

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#108 meeting. The objectives of performance part of this WI are described in [1] as:

The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

In last RAN4#116 meeting, RAN4 finalized the core part discussion on RRM core requirements for Rel-19 SBFD. In last RAN4#116 meeting, the open issues of RRM performance part of Rel-19 SBFD are captured in [2]. In this contribution, we provide our view of the RRM performance part of Rel-19 SBFD operation.

- Discussion

In last RAN4#116 meeting, RAN4 finalized the RRM core requirements for Rel-19 SBFD.

The performance requirements can be divided into two parts: Measurement accuracy and Test cases:

2.1 Test Cases

In last RAN4#116 meeting, the open issues of RRM performance part for reporting mapping are captured in [2]:

#### Sub-topic 3-3: Test cases #### Issue 3-3-1: General test setup #### Issue 3-3-2: Test case list #### Issue 3-3-3: Test case details and test purpose #### Issue 3-3-4: Test case scenarios Issue 3-3-5: Test applicability

In core requirements for Rel-19 SBFD, RAN4 finalized the new core requirements for SBFD operation:

- CSI-RS based RLM/BFD/CBD evaluation

- CSI-RS based L1-RSRP evaluation

- CSI-RS based L1-SINR evaluation

- CSI-RS L3 measurement

In general, RAN4 should add new test cases to verify the new core requirements above.

For SSB based measurement, although there are no new core requirements. RAN4 to reuse the existing requirements. However, SSB measurements are quite important. We prefer to add test cases for SBFD-aware UE to verify the SSB measurement as well.

Agreement in RAN4#112 Issue 2-3-2: Requirements for SSB based measurement Agreement: - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell. Agreement in RAN4#113 #### Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

In last RAN1#122 meeting, RAN1 has agreed that across two DL subbands is a basic UE feature for SBFD-aware UE in [3]. Thus, we support to add the TCs for DUD case.

Proposal 1: RAN4 to add new test cases for SBFD-aware UE for SBFD operation:

- SSB based measurement for L1 and L3 measurements.

- CSI-RS based RLM/BFD/CBD evaluation

- CSI-RS based L1-RSRP evaluation

- CSI-RS based L1-SINR evaluation

- CSI-RS L3 measurement

For L1 measurement, RAN4 to add the TCs to verify the measurement resource is collision with UL transmission.

For the general test configuration like DU and/or DUD case, we support DUD case.

For the CSI-RS based L1-RSRP and L1-SINR, the behavior and requirements are similar. We can pick up one for FR1 test and the other for FR2 test.

Proposal 2:

Test case list for SBFD operation:

TC1: FR1 SSB based measurement for L1 measurement for SBFD-aware UE

TC2: FR2 SSB based measurement for L1 measurement for SBFD-aware UE

TC3: FR1 SSB based measurement for L3 measurement for SBFD-aware UE

TC4: FR1 SSB based measurement for L3 measurement for SBFD-aware UE

TC5: FR1 CSI-RS based RLM

TC6: FR2 CSI-RS based RLM

TC7: FR1 CSI-RS based BFR

TC8: FR2 CSI-RS based BFR

TC9: FR1 CSI-RS based L1-RSRP

TC10: FR2 CSI-RS based L1-SINR

TC11: FR1 CSI-RS based L3 measurement

TC12: FR2 CSI-RS based L3 measurement

- Conclusion

In this contribution, we analyze the potential impact on RRM performance part of Rel-19 SBFD CLI and provide our proposals:

Proposal 1: RAN4 to add new test cases for SBFD-aware UE for SBFD operation:

- SSB based measurement for L1 and L3 measurements.

- CSI-RS based RLM/BFD/CBD evaluation

- CSI-RS based L1-RSRP evaluation

- CSI-RS based L1-SINR evaluation

- CSI-RS L3 measurement

For L1 measurement, RAN4 to add the TCs to verify the measurement resource is collision with UL transmission.

For the general test configuration like DU and/or DUD case, we support DUD case.

Proposal 2:

Test case list for SBFD operation:

TC1: FR1 SSB based measurement for L1 measurement for SBFD-aware UE

TC2: FR2 SSB based measurement for L1 measurement for SBFD-aware UE

TC3: FR1 SSB based measurement for L3 measurement for SBFD-aware UE

TC4: FR1 SSB based measurement for L3 measurement for SBFD-aware UE

TC5: FR1 CSI-RS based RLM

TC6: FR2 CSI-RS based RLM

TC7: FR1 CSI-RS based BFR

TC8: FR2 CSI-RS based BFR

TC9: FR1 CSI-RS based L1-RSRP

TC10: FR2 CSI-RS based L1-SINR

TC11: FR1 CSI-RS based L3 measurement

TC12: FR2 CSI-RS based L3 measurement

- Reference

[1] RP-251874, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei

[3] R4-2513004, LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122, RAN1