3GPP TSG-RAN4 Meeting #116bis	R4-2513836

Prague, Czech Republic, 13 October – 17 October 2025

Source:			Ericsson

Title:			US 4.9 GHz band – UE RF aspects

Agenda item:		5.11.4

Document for:		Approval

# Introduction

In RAN#108, a WI ([1]) has been approved to introduce a new 4.9GHz band for US.

In last RAN4#116 meeting, as captured in Chair notes ([3]), RAN4 agreed to conclude the study phase, meaning also that the specification work could start from this meeting. This contribution is discussing the UE RF requirements, analyzing the relevant regulation.

# Discussion

## Background

The Commission allocated the 4.9 GHz band for public safety use in 2002. Since then, the rules have evolved in response to changing conditions, including band use and technological innovation. In 2020, to stabilize the band, the Public Safety and Homeland Security Bureau (PSHSB) and the Wireless Telecommunications Bureau (WTB) announced a freeze on applications in this band. In January 2023, the Seventh Report and Order and Ninth Further Notice established a comprehensive and coordinated nationwide approach to managing the 4.9 GHz band, adding the role of a nationwide Band Manager. In the Eight Report and Order ([2]), FCC enforces the role of the Band Manager, further describing his role and responsibilities.

It should also be noted that the existing NR band n79 (4.4-5.0 GHz) encompasses this new 4.9 GHz band.

## FCC part 90 and 3GPP impacts

### Overview

The new 4.9 GHz US band is a public safety band and is specified in FCC §90. The following sub-parts are notably key inputs for the specification of this new band in 3GPP:

- §90.1213: band plan.

- §90.1215: Power limits.

- §90.210: Emission masks.

### Power limits (§90.1215)

FCC specified the following maximum conducted output power (see below table), mentioning that higher power devices, using other channel bandwidth or not, are limited to a peak PSD of 21dBm/MHz.

According to §90.1215(2), for devices transmitting with antennas of directional gain greater than 9 dBi, the maximum conducted output power and the peak PSD mentioned above shall be reduced by the corresponding exceeding value.

Observation 1: Considering on the requested channel bandwidths (10-50 MHz), PC3 UE complies with the FCC power limits for the US 4.9GHz band.

Further discussion would be needed in RAN4 on how to best handle the 21dBm/MHz PSD limit.

### Emission Masks (§90.210)

FCC specified 2 emissions masks in 90.210: mask L for low power transmitter (20dBm or less) and mask M for high power transmitter (greater than 20 dBm). When specifying the US 4.9 GHz band, UE and BS shall then follow the emission mask M.

First, it should be noted that the FCC emission mask M starts inside the channel bandwidth: (m)(2) specifies a limit in the 45-50% of the authorized bandwidth. This means that, for NR channel bandwidth with a spectrum utilization larger than 90%, the edge RB(s) might not be usable or, at least, they would have to be de-boosted.

Observation 2: The FCC mask M starts inside the 3GPP channel bandwidth domain. Some edge RBs of the maximum transmission bandwidth configuration might not be possible to use.

Moreover, FCC specifies a resolution bandwidth of at least 1% of the occupied bandwidth. This means that:

- For channel bandwidth < 50 MHz, the 3GPP SEM could be directly compared to FCC emission mask M, using a measurement BW of either 1% of the channel BW (when ΔfOOB ≤ 1 MHz) or 1 MHz (when ΔfOOB > 1 MHz), as shown in Figure 1 – right side and Figure 1– left side respectively.

- For channel bandwidth ≥ 50MHz:

  - When ΔfOOB ≤ 1 MHz, the 3GPP SEM is defined with a measurement bandwidth of 30 kHz. The corresponding 3GPP requirement should then be converted to 1 MHz before comparing to FCC mask M for this frequency range, as shown in Figure 2 - right side.

  - When ΔfOOB > 1 MHz, the 3GPP SEM could be directly compared to FCC emission mask  using 1 MHz measurement bandwidth, as shown in Figure 2 - left side.

Figure 1: Comparison of 3GPP SEM with FCC Mask M for 10 MHz channel BW

Figure 2:  Comparison of 3GPP SEM with FCC Mask M for 50 MHz channel BW

As it could be observed on those figures, for all UE PCs, the FCC Emission Mask M is more stringent than the FCC SEM. A new NS shall be specified with this FCC emission mask M and corresponding A-MPR should be evaluated.

Observation 3: The FCC mask M is more stringent then the 3GPP UE SEM mask.

Also, the FCC emission mask applicability goes beyond the 3GPP SEM domain, in the spurious domain, where it’s also more stringent.

Observation 4: The FCC mask M covers the 3GPP SEM domain (including inside the channel bandwidth, at 5% of the edge) and the 3GPP spurious domain.

Based on those observations, we make the following proposals.

Proposal 1: Specify a new NS for the US 4.9GHz band with the FCC emission limits specified in FCC §90.210. This new NS will indicate additional emission limits (including inside the channel bandwidth, at 5% of the edge) and additional spurious limit.

Proposal 2: Study and specify the corresponding A-MPR if needed.

Observation 5: This new NS could also be used to indicate the limited peak power spectral density of 21dBm/MHz.

## TS 38.101-1 impacts overview

The following Table 2 gives an overview of the TS 38.101-1 impacts, based on the different aspects discussed above.

Table 1: TS 38.101-1 impacts overview

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 6. Transmitter characteristics |  |  |
| 6.1 General | No |  |
| 6.2 Transmitter power |  |  |
| 6.2.1 UE maximum output power | Yes | Add the new US 4.9GHz band in the UE power class, supporting PC3. |
| 6.2.2 MPR | Tbc | Evaluate if any MPR when considering emission mask M in the 45%-50% of the authorized BW, see 2.2.4.1 |
| 6.2.3 A-MPR | Yes | New NS shall be added and A-MPR shall be further studied, see 2.2.4.1 |
| 6.2.4 Configured transmitted power | No |  |
| 6.3 Output power dynamics | No |  |
| 6.4 Transmitted signal quality | No |  |
| 6.5 Output RF spectrum emissions |  |  |
| 6.5.1 Occupied bandwidth | No |  |
| 6.5.2 Out of band emission | Yes |  |
| 6.5.2.1 General | No |  |
| 6.5.2.2 Spectrum emission mask | No |  |
| 6.5.2.3 Additional spectrum emission mask | Yes | Add an additional requirement corresponding to the new NS in clause 6.5.2.3 as described in 2.2.4.1 |
| 6.5.2.4 ACLR | No |  |
| 6.5.3 Spurious emission |  |  |
| 6.5.3.0 General | No |  |
| 6.5.3.1 General spurious emissions | No |  |
| 6.5.3.2 Spurious emissions for UE co-existence | Yes | Add the new US 4.9GHz band in Table 6.5.3.2-1, most likely considering similar exceptions as for n79. |
| 6.5.3.3 Additional spurious emission | Yes | Add an additional requirement corresponding to the new NS in clause 6.5.2.3 as described in 2.2.4.1 |
| 6.5.4 Transmit intermodulation | No |  |
| 7. Receiver characteristics |  |  |
| 7.1 General | No |  |
| 7.2 Diversity characteristics | No |  |
| 7.3 Reference sensitivity | Yes | Add the new US 4.9GHz band in the TDD REFSENS table and corresponding UL configuration, most likely based on n79 |
| 7.4 Maximum input level | No |  |
| 7.5 ACS | No |  |
| 7.6 Blocking characteristics |  |  |
| 7.6.1 General | No |  |
| 7.6.2 In-band blocking | Yes | Add the new US 4.9GHz band in the in-band blocking table for frequency above 3300MHz |
| 7.6.3 Out of band blocking | Yes | Add the new US 4.9GHz band in the out of band blocking table for frequency above 3300MHz |
| 7.6.4 Narrow band blocking | Yes | Add the new US 4.9GHz band in the narrow band blocking table |
| 7.7 Spurious response | No |  |
| 7.8 Intermodulation characteristics | No |  |
| 7.9 Spurious emissions | No |  |
| 7.10 Power imbalance | No |  |

Proposal 3: Agree with Table 1 which lists the impacts to TS 38.101-1 when specifying the new US 4.9GHz band.

# Conclusion

In this contribution, we studied the FCC regulation for the new US 4.9 GHz band and made the following observations and proposals related to the UE RF impacts:

Observation 1: Considering on the requested channel bandwidths (10-50 MHz), PC3 UE complies with the FCC power limits for the US 4.9GHz band.

Observation 2: The FCC mask M starts inside the 3GPP channel bandwidth domain. Some edge RBs of the maximum transmission bandwidth configuration might not be possible to use.

Observation 3: The FCC mask M is more stringent then the 3GPP UE SEM mask.

Observation 4: The FCC mask M covers the 3GPP SEM domain (including inside the channel bandwidth, at 5% of the edge) and the 3GPP spurious domain.

Proposal 1: Specify a new NS for the US 4.9GHz band with the FCC emission limits specified in FCC §90.210. This new NS will indicate additional emission limits (including inside the channel bandwidth, at 5% of the edge) and additional spurious limit.

Proposal 2: Study and specify the corresponding A-MPR if needed.

Observation 5: This new NS could also be used to indicate the limited peak power spectral density of 21dBm/MHz.

Proposal 3: Agree with Table 1 which lists the impacts to TS 38.101-1 when specifying the new US 4.9GHz band.

# References

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- Eight Report and Order, WP Docket No. 07-100, 22 October 2024.

- RAN4 Chair notes: https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_116/Inbox/Chair_Notes/Main_Session_Notes/1.Mon/7.Draft_R4-2509228_%5B116%5D%5B115%5D_US4.9GHz_Topic_Summary_moderator.docx