# 3GPP TSG-RAN WG4 Meeting #116bis		R4-2513862

Prague, Czech Republic, 13th to 18th October 2025

Agenda Item:	6.8.3.1

Source:	Ericsson

Title:	On Transmitter Spurious Emission Co-existence discussion

Document for:	Approval

# 1	Introduction

LS from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence Tx spurious emission requirement levels for BS. It was acknowledged by RAN4 in the response LS [2].

The co-existence spurious emission requirement was originally derived in 1999 for UTRA. The technical background at that time was very much focused on the introduction of UTRA FDD/TDD operating at 2 GHz. The assumptions used as references to the original requirement definition can be found in a contribution [3] presented at RAN4#113 in Orlando. Over the years RAN4 have defined new operating bands below and above 2 GHz using the same requirement level. In current specification, bands are defined within the range 410 MHz up to 7125 MHz with the same requirement level, except for a few occasions (e.g. protection of GSM). Therefore, it is now time to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

The objective can be summarized as:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

In this contribution, we share our calibration results and considerations to trigger further discussions on open issues.

# 2	Discussion

In the RAN4#116 meeting, it was agreed to follow a stepwise approach to align the simulator before there is any discussion on the requirement levels. The agreements related to previous WF [4] for calibration are captured below –

- ISD – 500m

- Grid shift – 100%

- Correlation value for out of band antenna gain – First 0 and then 1 (depending on the calibration step)

- Evaluation scenarios – Wide Area (WA) for BS-to-BS (Urban Macro in TR 38.922) and Medium Range (MR) for BS-to-UE (Dense Urban in TR 38.922)

- Pathloss model – FFS

- Simulation frequency – 2 GHz for aggressor system and 6.4 GHz for victim system

During the offline discussions, further informal agreements were agreed to assume LoS conditions for BS-to-BS propagation and consider Free Space Path Loss model for the calibration activity.

Further down this contribution, Section 2.1 presents the calibration results for BS-to-BS Urban Macro case, Section 2.2 presents the calibration results for BS-to-UE Urban Micro case and Section 2.3 discusses the next steps after the calibration activity.

## 2.1	BS-to-BS Urban Macro case

### 2.1.1	Calibration Assumptions

The assumptions are with 100% Outdoor UEs, wrap around on and the results are cross-network i.e. Aggressor BS of Network 1 to Victim BS of Network 2. Other assumptions are highlighted below -

- Aggressor

  - Transmitting frequency – 2 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – full array gain

  - BS to BS Propagation model – FSPL

  - BS to UE Propagation model - TR 38.901

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 6.4 GHz

  - Antenna model – AAS with 8x16 array and 3x1 subarray antenna configuration

  - Gain at Rx – full array gain

  - Propagation model – FSPL

  - Antenna height – 25m

### 2.1.2	Calibration Results

#### 2.1.2.1	Step 1 – Only Pathloss

Figure 2.1.2.1-1 shows the BS-to-BS Path Loss between different networks. The Path loss is deterministic and defined only by the assumed Free space model.

Figure 2.1.2.1-1 BS to BS Path Loss

#### 2.1.2.2	Step 2 – Pathloss + aggressor element gain at Tx

Figure 2.1.2.2-1 shows the BS to BS path loss plus element gain at the aggressor network. The model of the antenna at the aggressor transmitter and victim receiver in the Urban Macro scenario include subarrays.

Figure 2.1.2.2-1 BS to BS Path Loss with aggressor transmitter subarray gain.

#### 2.1.2.3	Step 3 – Pathloss + aggressor array gain at Tx

Figure 2.1.2.3-1 shows the BS to BS path loss plus the aggressor full array gain between two networks. The gain does not account for dual polarization, as we assume that the two polarizations would be uncorrelated at the victim receiver BS.

Figure 2.1.2.3-1 BS to BS Path Loss with aggressor transmitter array gain.

#### 2.1.2.4	Step 4 – Path loss + aggressor array gain at Tx + victim array gain at Rx

Finally, Figure 2.1.2.4-1 shows the BS to BS coupling loss between two networks considering both the array gain at the aggressor transmitter and the victim receiver. The result doesn’t include the polarization gain.

Figure 2.1.2.4-1 BS to BS Path Loss with aggressor transmitter and victim receiver array gain.

## 2.2	BS-to-UE Dense Urban case

### 2.2.1	Calibration Assumptions

The assumptions are with 80% Indoor UEs, wrap around on and the results are cross-network i.e. Aggressor BS of Network 1 to Victim UE of Network 2. Other assumptions are highlighted below -

are as follows –

- Aggressor

  - Transmitting frequency – 2 GHz

  - Antenna model – AAS with 4x8 array and 3x1 subarray antenna configuration

  - Gain at Tx – full array gain

  - Propagation model – TR 38.901

  - Minimum BS - UE distance (2D) – 35 m

  - Antenna height – 25m

- Victim

  - Transmitting frequency – 6.4 GHz

  - Isotropic UE with 0 dBi gain and without beamforming

  - Propagation model – TR 38.901

### 2.2.2	Calibration Results

#### 2.2.2.1	Step 1 – Only Pathloss

Figure 2.2.2.1-1 shows the Path Loss between the aggressor transmitter BS and the victim receiver UE.

Figure 2.2.2.1-1 BS to UE Path Loss, between the two networks

#### 2.2.2.2	Step 2 – Pathloss + aggressor element gain at Tx

Figure 2.2.2.2-1 shows the path loss plus the aggressor transmitter element gain. The result doesn’t include the polarization gain.

Figure 2.2.2.2-1 BS to UE Path Loss with antenna element gain, between the two networks

#### 2.2.2.3	Step 3 – Pathloss + aggressor array gain at Tx

Figure 2.2.2.3-1 shows the BS to UE Path Loss with array gain between the two networks, i.e. between the aggressor transmitter BS and the victim receiver UE. The result doesn’t include the polarization gain.

Figure 2.2.2.3-1 BS to UE Path Loss with antenna element gain, between the two networks

It has been highlighted during the email discussions that the antenna gain should also account for the additional 3 dB polarization gain. However, it is not clear the motivation behind adding this additional gain in the context of this study. In previous studies, like e.g. TR 38.803, there is a Note to add an additional 3 dB polarization gain to the BF gain. However, the context of this assumption refers to the serving link and is not clear if this assumption would make sense for the interfering link, out of band, as is the case in this study. Specifically, in the current study, we look at cross network coupling from the aggressor BS to the victim BS/ UE with the boresight towards the serving UE, from one band to another. In this view, off-boresight, the gain should come only from the actual radiation pattern of the two polarizations and not through the artificial 3 dB additional gain.
The need for this polarization gain should be clarified in the discussions during the meeting. Also, it would be important to clarify if the dual polarization is modelled by adding 3 dB to the antenna element, so that the impact would be baked into every direction of the radiation pattern. This way both the boresight and sidelobes (towards the victim BS/ UE) will also inherit the 3 dB gain.

Further clarification is needed on the assumption of the additional 3 dB polarization gain, which we interpret from previous studies primarily intended for the serving link and not the interfering link.

## 2.3	Considerations to derive the requirement level

### 2.3.1	Modelling of antenna gain for unwanted emissions

The model equation for analyzing decorrelation effects in advanced antenna systems (AAS) was first introduced in 3GPP Rel-11 as part of the AAS Study Item documented in TR 37.840. This equation provided a mathematical framework to capture the relationship between spatial separation, correlation, and system performance. However, at that time, RAN4 could not reach agreement on the most appropriate roll-off profile, which describes how correlation decreases as spatial separation increases. The absence of a consensus meant that the model remained incomplete in this respect, limiting its practical applicability for performance evaluations and simulations.

In Rel-19, a profile was finally introduced in TR 38.922 providing a reference curve for modeling decorrelation in certain deployment scenarios. This topic has also been discussed in other standardization bodies, including ITU-R and ECC, though no firm conclusions have been reached so far. Ultimately, RAN4 remains the expert group responsible for deciding on the most appropriate model for this purpose.

The equation describes the composite far field antenna gain for an AAS in the presence of partial spatial correlation – particularly relevant for unwanted emissions.

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1

As can be seen in the equation, the composite antenna pattern is the sum of the subarray element gain and an array gain component which depends on the array geometry, beamforming vector and element correlation. This model accounts for spatial decorrelation due to spectral separation from the carrier, which affects the coherent summation in the array factor. The key addition in this model is the ρ(f) where f=offset from carrier center, which models the roll-off of coherent gain as a function of offset from the carrier center frequency.

In the TR 38.922 Clause 7.3.2.1.2, a parameterized piece-wise linear roll-off model is suggested to capture fundamental aspects of the array antenna gain drop outside the wanted signal.

This function rolls off from:

- ρ(f)=1 when fc is [−0.5B, 0.5B] resulting in full coherent gain

- ρ(f)=0 when f = ±fo resulting in only subarray gain (no beamforming)

Many companies have raised concerns that assuming the correlation factor ρ(f) to be either 0 or 1 outside the desired carrier band is too simplistic and unrealistic for unwanted emissions from an Active Antenna System (AAS).

The unwanted emissions consist of a mix of two components:

- Correlated intermodulation distortion (IMD) products, which are partially coherent across antenna elements because they originate from the same nonlinear processes.

- Uncorrelated wideband transmitter noise, which is random and independent at each antenna element.

Because of this mixture, the correlation factor ρ(f) does not simply jump from 1 (fully correlated) inside the carrier to 0 (fully uncorrelated) immediately outside it. Instead, ρ(f) gradually decreases with frequency offset from the carrier edge as noise becomes more dominant and coherence diminishes. Assuming ρ(f)=1 outside the carrier tends to overestimate the coherent array gain, potentially underestimating interference levels, while assuming ρ(f)=0 can be too conservative by ignoring any partial correlation that exists.

For the calibration activity, we follow a stepwise approach and agreed to use ρ as 1.

A more realistic approach could be a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside.

Selecting ρ therefore balances between underestimating array gain by assuming too little correlation and overestimating it by assuming too much, making it crucial to choose a value that reflects the actual mixture of noise and distortion in the system.

The full array doesn’t make sense for integer multiples of the fundamental frequency as the antenna array and beamforming are primarily optimized for the fundamental frequency. Thus, the antenna efficiency drops reducing the effective radiated power at integer multiples regardless of the array size. We can also not ignore the fact that the physical size of the antenna elements is fixed but the wavelength λ changes with frequency which further drastically changes the radiation pattern, impedance input and efficiency. These combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain.

The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminate the possibility for the antenna array to achieve full array gain.

RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions.

As a tradeoff due to above reasons, we can look at 0.5 (partial coherent), as a way to model the partial coherence between array elements.

### 2.3.2	BS-to-BS coupling network grid shift

In Rel-99 TR 25.942, the BS-to-BS coupling loss was derived assuming an intermediate case scenario of 288m which reflects 50% grid shift. The layout is specified in Clause 5.1.3.1.2 and the Minimum Coupling Loss derivation is in Clause 10.2.1. To align with Rel-99 assumptions and considering what will depict a real deployment, grid shift 50% sounds reasonable.

RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation.

### 2.3.3	Percentile on the Coupling Loss CDF curves

In the RAN4#115 agreements, it was stated to use 99%ile as a starting point of the Coupling Loss CDF plots. However, in RAN4#116 it was decided to run a calibration activity as the results were spread between the participating companies. The discussion on what percentile to look at is still among the open issues. In Rel-99, the Minimum Coupling Loss which refers to 100% on the CDF curve was calculated towards the wanted UE or a neighboring base station. This value, however, is at the queue of the CDF, which is very sensitive to the simulation and can fluctuate across different simulation runs because the beams are dynamically adjusting, which might lead to variability in observed MCL values.

RAN4 to discuss the percentile on the Coupling Loss CDF curves, instead of the minimum value.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A more realistic approach could be a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside.

Observation 2	The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminate the possibility for the antenna array to achieve full array gain.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Further clarification is needed on the assumption of the additional 3 dB polarization gain, which we interpret from previous studies primarily intended for the serving link and not the interfering link.

Proposal 2	RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions.

Proposal 3	RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation.

Proposal 4	RAN4 to discuss the percentile on the Coupling Loss CDF curves, instead of the minimum value.

# References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[3]	   R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

[4]	   R4-2512578, “Way Forward on Tx spur co-ex”, RAN4#116