3GPP TSG-RAN WG4 Meeting #116bis	R4-2513874

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	7.5.3

Source:	Samsung

Title:	Discussion on Rel-20 performance requirements for FR1 SU-MIMO with spatial channel model

Document for:	Discussion

- Introduction

In the RAN#109 meeting, a new work item for “NR demodulation performance: Phase 6” has been approved [1], in which specifying optional performance requirements under spatial channel model (SCM) for SU-MIMO in FR1 is an important objective for the enhancement of UE demodulation performance parts. The detailed descriptions of this objective are as below:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

Generally, the UE demodulation performance requirements for SCM in this WI could be summarized as two parts. The first one is to specify PDSCH demodulation performance requirements with SCM, the second one is to study and specify PMI reporting performance requirements with SCM if performance alignment on PMI reporting is achieved.

For the SCM channel model which will be used for specifying performance requirements, rCDL-C1 channel model in TR38.753 has been agreed as the starting point [1]. rCDL-C1 [2] is a Clustered Delay Line (CDL) based channel model focusing on 3.5GHz carrier frequency, which starts from TR38.901 CDL-C constructed to represent one channel profile for NLOS, with angle scaling, randomness reduction (fixed coupling pattern for ray angles and fixed initial phases for polarization matrices in the channel coefficient generation), and cluster reduction (reduced to 12 clusters from 24 clusters).

Given that the study in TR38.753 on rCDL-C1 channel model is based on Rel-18 TR38.901, and only focus on 3.5GHz carrier frequency, further update to align with latest TR38.901 and further enhancement to implement frequency generalization are necessary. In addition, the evaluation results of rCDL-C1 are not aligned for PMI performance testing with eType-II codebook, three clusters of results can be observed [2], and current rCDL-C1 has fixed angular properties with non-uniform PMI statistics. Consequently, the PMI bias problem need to be studied and solved firstly, then check if the performance alignment on PMI reporting could be achieved.

In this contribution, we share our initial views and analysis on general scope and potential issues for FR1 SU-MIMO normative work under SCM channel model.

- Discussion

  - rCDL-C1 in TR38.753

Antenna model

In TR38.753 study for rCDL-C1, the antenna configurations used for BS side are

    - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

    - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

While the antenna configurations used for UE side are

    - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4 RX

    - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8 RX

Which means the antenna placements and polarizations are based on co-located cross-polarization antenna modelling for both BS and UE sides.

Furthermore, omni antenna radiation pattern is taken for UE side in TR38.753 study on rCDL-C1. Both co-located cross-polarization antenna modelling and omni antenna radiation pattern are not well aligned with the real network.

Observation 1: For rCDL-C1 in TR38.753, omni antenna radiation pattern and co-located cross-polarization antenna modelling are assumed for UE side.

Scaling of angles

In TR38.753 study for rCDL-C1, below angle scaling procedure is used for getting the scaled cluster angles with desired angular spread and direction. Particularly, the scaling factor is not only applied for cluster angles, but also applied for the tabulated cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD in Table 7.7.1-3 of 38.901.

38.753 5.1.3.1 Parameters for Angle Scaling

The angles need to be scaled for the scenario to achieve the desired angular spread and direction. The parameters for scaling the angles of arrival and departure are provided here. The CDL tables provided in 5.1.3.3 include the delay scaling for RMS delay spread of 365ns and desired angle scaling. The scaled cluster (φn,scaled) and scaled ray (φn,m,scaled) angles and cluster specific angular spread are obtained as described below for AOD, AOD, ZOA, and ZOD. φn,scaled=wrap(ASdesiredASmodel.wrap(φn,model-μφ,model)+μφ,model) (5.1.3.1-1) c{ASA, ASD,ZSA,ZSD}_scaled= ASdesiredASmodel. c{ASA, ASD,ZSA,ZSD}_model (5.1.3.1-2) φn,m,scaled= φn,scaled+c{ASA, ASD,ZSA,ZSD}_scaledαm (5.1.3.1-3) φn,model is the tabulated CDL cluster angle from 38.901 ASmodel is the rms angular spread of the tabulated CDL from 38.901 including the offset ray angles provided in Table 5.1.3.1-1 μφ,model is the power weighted mean angle of the tabulated CDL from 38.901, calculated using the definition in Annex A.2 of 38.901[7] cASA, ASD,ZSA,ZSD_model is the tabulated cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD from Table 7.7.1-3 of 38.901 [7]. ASdesired is the desired rms angular spread provided in Table 5.1.3.1-1 φn,scaled is the resulting scaled cluster angle. φn,m,scaled is the resulting scaled ray angle. cASA, ASD,ZSA,ZSD_scaled is the scaled cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD in Table 5.1.3.1-1. αm is the offset angle per ray from Table 7.5-3 of 38.901[7]

Observation 2: For rCDL-C1 in TR38.753, the scaling factor is not only applied for cluster angles, but also applied for the tabulated cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD in Table 7.7.1-3 of 38.901.

Carrier frequency

For rCDL-C1 in TR38.753, only FR1 3.5GHz carrier frequency is assumed for the performance evaluation.

Observation 3: For rCDL-C1 in TR38.753, only FR1 3.5GHz carrier frequency is assumed for the performance evaluation.

PMI bias and performance alignment

For rCDL-C1 in TR38.753, the angle values of CDL models are fixed, which is not very suitable for PMI reporting performance evaluation. The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with or even better than closed-loop or reciprocity beamforming since PMI reporting delay.

Besides, in the study phase for rCDL-C1 in TR38.753, the performance alignment for eType-II codebook is not achieved, the detailed descriptions about simulation results are as following. Before the second-round simulation campaign for PMI reporting performance evaluation with eType-II codebook, the PMI bias problem should be studied and solved firstly.

#### 8.2.1.1 4 layer, eType-II For PMI testing with 4 layer transmission, with rCDL-C1, in eType-II codebook case, three clusters of results can be observed; Cluster 1: include source #3, #7, Cluster 2: include #4, #5, #6, #9. The span of this cluster is < 2.5dB for both 70% and 90% normalized throughput percentiles, and Cluster 3 include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput percentiles.

Proposal 1: For rCDL-C1 in TR38.753, before the second-round simulation campaign for PMI reporting performance evaluation with eType-II codebook and SCM channel model, the PMI bias problem should be studied and solved firstly.

  - Analysis of potential issues

2.2.1 UE antenna model

As outcomes of RAN1 study item FS_NR_7_24GHz_Chmod, UT radiation power pattern, UT antenna placements and UT polarization are introduced in Rel-19 TR38.901 and copied as following, which are noteworthy for higher number of UE antennas modelling.

UT Radiation power pattern in TR38.901

Table 7.3-2: Radiation power pattern of a single antenna element for handheld UT Parameter; Values. Vertical cut of the radiation power pattern (dB); AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with θ3dB=125°,SLAV=22.5dB and θ″∈0°, 180°. Horizontal cut of the radiation power pattern (dB); AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with ϕ3dB=125°, Amax=22.5 dB and ϕ″∈-180°, 180°. 3D radiation power pattern (dB); AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax. Maximum directional gain of an antenna element GE,max; 5.3 dBi. NOTE: For UT antenna modelling of handheld devices, optional antenna imbalance can be modelled. If modelled, randomized loss is applied per UT antenna port and randomized loss can be applied independently for the UL and DL directions. No imbalance is modelled by default..

UE antenna placements for handheld UT devices and CPE device in TR38.901

For handheld UT devices, the following additional properties apply: - Four corners of the rectangle and the center of edges of the rectangle that reflect UT form factor are identified as potential locations of the UT antenna and potential locations are enumerated as illustrated in Figure 7.3-2. - Each antenna is assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location. - Reference UT orientation vector of the handheld UT is perpendicular to the plane of the flat UT handheld device, and reference point for near field phase calculation of the UT is assumed to be the center of the plane of the UT handheld. - Device dimensions for UT antenna modelling are (15 cm, 7 cm, 0 cm). Figure 7.3-2: Handheld UT antenna placement candidate locations relative to centre of device (top down view) … For consumer premise equipment (CPE) devices, the following properties apply: - Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. Note that a candidate antenna location (e.g., 9) can be used for multiple antennas. - Device dimensions for UT antenna modelling are (0 cm, 20 cm, 20 cm) Figure 7.3-6: CPE UT antenna placement candidate locations relative to centre of device (side view)

UT polarization direction defined in TR38.901

For cases when a candidate antenna placement location is used for one antenna field pattern (e.g., single polarization): - The polarization direction is indicated by the arrow in Figure 7.3-7, which is parallel with the plane of the handheld UT and perpendicular to the direction from the UT center to the candidate antenna location. Figure 7.3-7: Handheld UT antenna polarization directions for one antenna field pattern (top down view)

Considering one of the major test cases is 8T8R with 2 codewords, which has multiple number of UE antennas, while rCDL-C1 is generated based on omni antenna radiation pattern and co-located cross-polarization antenna modelling for UE side, which may not match the actual situation on the real network, RAN4 should clarify if the UT radiation power pattern, UT antenna placements and UT polarization direction defined in Rel-19 TR38.901 should be considered as the simulation assumption for FR1 SU-MIMO demodulation performance evaluation.

Observation 4: UT radiation power pattern, UT antenna placements and UT polarization direction are introduced in Rel-19 TR38.901, which probably be able to match well with the actual situation on the real network.

Proposal 2: RAN4 should clarify if the UT radiation power pattern, UT antenna placements and UT polarization direction defined in Rel-19 TR38.901 should be considered as the simulation assumption for FR1 SU-MIMO demodulation performance evaluation.

2.2.2 Scaling of angles

Scaling of angles procedure for CDL extension is reorganized as an outcome of RAN1 study item FS_NR_7_24GHz_Chmod as following, in which scaling factor is not applied for cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD. Consequently, the procedure for scaling of angles should be updated according latest TR38.901, in order to avoid mis-alignment between TR38.753 and TR38.901.

Scaling of angles in TR38.901

- 7.7.5.1 CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle μϕ,model can be changed to μϕ,desired and angular spread ASmodel can be changed to match ASdesired by scaling. The translated and scaled ray angles can be obtained according to the following equation: ϕn,m,scaled=ϕn,scaled+c{ASA, ASD,ZSA,ZSD}αm (7.7-5) ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desired (7.7-5a) ϕn,intermediate=s∙WrapTo180ϕn,model-μϕ,model (7.7-5b) ϕLOS,intermediate=s∙WrapTo180ϕLOS,model-μϕ,model (7.7-5c) ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired (7.7-5d) in which: - μϕ,model is the cluster mean angle calculated using Annex A.4 with ϕn,model and ϕLOS,model as the input cluster angles, - μϕ,intermediate is the intermediate cluster mean angle calculated using Annex A.4 with ϕn,intermediate and ϕLOS,intermediate as input cluster angles, - s is a scale factor chosen to change the distribution of the angles based on calculation in Annex A.5. Table 7.7.5.1-1 shows required scale factor for typical desired angular spread values for AOD, AOD, ZOA, and ZOD, - ϕn,model is the tabulated CDL cluster angle, - ASmodel is the rms cluster angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A.3, - μϕ,desired is the desired cluster mean angle, - ASdesired is the desired rms cluster angular spread, - ϕn,scaled is the scaled CDL cluster angle of the nth cluster, - ϕn,m,scaled is the resulting scaled ray angle that corresponds to mth subpath angle of nth cluster, - cASA, ASD,ZSA,ZSD is the cluster-wise RMS angular spread of either custer ASA, ASD, ZSA, or ZSD from Table 7.7.1-1, 7.7.1-2, 7.7.1-3, 7.7.1-4, and 7.7.1-5, - αm is the offset angles from Table 7.5-3, - WrapTo180⋅ is a function which wraps an azimuth angle to the half-open interval (-180, 180].

Additionally, typical angular spreads for CDL-C scenario obtained from the system-level model in TR38.901 are captured as Table 2-1, and the scaling factors used for rCDL-C1 in TR38.753 is listed in Table 2-2. From the comparsion of Table 2-1 and Table 2-2, for rCDL-C1, the scaling factors for Azimuth angles are larger than the values obtained from the system-level model in TR38.901.

Table 2-1. Scaling factor values for CDL-C in TR38.901

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-C | 5 | 0.1281 | 30 | 0.4307 | 5 | 0.6476 | 1 | 0.3643 |
|  | 10 | 0.2568 | 45 | 0.6447 | 10 | 1.2971 | 3 | 1.0929 |
|  | 15 | 0.3864 | 60 | 0.8585 | 15 | 1.9504 | 5 | 1.8219 |
|  | 25 | 0.6513 |  |  |  |  |  |  |

Table 2-2. Scaling factor values used for rCDL-C1 in TR38.753

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-C | 25.7620 | 0.6888 | 74.1138 | 1.0372 | 18.2050 | 1.7474 | 4.8978 | 1.2044 |

Observation 5: The angle scaling procedure for CDL extension is reorganized as an outcome of RAN1 study item FS_NR_7_24GHz_Chmod, in which scaling factor is not applied for cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD.

Observation 6: For rCDL-C1 in TR38.753, the scaling factors for Azimuth angles are larger than the values obtained from the system-level model in TR38.901.

Proposal 3: RAN4 should use the scaling of angles defined in latest TR38.901 for FR1 SU-MIMO demodulation performance evaluation.

2.2.3 Frequency Generalization

For rCDL-C1 study in TR38.753, only FR1 3.5GHz carrier frequency is assumed for the performance evaluation. For different carrier frequencies, the impacts could be summarized as:

- Delay spread and angular spreads

- Cluster powers

- Cluster delays and angles since DS and/or AS are frequency-dependent

- The space between antenna elements, e.g., currently 0.5 lambda assumed for rCDL-C1 study

- The wavelength which has impacts on the Tx steering vector expj2πrrx,n,mT⋅drx,uλ0, Rx steering vector expj2πrtx,n,mT⋅dtx,sλ0, and UE moving doppler expj2πrrx,n,mT⋅vλ0t

For the frequency generalization, for above items 1)-3), on one hand, these parameters are indeed affected by different carrier frequency, which in turn may influence the TDL-C channel profiles captured in TR38.901 Table 7.7.1-3. On the other hand, TR38.901 defines only one TDL-C table and does not specify which frequency band or carrier frequency it is based on. Therefore, RAN4 first needs to clarify whether the above items 1)-3) should be considered. If considered, the alternative channel generation method defined in TR38.901 chapter 7.6.5.1 could be considered as one possible solution for frequency generalization. If not considered, one uniform table for AoA/DoA/ZoA/ZoD angles, AS desired, AS model for different carrier frequencies is enough. For above items 4)-5), the impact of different carrier frequency could be addressed when calculating steering vectors and doppler.

Observation 7: For different carrier frequencies, the impacts on CDL channel model could be summarized as:

- Delay spread and angular spreads

- Cluster powers

- Cluster delays and angles since DS and/or AS are frequency-dependent

- The space between antenna elements, e.g., currently 0.5 lambda assumed for rCDL-C1 study

- The wavelength which has impacts on the Tx steering vector expj2πrrx,n,mT⋅drx,uλ0, Rx steering vector expj2πrtx,n,mT⋅dtx,sλ0, and UE moving doppler expj2πrrx,n,mT⋅vλ0t

Proposal 4: RAN4 first needs to clarify whether the above items 1)-3) should be considered for frequency generalization. If considered, the alternative channel generation method defined in TR38.901 chapter 7.6.5.1 could be considered as one possible solution.

2.2.4 PMI bias and performance alignment

Currently, rCDL-C1 defined in TR38.901 has fixed angular properties with non-uniform PMI statistics, which result in the PMI bias problem. Similar issue could also be observed for PMI reporting performance evaluation with legacy TDL channel model but without beam steering approach. Beam steering approaches defined in TS38.101-4 B.2.3.2.3 and B.2.3.2.3A are the solutions for legacy TDL channel model with fixed angular properties. Hence, beam steering approach could be a possible solution for rCDL-C1 with PMI bias issue.

Observation 8: Beam steering approaches defined in TS38.101-4 B.2.3.2.3 and B.2.3.2.3A are the solutions for legacy TDL channel model with fixed angular properties.

Proposal 5: Take beam steering approach defined in TS38.101-4 B.2.3.2.3 or B.2.3.2.3A as a possible solution for rCDL-C1 with PMI bias issue.

- Conclusion

In this contribution, we provide analysis and views on FR1 SU-MIMO SCM topic.

The observations and proposals could be summarized as:

Observation 1: For rCDL-C1 in TR38.753, omni antenna radiation pattern and co-located cross-polarization antenna modelling are assumed for UE side.

Observation 2: For rCDL-C1 in TR38.753, the scaling factor is not only applied for cluster angles, but also applied for the tabulated cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD in Table 7.7.1-3 of 38.901.

Observation 3: For rCDL-C1 in TR38.753, only FR1 3.5GHz carrier frequency is assumed for the performance evaluation.

Proposal 1: For rCDL-C1 in TR38.753, before the second-round simulation campaign for PMI reporting performance evaluation with eType-II codebook and SCM channel model, the PMI bias problem should be studied and solved firstly.

Observation 4: UT radiation power pattern, UT antenna placements and UT polarization direction are introduced in Rel-19 TR38.901, which probably be able to match well with the actual situation on the real network.

Proposal 2: RAN4 should clarify if the UT radiation power pattern, UT antenna placements and UT polarization direction defined in Rel-19 TR38.901 should be considered as the simulation assumption for FR1 SU-MIMO demodulation performance evaluation.

Observation 5: The angle scaling procedure for CDL extension is reorganized as an outcome of RAN1 study item FS_NR_7_24GHz_Chmod, in which scaling factor is not applied for cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD.

Observation 6: For rCDL-C1 in TR38.753, the scaling factors for Azimuth angles are larger than the values obtained from the system-level model in TR38.901.

Proposal 3: RAN4 should use the scaling of angles defined in latest TR38.901 for FR1 SU-MIMO demodulation performance evaluation.

Observation 7: For different carrier frequencies, the impacts on CDL channel model could be summarized as:

- Delay spread and angular spreads

- Cluster powers

- Cluster delays and angles since DS and/or AS are frequency-dependent

- The space between antenna elements, e.g., currently 0.5 lambda assumed for rCDL-C1 study

- The wavelength which has impacts on the Tx steering vector expj2πrrx,n,mT⋅drx,uλ0, Rx steering vector expj2πrtx,n,mT⋅dtx,sλ0, and UE moving doppler expj2πrrx,n,mT⋅vλ0t

Proposal 4: RAN4 first needs to clarify whether the above items 1)-3) should be considered for frequency generalization. If considered, the alternative channel generation method defined in TR38.901 chapter 7.6.5.1 could be considered as one possible solution.

Observation 8: Beam steering approaches defined in TS38.101-4 B.2.3.2.3 and B.2.3.2.3A are the solutions for legacy TDL channel model with fixed angular properties.

Proposal 5: Take beam steering approach defined in TS38.101-4 B.2.3.2.3 or B.2.3.2.3A as a possible solution for rCDL-C1 with PMI bias issue.

- Reference

[1] RP-252957, WID: NR demodulation performance: Phase 6, RAN#109 meeting, Apple

[2] R4-2509114, 3GPP TR38.753 V1.1.0