3GPP TSG-RAN WG4 Meeting #116bis	R4-2513887

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Perf part of SSB adaptation for R19 NES

Agenda Item:	6.15.2.2

Document for:	Discussion

- Introduction

Until last RAN4 meeting, both OD-SSB and SSB adaptation have already achieved good progress and the core part of R19 NES was completed. Based on the scheduling, the discussion on the performance part would still continue in this meeting. The discussion on the test cases was developed and some initial progress were achieved in [1]. In this paper, we provide our views on the performance part of SSB adaptation.

- Discussion

- Based on the core part discussion during previous meetings, It is confirmed that there are RRM impact by SSB adaptation for the following cases around SCell:

- -  L1-RSRP

- -  L1-SINR

- -  CBD

- -  TCI state switching

- -  NR intra/inter-frequency L3 measurement

- Since RAN2 does not achieve further progress to revert the existing specification around which operation is not intended for the deactivated SCell as below:

TS 38.321 1> if the SCell is deactivated: - 2> not transmit SRS on the SCell; - 2> not report CSI for the SCell; - 2> not transmit on UL-SCH on the SCell; - 2> not transmit on RACH on the SCell; - 2> not monitor the PDCCH on the SCell; - 2> not monitor the PDCCH for the SCell; - 2> not transmit PUCCH on the SCell.

- So no need to consider any test case for the deactivated SCell measurement and the SCell activation procedure.

- Around the L1 measurement, the requirements at transition was defined and the additional UE processing delay was introduced in core part, since it is specific for SSB adaptation, it should be verified. Since the additional processing delay for FR1 and FR2 are different, so both FR1 and FR2 should be verified.

- Around the L3 measurement, the requirements at transition was also defined and should be tested. Furthermore, the corresponding SMTC adaptation is accompanied with SSB adaptation. So regarding the test scope, we have the following proposal.

- Proposal 1: The following cases should be verified around the SSB adaptation:

- The L1-RSRP/L1-SINR measurement on the SCell after SSB adaption in FR1 and FR2;

- The CBD measurement/evaluation on the SCell after SSB adaption in FR1 and FR2;

- L3 measurement with the adapted SMTC corresponding to the SSB adaptation.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements around the performance part of NES:

- Proposal 1: The following cases should be verified around the SSB adaptation:

- The L1-RSRP/L1-SINR measurement on the SCell after SSB adaption in FR1 and FR2;

- The CBD measurement/evaluation on the SCell after SSB adaption in FR1 and FR2;

- L3 measurement with the adapted SMTC corresponding to the SSB adaptation.

- Reference

- R4-2512327 WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#116.