3GPP TSG-RAN WG4 Meeting #116bis	R4-2513888

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Perf part of R19 SBFD operation

Agenda Item:	6.13.3.2

Document for:	Discussion

- Introduction

After several meetings’ discussion, good progress have been achieved for core part and it was completed in last meeting. Based on the work plan for Perf part approved in [1], the performance part for the R19 Duplex is to be discussed further in this meeting. Based on the progress on core part for the RRM requirements[2][3][4][5][6][7], we provide our views on performance part in this paper.

- Discussion

- Around the SBFD, two sub-topics including UE-to-UE CLI and SBFD operation impact were discussed in core part and RRM requirements were defined for each sub-topic. For the performance part, we discuss SBFD operation impact in this paper.

- The SBFD operation impact on L1 CSI-RS based measurement and L3 CSI-RS based measurement were discussed in core part.

- Around the L1 CSI-RS based measurement, the priority order between dynamic scheduled UL transmission and the P/SP CSI-RS was determined. Accordingly, the measurement period for P/SP CSI-RS based L1-RSRP/L1-SINR/RLM/BFD/CBD is allowed to extend following the NR-U approach when collision happens. Such collision handling and the exact measurement period with extension should be verified.

- Proposal 1: For L1 CSI-RS based measurement, collision handling between the L1 CSI-RS and dynamic scheduled UL transmission and the exact measurement period with extension should be verified.

- For the DUD case, the following three cases are touched corresponding to different BW of the CSI-RS resource cases during core part discussion:

- Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

- No requirements impact for Case 1 and 2. For Case 3, the accuracy requirements are relaxed provided some certain conditions meet. So the relaxed accuracy requirements under Case 3 should be verified.

- Proposal 2: For DUD case, the relaxed accuracy requirements should be verified provided some certain conditions meet.

- For the L3 CSI-RS based measurement, wait for the progress of core part discussion and decide which test case should be defined.

- Proposal 3: For L3 CSI-RS based measurement, wait for the progress of core part discussion and decide which test case should be defined.

- The Conclusion

In this contribution, we have the following proposals on the performance part of SBFD:

- Proposal 1: For L1 CSI-RS based measurement, collision handling between the L1 CSI-RS and dynamic scheduled UL transmission and the exact measurement period with extension should be verified.

- Proposal 2: For DUD case, the relaxed accuracy requirements should be verified provided some certain conditions meet.

- Proposal 3: For L3 CSI-RS based measurement, wait for the progress of core part discussion and decide which test case should be defined.

- Reference

- R4-2507487, “Workplan for Evolution of NR Duplex Operation (RAN4 Performance Part)”, Samsung, Huawei, 3GPP RAN4#115, 2024.

- R4-2413900, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#112, 2025.

- R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#112bis, 2025.

- R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#113, 2025.

- R4-2502597, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#114, 2025.

- R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#115, 2025.

- R4-2512133, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#116, 2025.