3GPP TSG-RAN WG4 Meeting#116bis                                  R4-2513898

Prague, CZ, 13rd – 17th, Oct. 2025

Agenda item:	6.17.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on performance requirements for event triggered L1 measurement reporting

Document for:	Approval

Introduction

The RRM core requirement on Mobility work item has been summarized, which include

- Event triggered L1 measurement reporting

- CSI-RS measurements for LTM procedures

- (subsequent) conditional LTM

This contribution provides our view on performance requirements for R19 mobility enhancement.

Discussion

For Event triggered L1 measurement reporting, we can use the test case for L3 event triggered reporting as baseline and further discuss whether it is needed or not for L1 event triggered reporting. In current spec, there are many test cases for intra-f in clause A6.6 and inter-f in clause A6.7. We have summaried in follow table.

| A.6.6.1.1 | SA event triggered reporting tests without gap under non-DRX |
| --- | --- |
| A.6.6.1.2 | SA event triggered reporting tests without gap under DRX |
| A.6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX |
| A.6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX |
| A.6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |
|  | ... |
| A.6.6.2.1 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used |
| A.6.6.2.2 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used |
| A.6.6.2.5 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |
| A.6.6.2.6 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used |
| A.6.6.2.10 | SA event triggered reporting tests for FR1 when DRX is used |
| A.6.6.2.11 | SA event triggered reporting tests for FR1 without gap when DRX is not used |
|  | ... |

In our inderstanding, for SSB based L1-RSRP measurement, we considered three scenarios that intra-f measurement without gap, inter-f measurement with gap and inter-f measuremnt without gap. Meanwhile, with or without SSB index reading also considered. Therefore, we should at least consider follow aspects when define test cases:

- With gap &without gap

- Intra-f & Inter-f

- FR1 & FR2

In the last meeting, there are two approaches to verify reporting behavuor. One is to duplicate all R18 test cases and add event triggered reporting. In this way, UE which can pass R19 test cases can skip R18 test cases. And the other is to keep R18 test cases and develop necessary new R19 test cases. From our point, both methods are workable.

- Further discuss based on the following directions - For SSB based L1 RSRP measurement reporting: - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple) - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW)

If we use approach 2, following test cases should be considered for SSB based event-triggered L1 reporting to verfied the UE behavior according to the RRM requirements that have defined in core part for event triggered L1 measurement report:

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

There are 6 test cases need to be introduced.

If we use approach 1, according to the test list in R18, there are 7 test cases need to be introduced. Compared to develop necessary new R19 test to verify reporting behavior, the first approach is more specific and complete.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

If approach 1 is adopted, all UEs that have passed the R19 tests can definitely pass the R18 tests, as the R18 test is part of the R19 testing procedure. Under this design, the number of tests required by the UE can be effectively reduced.

In addition, we have reached consensus on the test for CSI-RS based L1 RSRP measurement reporting that CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting can be tested in the same test case if UE support both. If approach 1 is adopted for SSB, then the logic is aligned with CSI-RS.

Proposal 1: For SSB based L1 RSRP measurement reporting, prefer to duplicate all R18 test cases and add event triggered reporting in R19, and UE which can pass R19 test cases can skip R18 test cases.

Further discuss based on the following directions - For CSI-RS based L1 RSRP measurement reporting: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1)

For CSI-RS based L1 RSRP measurement reporting, it is agreed to introduce test cases to verify CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting. Following the same logic as approach 1 of SSB based L1 RSRP measurement reporting, adding event triggered reporting to all test cases introduced for CSI-RS based L1-RSRP measurement in R19.

Observation 1: For CSI-RS based L1 RSRP measurement reporting, reporting behavior can be verified by adding event triggered reporting to all test cases that introduced for CSI-RS based L1-RSRP measurement in R19.

- Proposal 2: For CSI-RS based L1 RSRP measurement reporting, test cases are introduced to jointly verify CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting.

Conclusions

In this contribution, we put forward the following proposal on performance part for mobility enhancements.

Proposal 1: For SSB based L1 RSRP measurement reporting, prefer to duplicate all R18 test cases and add event triggered reporting in R19, and UE which can pass R19 test cases can skip R18 test cases.

Observation 1: For CSI-RS based L1 RSRP measurement reporting, reporting behavior can be verified by adding event triggered reporting to all test cases that introduced for CSI-RS based L1-RSRP measurement in R19.

Proposal 2: For CSI-RS based L1 RSRP measurement reporting, test cases are introduced to jointly verify CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple