3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2513903

Prague, Czech Republic, Oct. 13-17, 2025

Source: 	MediaTek Inc.

Title: 	Initial views on NTN HD-FDD VSAT RF Requirements for Ku-band

Agenda Item:	7.6.4.1

Document for:	Approval

- Introduction

In the RAN#109, the new WI on NTN HD-FDD VSAT for the Ku/Ka bands [1] was approved, and its objectives are shown below. This paper provides our initial views as this WI begins.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

- Discussion

  - Requirements for HD-FDD VSAT types 2, 3, 5 and 6

Regarding” Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna”.

- (b)

Figure 1. An example of VSAT block diagram (a) FDD VSAT (b) HD-FDD VSAT

First, we can discuss the implementation differences between HD-FDD VSAT and FDD VSAT. The implementation of HD-FDD VSAT differs from that of FDD VSAT. Typically, FDD uses two separate antenna arrays connected to the Tx and Rx chains, respectively, while HD-FDD may use a single aperture with a switch that alternates between Tx and Rx , as shown in Figure 1. For HD-FDD VSAT, the switch after power amplifier can introduce additional insertion loss that should be accounted for in the link-budget calculation. Additionally, for FDD VSAT, interference between Tx and Rx may sometimes require additional suppression components, which can introduce extra insertion loss. Therefore, in our view, the existing RF requirements for FDD VSAT types 2, 3, 5, and 6 can be reused for HD-FDD operation.

Observation 1: The implementation of HD-FDD VSAT differs from that of FDD VSAT. Typically, FDD uses two separate antenna arrays connected to the Tx and Rx chains, respectively, while HD-FDD may use a single aperture with a switch that alternates between Tx and Rx , as shown in Figure 1. For HD-FDD VSAT, the switch after power amplifier can introduce additional insertion loss that should be accounted for in the link-budget calculation. Additionally, for FDD VSAT, interference between Tx and Rx may sometimes require additional suppression components, which can introduce extra insertion loss.

Proposal 1 : RAN4 to consider reusing the existing FDD VSAT RF requirements for VSAT types 2, 3, 5, and 6 for HD-FDD operation.

  - New VSAT type: VSAT with small antenna

In the R19 Ku-band WI, we discussed two mobile electrically beam-steered VSAT types: VSAT Type 5, based on a 78×78-element antenna array, and VSAT Type 6, based on a 45×45-element antenna array. We also specified VSAT RF requirements based on these phased array configurations. In the Rel-20 HD-FDD VSAT WI, with respect to the objective “Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20×20 cm) with a new VSAT type,” a new HD-FDD VSAT type with a small antenna (e.g., 20×20 cm) needs to be defined first, as its antenna size differs from the VSAT types defined in 38.101-5. Therefore, RAN4 could follow the WI objective and define a new HD-FDD VSAT type. VSAT Type 7: a small-antenna (e.g., 20 × 20 cm) VSAT with electronic beam steering for LEO.

Proposal 2 :Following WI objective, RAN4 to define a new HD-FDD VSAT type. VSAT type 7: VSAT with small antenna (e.g., 20 × 20 cm) with electrical beam steering for LEO.

In the WI, the size of the small antenna is still under discussion. For the time being, we assume 20 × 20 cm. We propose determining the antenna size for VSAT Type 7 based on the link-budget evaluation.

Proposal 3: RAN4 to determine the antenna size for VSAT Type 7 based on the link-budget evaluation.

In the Rel-19 Ku-band WI, the ACLR and ACS requirements for VSAT Types 4, 5, and 6 are based on coexistence simulations, the Ka-band requirements, an intra-SAN simulation provided by Eutelsat, and practical implementation considerations. RAN4 spent considerable time discussing the coexistence-simulation assumptions and scenarios. However, during the evaluation, we found that the assumptions regarding TN/NTN frequency allocation do not reflect practical deployments, and some of the resulting simulation outcomes are not feasible. Therefore, we recommend not performing a Ku-band TN/NTN coexistence evaluation for the new VSAT type and reusing the existing Ku-band ACLR and ACS requirements.

Observation 2: In the Rel-19 Ku-band WI, the ACLR and ACS requirements for VSAT Types 4, 5, and 6 are based on coexistence simulations, the Ka-band requirements, an intra-SAN simulation provided by Eutelsat, and practical implementation considerations.

Proposal 4: RAN4 to consider not performing a Ku-band TN/NTN coexistence evaluation for the new VSAT type and reusing the existing Ku-band ACLR and ACS requirements.

  - VSAT RF requirements to support HD-FDD operation.

Table 1 reviews each VSAT Tx and Rx RF requirement individually to determine which require modification with the introduction of HD-FDD operation. It also presents our initial views on those modifications.

Table 1. VSAT RF requirements to support HD-FDD operation.

| VSAT Tx/Rx requirement | HD-FDD operation |
| --- | --- |
| 9.2 Transmitter power |  |
| 9.2.1 NTN VSAT maximum output power | 1. Introduce a new HD-FDD VSAT type with small antenna 2. EIRP requirements for the new VSAT type need to be specified |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | No impact expected. |
| 9.2.3 Configured transmitted power | No impact expected. |
| 9.3 Output power dynamics |  |
| 9.3.1 Minimum output power | No impact expected. |
| 9.3.2 Transmit OFF power | No impact expected. |
| 9.3.3 Transmit ON/OFF time mask | No impact expected. |
| 9.3.4 Power control | No impact expected. |
| 9.4 Transmit signal quality |  |
| 9.4.1 Frequency error | No impact expected. |
| 9.4.2 Transmit modulation quality | No impact expected. |
| 9.5 Output RF spectrum emissions |  |
| 9.5.1 Occupied bandwidth | No impact expected. |
| 9.5.2.2 Spectrum emission mask | No impact expected. |
| 9.5.2.3 ACLR | ACLR requirement for the new VSAT type needs to be specified |
| 9.5.3 Spurious Emissions | Update On-axis spurious limits in “Carrier-on” state to include the new VSAT type. |
| 9.6 Antenna pointing accuracy and performance |  |
| 9.6.1 Antenna pointing accuracy | No impact expected. |
| 9.6.2 Antenna performance | No impact expected. |
| 9.7 Additional regional requirements indicated by NS |  |
| 9.7.1 General | No impact expected. |
| 10 Radiated receiver characteristics |  |
| 10.3 OTA reference sensitivity level | EIS requirement for the new VSAT type needs to be specified |
| 10.4 Maximum input level | Maxiumun input level requirement for the new VSAT type needs to be specified |
| 10.5 Adjacent channel selectivity | ACS requirements for the new VSAT type needs to be specified |
| 10.6 Blocking characteristics | No impact expected. |
| 10.8 Receiver antenna off-axis performance | No impact expected. |

Proposal 5 : Table 1 is used for VSAT RF requirements to support HD-FDD operation.

- 3	Conclusion

In this contribution, we provide our views on the Rel-20 NTN HD-FDD VSAT for Ku band. The proposals and observations are summarized as follows:

Observation 1: The implementation of HD-FDD VSAT differs from that of FDD VSAT. Typically, FDD uses two separate antenna arrays connected to the Tx and Rx chains, respectively, while HD-FDD may use a single aperture with a switch that alternates between Tx and Rx. For HD-FDD VSAT, the switch after power amplifier can introduce additional insertion loss that should be accounted for in the link-budget calculation. Additionally, for FDD VSAT, interference between Tx and Rx may sometimes require additional suppression components, which can introduce extra insertion loss.

Proposal 1 : RAN4 to consider reusing the existing FDD VSAT RF requirements for VSAT types 2, 3, 5, and 6 for HD-FDD operation.

Proposal 2 :Following WI objective, RAN4 to define a new HD-FDD VSAT type. VSAT type 7: VSAT with small antenna (e.g., 20 × 20 cm) with electrical beam steering for LEO.

Proposal 3: RAN4 to determine the antenna size for VSAT Type 7 based on the link-budget evaluation.

Observation 2: In the Rel-19 Ku-band WI, the ACLR and ACS requirements for VSAT Types 4, 5, and 6 are based on coexistence simulations, the Ka-band requirements, an intra-SAN simulation provided by Eutelsat, and practical implementation considerations.

Proposal 4: RAN4 to consider not performing a Ku-band TN/NTN coexistence evaluation for the new VSAT type and reusing the existing Ku-band ACLR and ACS requirements.

Proposal 5 : Table 1 is used for VSAT RF requirements to support HD-FDD operation.

- 4	References

- RP-252958, “Enhanced requirements for NR NTN and IoT NTN Phase 2”, Qualcomm, RAN#109.