3GPP TSG RAN WG4 Meeting #116-bis	R4-2513907

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			4.25.3

Source:	vivo

Title:	Discussion on core requirements of R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 almost finalized the RRM requirements on the NR NTN Phase 3. The latest progress was summarized in WF [1]. In this contribution, we would like to provide analysis on remain issues on DL coverage.

- 2.	Discussion

## [Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6) RAN4 to further discuss the following in RAN4#116-bis: - For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant: - The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE. - Note: The SMTC selection rule may be further discussed and specified or left to UE implementation.

In the scenario where the NW configures more than 4 STMCs, RAN2 has newly reached the following conclusions for idle/inactive and connected states, respectively. For the connected mode, RAN2 provided a detailed procedure on how to select SMTC based on location. However, this procedure does not cover the idle/inactive mode. For idle/inactive mode, location can serve as reference information to assist the UE in selecting SMTC, but there is no standardized procedure and there will be no corresponding specification impact. The selection of SMTC remains dependent on UE implementation. The existing specification statement, "The selection of SMTCs to be used is up to UE implementation," still applies.

RAN2 agreements in RAN2#131 2. RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements 4. UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located 5. The procedure for UE assisted SMTC configuration in connected mode is follows: - For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused. - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations. - The UE can report the N closest reference locations via the RRCReconfigurationComplete message. - The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy). - The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

Proposal 1: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation and no update is needed for current RRC Idle/Inactive mode RRM requirements.

## [Online # 3] Issue 0-B: Two different SMTC periodicities (with different offsets) in one frequency layer Agreement (online): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - No requirements if the SMTC periodicity is smaller than the actual SSB periodicity of the cells associated with the SMTC - For Trs in Handover requirement: - Trs is the SMTC corresponding to the physical cell ID of the target cell - Further discuss the proposal “or the longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.” - For Trs in RRC Connection Release with Redirection requirement: - If physical cell ID is provided in RAN2 signalling design, Trs is the SMTC corresponding to the physical cell ID of the target cell - Otherwise, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. - Kp and Kgap in RRC Connected mode intra/inter-frequency measurement requirement - The SMTC period in the requirement is the one used by the cell being identified

Trs definition in Handover delay requirement - Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

For the scenario highlighted in the following existing handover Trs definition, where the SMTC configuration for the target cell is not provided, this issue was raised in contribution [2] during RAN4 #89. It mentioned that, according to RAN2's design, if the SMTC configuration for the target cell is not provided, the UE will use the SMTC configured in the measObjectNR with the same SSB frequency and subcarrier spacing. The final description in the RAN4 specification followed this exact statement. Therefore, this case must also be considered in the discussion here. Accordingly, the definition of the corresponding Trs in Handover requirement can be updated as follows:

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and intra-NR handover. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of intra-NR handover, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

As for the Trs definition for RRC Connection Release with Redirection requirement, it was discussed and modified around the same time. According to the analysis in contribution [3], the same solution to the unknown SMTC problem adopted in NR handover can be applied. The corresponding CR [4] was agreed in that meeting. Therefore, for the RRC Connection Release with Redirection requirement, the Trs definition in Handover delay requirement can be directly reused.

RedirectedCarrierInfo ::= CHOICE { nr CarrierInfoNR, eutra RedirectedCarrierInfo-EUTRA, ... } CarrierInfoNR ::= SEQUENCE { carrierFreq ARFCN-ValueNR, ssbSubcarrierSpacing SubcarrierSpacing, smtc SSB-MTC OPTIONAL, -- Need S ... } smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Proposal 2: When two different SMTC periodicities (with different offsets) in one frequency layer are configured,

- For Trs in Handover requirement:

  - Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

- For Trs in RRC Connection Release with Redirection requirement:

  - Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

Regarding RAN2’s reply LS [5], the main points are as follows:

- One key update compared to the previous conclusion is:

The maximum number of configured SMTCs for idle/inactive is 7, and this also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 SMTCs).

The maximum number of configured SMTCs for idle/inactive has been increased from 6 to 7. Since the NW configures more than 4 SMTCs for the UE in idle/inactive, the UE selects up to 4 SMTCs (or less, depending on UE capability), and the selection depends on UE implementation, this update will not impact RAN4.

- Regarding whether the support for SMTC enhancements is limited to FR1 or can also be extended to FR2:

We support restricting this enhancement to FR1. For FR2 scenarios, particularly in earlier discussions on RRM requirements for the Ka band, several scenarios were distinguished (e.g., intra-satellite and inter-satellite) along with different antenna scanning types (e.g., electronic steering and mechanical steering), which imposed certain constraints and assumptions. RAN4 has not previously discussed whether or how SMTC enhancements should be applied to FR2 requirements. If such a discussion were to take place, it would require a case-by-case review. Given that the core part of the work has already been concluded, we prefer not to extend the discussion or define requirements for FR2.

Specifically, for inter-satellite scenarios, measurements related to neighbour cells are not defined (e.g., for Handover: only blind HO is considered, with no neighbour cell measurement; there are no corresponding requirements for measurement procedures). Whether there is a practical use case requiring SMTC enhancements in such scenarios also needs clarification.

Proposal 3: In response to RAN2’s reply LS on supporting SMTC enhancements, RAN4 provides the following feedback:

- Regarding the increase in the maximum number of configured SMTCs for idle/inactive from 6 to 7, no update is needed for current RRC Idle/Inactive mode RRM requirements.

- The applicability of RAN4 requirements for 6 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN.

- Summary

Proposal 1: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation and no update is needed for current RRC Idle/Inactive mode RRM requirements.

Proposal 2: When two different SMTC periodicities (with different offsets) in one frequency layer are configured,

- For Trs in Handover requirement:

  - Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

- For Trs in RRC Connection Release with Redirection requirement:

  - Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

Proposal 3: In response to RAN2’s reply LS on supporting SMTC enhancements, RAN4 provides the following feedback:

- Regarding the increase in the maximum number of configured SMTCs for idle/inactive from 6 to 7, no update is needed for current RRC Idle/Inactive mode RRM requirements.

- The applicability of RAN4 requirements for 6 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN.

- References

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated

- R4-1815152, Discussion on Trs definition, Huawei, HiSilicon

- R4-1901120, Discussion on the redirection to NR requirements, Huawei, HiSilicon

- R4-1902372, draftCR on the release with redirection to NR (section 6.2.3), Huawei, HiSilicon, MediaTek, Ericsson

- R2-2506298, LS on additional agreements on SMTC enhancements, xiaomi