3GPP TSG RAN WG4 Meeting #116-bis	R4-2513914

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			7.12.3

Source:	vivo

Title:	Genernal discussion on RRM impact on RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN#109 meeting, the latest WID [1] was approved. The description of objectives is copied as:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

The RAN4 task specifies the highlighted sections above, primarily covering both RRM measurement prediction and measurement event prediction for the UE-sided model.

In this contribution, we initially provide our views on the RRM impacts for RRM measurement prediction based on the WID above and outcome reached in study item.

- Discussion

  - Scenarios

During the SI phase, RAN2 discussed the evaluation scenarios and their associated priorities, which are captured in TR38.744, as shown in the table below. Previous RAN2 studies and discussions have mainly focused on high-priority scenarios, which are Scenario 2, 3, and 4.

Table 5.2.1.1-1: Prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | intra-cell |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Additionally, according to the justification described in the following WID, the SLS simulation evaluations conducted by RAN2 have also primarily focused on these three scenarios.

Extensive simulations for RRM measurement prediction, measurement event prediction including corresponding SLS are conducted on high priority scenarios i.e. FR2 intra-frequency temporal domain case A (i.e., prediction of measurements in a prediction window by using historical actual measurements in an observation window), FR1 intra-frequency temporal domain case B (similar to case A, but predicted measurement result(s) in previous prediction window are skipped), and FR1 inter-frequency prediction.

However, the description of WID objectives does not specifically mention scenarios. Instead, it is described as:

Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

If interpreted this way, scenarios 1–5 are all encompassed within this description. Therefore, at least from the RAN4 perspective, clarification and alignment are needed regarding the scenarios to be defined by RAN4. Based on our understanding, in the previous SI discussions, we did not aim to study the related RRM impacts for low-priority scenarios (Scenario 1 and 5). Moreover, for instance, in the case of FR2 to FR2 intra-frequency temporal domain case B, if partial measurement is skipped, the prediction performance may degrade compared to FR1. Furthermore, if all five scenarios are considered, the workload for RAN4 would be substantial from the perspective of evaluation and requirement definition. Therefore, we believe that the target scenarios for the WI should only focus on the high-priority scenarios identified in the SI, and evaluation and requirement definition should be conducted accordingly.

Proposal 1: For RAN4 performance evaluation and requirement definition, RAN4 should focus on the following high-priority scenarios:

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

  - Factors to be considered(for core part)

- RRM sub-cases

During the discussions on the RAN2 study item, the following conclusion was reached at the RAN2#130 meeting. Regarding the RRM sub-use cases, RAN2 agreed that it would depend on the UE implementation.

Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Observation 1: Based on RAN2's conclusion, the implementation of RRM sub-use cases can be left to UE implementation.

On the other hand, the WID for UE-side models does not specifically mention RRM sub-use cases. The main reason is that different companies use different types of models, and the resulting performance also varies. Therefore, as long as the predictive performance is ensured, the choice of RRM sub-use cases can be left to UE implementation.

Additionally, in previous RAN2 evaluations, at least for the FR2 intra-frequency temporal domain Case A, the prediction performance of RRM sub-use case 2 was better than the other two sub-use cases. Moreover, for sub-use case 2, L3 predictions are based on actual L3 results, while the L1-related processes remain consistent with legacy methods, making it simpler. Therefore, a possible approach is to use the performance of sub-use case 2 as the basis for defining performance metrics and requirements during evaluation and requirement definition. In the specification, the requirement definitions will not distinguish or reflect the three RRM sub-use cases.

- 5.2.2.1.4 Summary of performance results for RRM measurement prediction For FR2 intra-frequency temporal domain case A the following observations are made: - Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit; - A majority of the companies observe that RRM sub-use case 2 demonstrates higher prediction accuracy than RRM sub-use case 1 and RRM sub-use case 3 at least with short PW length;

From RAN4 perspective, for the following three RRM sub-use cases, another approach is to consider all of them during the RAN4 performance evaluation phase. If the evaluation shows significant performance differences among these three cases, it may be necessary to define three separate sets of metrics for each case accordingly.

-	RRM sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated;

-	RRM sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s);

-	RRM sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

Therefore, in the initial discussions, RAN4 first needs to decide how to address/handle the RRM sub-use cases when evaluating performance and defining requirements.

Proposal 2: RAN4 first to decide how to address/handle the RRM sub-use cases when evaluating performance and defining requirements.

- Option 1: During the performance evaluation phase, all three RRM sub-use cases are considered. Whether the requirement definition distinguishes between these three cases depends on the performance evaluation results. If there are significant performance differences among the three cases, three separate sets of metrics are defined for each case.

- Option 2: During performance evaluation and requirement definition, the performance of sub-use case 2 is used as the basis for defining the metrics. In the specification, the requirement definition does not distinguish or reflect the three RRM sub-use cases.

- OW window and Sliding/non-sliding window

In the SI discussions, RAN4 reached the following conclusions regarding the observation window:

For intra-frequency temporal domain prediction, the impact on prediction accuracy by UE speed, MRRT, and OW/PW length will be considered. The number of measurements performed in OW could be also considered.

According to the aforementioned RAN2 agreement, the OW length also depends on UE implementation. However, from RAN4 perspective, different OW lengths may lead to different prediction performances. Therefore, RAN4 needs to at least consider different OW lengths in the evaluation to determine the minimum OW length that satisfies the defined prediction accuracy.

Additionally, regarding the core part, based on the outcomes discussed in AI/ML beam management, we defined the prediction delay in the core part. This delay is intended to define the time gap from when the UE performs measurements for inference to when it reports the prediction results. The relevant agreement is as follows. In this context, the OW length and the number of measurements performed within the OW are considered for the part where the UE performs measurements for inference. Furthermore, whether a sliding or non-sliding window is used for this part will also affect the length of the inference delay. Therefore, for AI-based mobility (AI mob), a similar prediction delay may be defined, which is related to the observation window and the use of a sliding or non-sliding window.

Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+ time for the first available reporting occasion Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement

Proposal 3: During the performance evaluation phase and the requirements definition phase (e.g., potential prediction delay), RAN4 needs to consider both the OW window and the sliding/non-sliding window factors.

- case A/B

Specifically, for Case B, the main difference compared to Case A is that certain actual measurements will be skipped and replaced with predicted results. Regarding the specific skipping pattern, RAN2 reached relevant conclusions during the RAN2#131 meeting. As shown below, it was agreed that for the skipping pattern in the temporal domain of Case B, the interpretation is from NW perspective, referring to the SSB configuration used by the network to indicate the timing of its own SSB transmission, rather than from the UE perspective regarding how the UE actually skips measurements. This SSB configuration potentially impacts the MRRS parameter, which will effects prediction performance. Further details are expected to depend on additional conclusions from RAN2.

RAN2#131 meeting Agreements

- For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

Proposal 4: For discussion on Case B, it depends on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission.

- inter-frequency prediction

In addition to time domain prediction, additional considerations for frequency domain prediction are required: Since inter-frequency cells are obtained through prediction based on the current cell, this particular characteristic may impact requirements such as cell known condition and CSSF. RAN4 needs to take these into account in subsequent discussions.

Proposal 5: For inter-frequency cell prediction, RAN4 should identify the potential RRM impacts resulting from the fact that inter-frequency cells are obtained through prediction and may not have been measured, such as cell known condition and CSSF

- LCM related

Regarding LCM, depending on further procedure and signaling design from RAN2, RAN4 can further discuss whether related LCM requirements are necessary based on further conclusions from RAN2.

Proposal 6: RAN4 further discusses whether related LCM requirements are needed, depending on RAN2's design.

  - KPIs

For KPIs related to prediction performance, RAN4 reached a consensus on the definition of RSRP accuracy during the SI phase, and initially identified factors that may affect prediction accuracy.

The absolute accuracy of predicted L3-RSRP is defined as: - Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP. The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported L3-RSRP of cell 2 can be measured or predicted. - Editor Note: The relative accuracy of predicted L3-RSRP for beam level measurements may be further discussed during WI phase depending upon RAN2 progress. The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported L3-RSRP of cell 2 can be measured or predicted. NOTE: It is not precluded to update the definition based on further RAN2 progress in WI phase. As a baseline, the ground truth of L3-RSRP in FR2 is defined as the reported L3-RSRP measurement result under sufficient high SNR.

For this part, RAN4 needs to clarify the factors to be considered in simulation evaluations for each scenario. Below, we summarize the factors that may impact performance in three high-priority scenarios, including performance results observed in the current SI-phase RAN2 evaluations. RAN4 can further discuss and determine the factors to be considered in simulation evaluations based on the following table.

Proposal 7: For performance evaluation, RAN4 to identify the factors to be considered for each scenario:

| Scenario 1: FR2 intra-frequency temporal domain case A |  |  |
| --- | --- | --- |
| Factor | Impact on performance (in RAN2 evaluation) | Whether to consider in the RAN4 evaluation |
| Prediction window length | Yes, Longer PW length correlates with decreased prediction accuracy | Yes |
| UE speed | Yes, Higher UE speed correlates with decreased prediction accuracy | Yes |
| Cluster-based approach& single cell approach | Not much impact, The gain of cluster approach against single cell approach is not clear | FFS |
| Observation window length | Yes, Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit | Yes |
| Sub-use cases | Yes, A majority of the companies observe that RRM sub-use case 2 demonstrates higher prediction accuracy than RRM sub-use case 1 and RRM sub-use case 3 at least with short PW length | FFS Depending on the RAN4 discussion, whether all three cases will be considered in the performance evaluation. |
| Scenario 2: FR1 intra-frequency temporal domain case B |  |  |
| Factor | Impact on performance (in RAN2 evaluation) | Whether to consider in the RAN4 evaluation |
| Prediction window length | Same observation as scenario 1 | Yes |
| UE speed | Same observation as scenario 1 | Yes |
| Cluster-based approach& single cell approach | Same observation as scenario 1 | FFS |
| MRRT | Yes, Increasing MRRT correlates with decreased prediction accuracy | Yes |
| skipping patterns | Yes, Under the same MRRT setting, different measurement skipping patterns can result in different prediction accuracy | FFS Reason: According to RAN2 conclusions, the interpretation of skipping patterns is from the network perspective. It refers to the SSB configuration used to indicate the timing of the network's SSB transmission—not the timing of the UE's SSB measurement or skipping. RAN4 needs to clarify whether the understanding of skipping patterns here refers to skipping from the network perspective or the UE perspective. |
| Scenario 3: FR1 inter-frequency predictions in co-located scenario |  |  |
| Factor | Impact on performance (in RAN2 evaluation) | Whether to consider in the RAN4 evaluation |
| higher-to-lower frequency and lower-to-higher frequency case | Not much impact, The prediction accuracy is comparable between higher-to-lower frequency and lower-to-higher frequency case | Yes, Reason: It is recommended to consider both scenarios during performance evaluation to observe the performance differences between the two scenarios with consideration of measurement error. |
| Speed | Not much impact, The UE speed has minor impact on the prediction accuracy | No |
| correlation coefficient | Yes, The higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy | Yes |
| Cluster-based approach& single cell approach | Yes, The cluster approach can improve the prediction accuracy compared to single cell approach | Yes |

  - Performance monitoring

For performance monitoring, RAN2 conducted a general analysis of several monitoring methods during the SI phase. Currently, it remains open whether monitoring will be performed by the network side, the UE side, or via NW decision with UE assistance. Essentially, RAN4 needs to await further conclusions from RAN2 regarding the specific procedures for monitoring.

Captured in TR38.744 Performance of UE-sided model can be monitored in either network side or UE side after UE is configured with monitoring configuration and inference configuration. A monitoring window can be configured, over which the performance monitoring metric can be calculated. For network-sided monitoring, UE can report ground-truth measurement result and inference output to network. And it is up to network implementation to perform monitoring and make further management decision. For UE-sided monitoring, UE can report performance result, i.e. RSRP difference, to network based on ground-truth measurement result and inference output. It is up to network implementation to make management decision based on received performance result. For UE-sided monitoring, it can be considered for UE to make management decision based on network’s configuration and report the decision to network instead of performance result.

Specifically, at the RAN2#131 meeting, RAN2 reached the following conclusions on performance monitoring. From these conclusions, unlike AIML BM, an additional monitoring window has been introduced here. This monitoring window may impact the RRM requirements of the core part (e.g., defining a performance monitoring delay). For instance, the UE needs to perform measurements within this monitoring window (measurement delay for performance monitoring). In the case of UE-assisted monitoring (where the UE calculates the performance monitoring metric), it may be necessary to define the time within which the UE needs to complete monitoring and report the results.

RAN2# 131 meeting agreement: A monitoring window can be configured, over which the performance monitoring metric can be calculated.

Proposal 8: RAN4 to consider the RRM impact on Performance monitoring, taking into account further relevant conclusions from RAN2 (how the monitoring window works, monitoring/decision reporting schemes—periodic reporting or event-triggered reporting, UE/NW decision).

On the other hand, regarding KPIs related to performance monitoring, since the KPI discussions for RRM prediction are defined by RAN4, similarly, for performance monitoring, our understanding is that the definition of metrics also falls within RAN4's scope of discussion. Therefore, RAN4 also needs to define them.

Proposal 9: For RRM measurement prediction, it is within RAN4's scope of discussion to define the relevant metrics for performance monitoring (if any).

- Summary

Proposal 1: For RAN4 performance evaluation and requirement definition, RAN4 should focus on the following high-priority scenarios:

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Observation 1: Based on RAN2's conclusion, the implementation of RRM sub-use cases can be left to UE implementation.

Proposal 2: R RAN4 first to decide how to address/handle the RRM sub-use cases when evaluating performance and defining requirements.

- Option 1: During the performance evaluation phase, all three RRM sub-use cases are considered. Whether the requirement definition distinguishes between these three cases depends on the performance evaluation results. If there are significant performance differences among the three cases, three separate sets of metrics are defined for each case.

- Option 2: During performance evaluation and requirement definition, the performance of sub-use case 2 is used as the basis for defining the metrics. In the specification, the requirement definition does not distinguish or reflect the three RRM sub-use cases.

Proposal 3: During the performance evaluation phase and the requirements definition phase (e.g., potential prediction delay), RAN4 needs to consider both the OW window and the sliding/non-sliding window factors.

Proposal 4: For discussion on Case B, it depends on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission.

Proposal 5: For inter-frequency cell prediction, RAN4 should identify the potential RRM impacts resulting from the fact that inter-frequency cells are obtained through prediction and may not have been measured, such as cell known condition and CSSF

Proposal 6: RAN4 further discusses whether related LCM requirements are needed, depending on RAN2's design.

Proposal 7: For performance evaluation, RAN4 to identify the factors to be considered for each scenario:

| Scenario 1: FR2 intra-frequency temporal domain case A |  |  |
| --- | --- | --- |
| Factor | Impact on performance (in RAN2 evaluation) | Whether to consider in the RAN4 evaluation |
| Prediction window length | Yes, Longer PW length correlates with decreased prediction accuracy | Yes |
| UE speed | Yes, Higher UE speed correlates with decreased prediction accuracy | Yes |
| Cluster-based approach& single cell approach | Not much impact, The gain of cluster approach against single cell approach is not clear | FFS |
| Observation window length | Yes, Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit | Yes |
| Sub-use cases | Yes, A majority of the companies observe that RRM sub-use case 2 demonstrates higher prediction accuracy than RRM sub-use case 1 and RRM sub-use case 3 at least with short PW length | FFS Depending on the RAN4 discussion, whether all three cases will be considered in the performance evaluation. |
| Scenario 2: FR1 intra-frequency temporal domain case B |  |  |
| Factor | Impact on performance (in RAN2 evaluation) | Whether to consider in the RAN4 evaluation |
| Prediction window length | Same observation as scenario 1 | Yes |
| UE speed | Same observation as scenario 1 | Yes |
| Cluster-based approach& single cell approach | Same observation as scenario 1 | FFS |
| MRRT | Yes, Increasing MRRT correlates with decreased prediction accuracy | Yes |
| skipping patterns | Yes, Under the same MRRT setting, different measurement skipping patterns can result in different prediction accuracy | FFS Reason: According to RAN2 conclusions, the interpretation of skipping patterns is from the network perspective. It refers to the SSB configuration used to indicate the timing of the network's SSB transmission—not the timing of the UE's SSB measurement or skipping. RAN4 needs to clarify whether the understanding of skipping patterns here refers to skipping from the network perspective or the UE perspective. |
| Scenario 3: FR1 inter-frequency predictions in co-located scenario |  |  |
| Factor | Impact on performance (in RAN2 evaluation) | Whether to consider in the RAN4 evaluation |
| higher-to-lower frequency and lower-to-higher frequency case | Not much impact, The prediction accuracy is comparable between higher-to-lower frequency and lower-to-higher frequency case | Yes, Reason: It is recommended to consider both scenarios during performance evaluation to observe the performance differences between the two scenarios with consideration of measurement error. |
| Speed | Not much impact, The UE speed has minor impact on the prediction accuracy | No |
| correlation coefficient | Yes, The higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy | Yes |
| Cluster-based approach& single cell approach | Yes, The cluster approach can improve the prediction accuracy compared to single cell approach | Yes |

Proposal 8: RAN4 to consider the RRM impact on Performance monitoring, taking into account further relevant conclusions from RAN2 (how the monitoring window works, monitoring/decision reporting schemes—periodic reporting or event-triggered reporting, UE/NW decision).

Proposal 9: For RRM measurement prediction, it is within RAN4's scope of discussion to define the relevant metrics for performance monitoring (if any).

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital