3GPP TSG-RAN WG4 Meeting #116bis	R4-2513973

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.11.3

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# Introduction

RAN#109 approved the WI Exception for Rel-19 WI AI/ML for NR air interface [1], where this quarter focuses on the CSI prediction framework and requirements for LCM to complete this WI by RAN#110.

This contribution discusses the remaining open issues on CSI prediction framework according to the WI exception sheet and way forwards on AI/ML for NR air interface [2].

# Performance monitoring

RAN4#116 agreed to introduce Type 2 performance monitoring requirements for CSI prediction, but RAN4 could not conclude the detailed requirements.

Issue 1-1: Performance monitoring Agreement: - RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction - FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: - Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this Issue 1-4: Reporting delay requirement Agreement: - wait for RAN1 decision

## SGCS quantization mapping table

RAN1 has specified that UE reports the following two SGCS values in CSI Prediction Accuracy Indicator (CSI-PAI) for the UE-assisted performance monitoring metric:

- First SGCS (SGCSk1): calculated from the predicted PMI (wk) for one inference reporting and the non-predicted PMI (wk) based on the second Reporting Setting. UE shall report the first SGCS (SGCS1) for each MIMO layer k.

- Second SGCS (SGCSk2): calculated from the non-predicted PMI (wk) based on the second Reporting Setting and the PMI based on the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance (wk). UE shall report the second SGCS (SGCS2) for each MIMO layer k.

Figure 1 illustrates the relation between the first SGCS and second SGCS values.

Figure 1	Illustration of first SGCS and second SGCS reported in CSI-PAI.

RAN1 also agreed that the reported SGCS values range from 0 to 15 (4 bits), where 15 codepoints are mapped to uniformly quantize in range [0.3, 1] in a linear scale with 0.05 step size, and one codepoint is used to indicate the value range of (0, 0.3]. RAN plenary approved the CR to introduce the SGCS quantization mapping table in TS 38.214 5.2.1.4.6 [4] (See Appendix).

Observation 1: RAN1 has specified the SGCS quantization mapping table in TS 38.214 5.2.1.4.6.

Proposal 1: RAN4 does not need to specify the SGCS quantization mapping table in TS 38.133.

According to the latest UE feature list [3], the performance monitoring for CSI prediction (FG 58-3-5) is optional feature even if UE supports CSI prediction for UE-sided inference when N4=1 (FG 58-3-1). In other words, if the UE supports the CSI-PAI reporting, this UE should support AI/ML-based CSI prediction.

Observation 2: The performance monitoring for CSI prediction (FG 58-3-5) is optional even if UE supports CSI prediction for UE-sided inference when N4=1 (FG 58-3-1).

Observation 3: If UE supports the CSI-PAI reporting, this UE should support AI/ML-based CSI prediction.

## Reporting delay

RAN1 has specified the measurement resources used for CSI-PAI reporting in TS 38.214 5.2.1.4.6 for both aperiodic and semi-persistent CSI-PAI reporting. Since TS 38.214 clearly specifies the measurement period of CSI-PAI, RAN4 does not need to specify the reporting delay requirements in TS 38.133; RAN4 only needs to refer to TS 38.214 5.2.1.4.6.

The following texts are an example TP to TS 38.133 how to specify the CSI-PAI reporting delay.

The UE shall transmit the semi-persistent CSI-PAI reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4.6 in TS 38.214. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic CSI-PAI reporting on PUSCH over the air interface at the time specified according to clause 5.2.1.4.6 in TS 38.214.

Observation 4: RAN1 has specified the measurement period for aperiodic/semi-persistence CSI-PAI reporting in TS 38.214.

Proposal 2: RAN4 only needs to refer to TS 38.214 5.2.1.4.6 if RAN4 will define the CSI-PAI reporting delay requirements in TS 38.133.

## SGCS measurement accuracy

RAN4#116 also discussed whether to define the CSI-PAI reporting accuracy requirements. The measurement accuracy requirements specified in TS 38.133 verify the measurement errors from the true value. For example, L1-RSRP measurement accuracy is defined as the difference between the reported L1-RSRP value and true L1-RSRP value where TE sets during the measurement test.

On the other hand, SGCS1 and SGCS2 in CSI-PAI reporting is calculated from the predicted PMI and non-predicted PMI. Since both the quantities are measured inside UE, TE does not know the true PMI. Therefore, it is not feasible to verify the ‘measurement accuracy’ of SGCS1 and SGCS2 in TE.

Observation 5: It is not feasible to verify the measurement accuracy of SGCS1 and SGCS2 in TE.

One test metric is to check a statistic of reported SGCS1, where a requirement is that SGCS1 should be more than X with 90% of the test time, e.g., X=0.85, like the CQI fading test specified in TS38.101-4.

Figure 2 and Figure 3 show CDFs of SGCS1 and SGCS2, respectively, for 32 CSI-RS ports with TDLD30-20 with different SNR test points, according the simulation assumption [5]. The simulation results show that the reported SGCSs becomes low (<0.3) as the SNR test point is decreased.

| (a) Unquantized SGCS1 | (b) Quantized SGCS1 |
| --- | --- |

Figure 2	CDF of CSI-PAI SGCS1 for 32 CSI-RS ports with TDLA30-20 medium correlation

| (a) Unquantized SGCS2 | (b) Quantized SGCS2 |
| --- | --- |

Figure 3	CDF of CSI-PAI SGCS1 for 32 CSI-RS ports with TDLA30-20 medium correlation

Another test metric could be a ratio of the first SGCS and second SGCS, that is, SGCS1/SGCS2. Figure 4 shows CDFs of the ratio of SGCS1 and SGCS2 for 32 CSI-RS ports with TDLD30-20 with different SNR test points.

If we focus on 90%-ile of CDF, the ratio becomes less than 1.0 for SNR < 0dB, which means the SGCS of the predicted PMI is worse than the non-predicted PMI. On the other hand, the ratio SGCS1/SGCS2 > 1.3 for higher SNR test points such as SNR>10dB.

| (a) Unquantized SGCS1/SGCS2 | (b) Quantized SGCS1/SGCS2 |
| --- | --- |

Figure 4	CDF of ratio CSI-PAI SGCS1/SGCS2 for 32 CSI-RS ports with TDLA30-50 medium correlation

We propose RAN4 should consider using the statistics of reported SGCS1 and/or ratio of reported SGSC1 and SGCS2 as the test metric of CSI-PAI reporting performance requirements. However, the issue of such a requirement is that UE can report any SGCS values.

As we observed before, since the UE supporting CSI-PAI reporting (FG 58-3-5) should also support the predicted PMI reporting (FG 58-3-1), we propose to check the PDSCH throughput with the reported predicted PMI discussed in the performance part. Considering the purpose of CSI-PAI is to give information to the NW-side the AI/ML-based CSI prediction performance, RAN4 should consider at least two cases: one case for the prediction works, and another case for the prediction does not work.

Proposal 3: Study the feasibility to set the CSI-PAI reporting requirements with that the SGCS1 should be more than X1 in [90]% of the test time with SNR=Y1 (dB), e.g., X1=0.85 and Y1=10dB.

Proposal 4: Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than X2 in [90]% of the test time with SNR=Y2 (dB), e.g., X1=1.2 and Y1=10dB.

Proposal 5: If UE support CSI-PAI reporting capability, check the statistics of SGCS together with throughput ratio of follow predicted PMI and random Rel-15 TypeI PMI.

Proposal 6: RAN4 should consider at least two cases: one case for the AI/ML-based prediction works, and another case for the prediction does not work well.

# Summary

Observations:

Observation 1: RAN1 has specified the SGCS quantization mapping table in TS 38.214 5.2.1.4.6.

Observation 2: The performance monitoring for CSI prediction (FG 58-3-5) is optional even if UE supports CSI prediction for UE-sided inference when N4=1 (FG 58-3-1).

Observation 3: If UE supports the CSI-PAI reporting, this UE should support AI/ML-based CSI prediction.

Observation 4: RAN1 has specified the measurement period for aperiodic/semi-persistence CSI-PAI reporting in TS 38.214.

Observation 5: It is not feasible to verify the measurement accuracy of SGCS1 and SGCS2 in TE.

Proposals:

Proposal 1: RAN4 does not need to specify the SGCS quantization mapping table in TS 38.133.

Proposal 2: RAN4 only needs to refer to TS 38.214 5.2.1.4.6 if RAN4 will define the CSI-PAI reporting delay requirements in TS 38.133.

Proposal 3: Study the feasibility to set the CSI-PAI reporting requirements with that the SGCS1 should be more than X1 in [90]% of the test time with SNR=Y1 (dB), e.g., X1=0.85 and Y1=10dB.

Proposal 4: Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than X2 in [90]% of the test time with SNR=Y2 (dB), e.g., X1=1.2 and Y1=10dB.

Proposal 5: If UE support CSI-PAI reporting capability, check the statistics of SGCS together with throughput ratio of follow predicted PMI and random Rel-15 TypeI PMI.

Proposal 6: RAN4 should consider at least two cases: one case for the AI/ML-based prediction works, and another case for the prediction does not work well.

# References

- RP-252840, “Rel-19 Work Item Exception for Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, CAICT, Ericsson.

- R4-2511890, “WF on AI/ML for air interface for [116][131]”, Qualcomm Incorporated.

- R1-2506627, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #122”, RAN1.

- RP-252627, “Correction to Rel-19 AI/ML for NR Air Interface (38.212/214/215)”, RAN1.

- R4-2511796, “WF on updated simulation assumptions for AI based CSI prediction”, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple.

# Appendix

Table 5.2.1.4.6-1: SGCS Quantization Mapping

| Code Point | Range |
| --- | --- |
| 0 | [0.0000, 0.3000] |
| 1 | (0.3000, 0.3500] |
| 2 | (0.3500, 0.4000] |
| 3 | (0.4000, 0.4500] |
| 4 | (0.4500, 0.5000] |
| 5 | (0.5000, 0.5500] |
| 6 | (0.5500, 0.6000] |
| 7 | (0.6000, 0.6500] |
| 8 | (0.6500, 0.7000] |
| 9 | (0.7000, 0.7500] |
| 10 | (0.7500, 0.8000] |
| 11 | (0.8000, 0.8500] |
| 12 | (0.8500, 0.9000] |
| 13 | (0.9000, 0.9500] |
| 14 | (0.9500, 1.0000] |
| 15 | Reserved |