3GPP TSG RAN WG4 #116-bis	R4-2513981

# Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.16.3

Source: 	Ericsson

Title:	Discussion on performance requirement for LP-WUR

Document for:	Discussion

# Introduction

In last meeting, RAN4 mainly focus on complete the core requirements on Rel-19 LP-WUS WI and didn’t discuss too much about the performance part. In this contribution, we’ll discuss the general test case design for LP-WUS [1].

- General aspects

Scenario

When LP-WUS is introduced in Rel-19, RAN4 defined two different scenarios to trigger the LP-SS measurement, one is MR relaxation and the other is MR offloading. RAN4 to define the test cases to verify both scenarios.

Proposal 1: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.

Test Methodology

In LP-WUS, for RRM relaxation, the test cases similar as Rel-16 cell reselection with relaxation can be introduced.

  - intra-frequency cell re-selection (LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (LP-SS based LR and/or SSB based LR)

Proposal 2: RAN4 to introduce the RRM relaxation test cases for cell reselection as follow.

  - intra-frequency cell re-selection (LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (LP-SS based LR and/or SSB based LR)

In LP-WUS, a new MR offloading mode is defined. In this mode, UE is only leveraging the LR to monitor the serving cell status and the MR is totally shut off. If we define the test case based on legacy cell reselection, one possible solution is to define a long cell reselection period. In time T1, UE enters LR mode based on good quality and reasonable threshold. After T1, the transmitted signal quality will be lower and trigger the UE to fallback to MR mode and UE will start cell reselection procedure. However, this method seems not verify the UE’s behvaiour in LR mode directly.  Another solution is to introduce a new test mode, when UE enters LR or fallbacks to MR, some new indications will be triggered to test equipment. Therefore, RAN4 needs to discuss the test methodology for MR offloading test first.

Proposal 3: RAN4 to discuss the test methodology about how to verify MR RRM offloading scenario, such as whether to introduce a new test mode.

Test Configuration

In LP-WUS, two different reference signals requirement are defined, LP-SS and legacy SSB. RAN4 can define a single test case to verify the LP-WUS UE with different capabilities for either LP-SS or SSB. However, if UE supports both LP-SS and SSB, RAN4 needs to discuss whether to introduce the applicability to the test, and which RS can be used for test. Currently, RAN4 defined both FR1 and FR2 requirement for LP-WUS.. Thus, we prefer to introduce the test case related to FR1 and OFDM based LR for FR2. RAN4 needs to introduce the new LP-SS configuration with 320ms periodicity.

Proposal 4: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE.

Proposal 5: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB.

Proposal 6: RAN4 to introduce both FR1 and FR2 based LP-WUS test case.

Proposal 7: RAN4 to introduce new LP-SS configuration for LP-WUS test case.

- Summary

In this contribution, we have initially discussed the LP-WUS performance part. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.

Proposal 2: RAN4 to introduce the RRM relaxation test cases for cell reselection as follow.

Proposal 3: RAN4 to discuss the test methodology about how to verify MR RRM offloading scenario, such as whether to introduce a new test mode.

Proposal 4: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE.

Proposal 5: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB.

Proposal 6: RAN4 to introduce both FR1 and FR2 based LP-WUS test case.

Proposal 7: RAN4 to introduce new LP-SS configuration for LP-WUS test case.

- References

[1]. R4-2512139, “WF on RRM requirements for NR_LPWUS”, vivo

[2]. RP-240773, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony