3GPP TSG RAN WG4 #116-bis	R4-2513984

# Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.15.2.3

Source: 	Ericsson

Title:	Discussion on performance requirements for OD-SIB1

Document for:	Discussion

# Introduction

In RAN4 #115 meeting, RAN4 discussed about OD-SIB1 procedure’s impact to cell reselection procedure with he following agreement.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

In this contribution, we’ll discuss the related test case design for OD-SIB1 procedure[1].

# OD-SIB1 test case

In RAN4 #115 meeting, RAN4 discussed whether to define additional paging interruption requirement for OD-SIB1 acquisition with cell reselection in IDLE mode. RAN4 agrees to define a test case with longer TSI-NR in performance part. To verify the new OD-SIB1 acquisition procedure in IDLE mode, the following test setup steps can be follows.

- Step 0: Two cells are configured, cell 1 and cell 2. Cell 2 is an OD-SIB1 cell.

- Step 1: The UE evaluates cell reselection based on measurements towards SSB of target cell2.

- Step 2: After cell reselection delay, the UE initiates OD-SIB1 procedure. (The total delay should be defined in the test)

  - Step 2-1: UE transmits the UL-WUS(PRACH)

  - Step 2-2: UE receives the RAR

  - Step 2-3: UE monitors OD-SIB1 window to decode the SIB1 and OSIs

Proposal 1:  RAN4 to define a new test case to verify the OD-SIB1 procedure in IDLE mode.

- Summary

In this contribution, we have discussed the performance part of OD-SIB1 procedure. Based on the discussions, we made the following proposals and observations:

Proposal 1:  RAN4 to define a new test case to verify the OD-SIB1 procedure in IDLE mode.

- Step 0: Two cells are configured, cell 1 and cell 2. Cell 2 is an OD-SIB1 cell.

- Step 1: The UE evaluates cell reselection based on measurements towards SSB of target cell2.

- Step 2: After cell reselection delay, the UE initiates OD-SIB1 procedure. (The total delay should be defined in the test)

  - Step 2-1: UE transmits the UL-WUS(PRACH)

  - Step 2-2: UE receives the RAR

  - Step 2-3: UE monitors OD-SIB1 window to decode the SIB1 and OSIs

- References

- R4-2512327, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson