3GPP TSG-RAN WG4 Meeting #116bis	R4-2514055

Prague, Czech Republic, 13-17 October, 2025

Source:	Nokia

Title:	Discussion on LTM event-triggered reporting test cases

Agenda item:	6.17.2.1

Document for:	Discussion

Introduction

In this contribution we discuss the test cases for event-triggered L1 reporting for LTM.

Discussion

Event-triggered L1 measurement reporting

For event-triggered L1 reporting, RAN4 defined and agreed draft CRs for event-triggered reporting delay considering:

- Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

- Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

- Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

In the last meeting, the test coverage was discussed under issue 4-1-1, where the recommended way forward was the following:

Issue 4-1-1: test case list for event-triggered L1 reporting (Proposals not included) Recommended WF: - Further discuss based on the following directions - For SSB based L1 RSRP measurement reporting: - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple) - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW) - For CSI-RS based L1 RSRP measurement reporting: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1)

As event-triggered L1 reporting is a new type of reporting, the related test cases obviously need to be defined in addition to the Rel-18 test cases. RAN4 should define test cases to cover all the listed event-triggered reporting cases, including the supported SSB and CSI-RS based reporting scenarios. Additionally, RAN4 should define at least one test case to verify that the UE correctly follows event-triggered periodic reporting requirements.

To cover the testing of all these requirements, we propose to introduce the following test cases:

Introduce the following test cases for event-triggered LTM L1 reporting:

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case should test event-triggered periodic reporting (e.g. FR1 Intra-frequency SSB-based event-triggered L1 reporting)

From our side it is acceptable to consider that for SSB based reporting test cases, if the UE passes a Rel-19 event-triggered reporting test case, the UE can skip the corresponding Rel-18 test case.

For SSB based measurement reporting, if the UE passes the Rel-19 event-triggered L1 reporting test case, the UE can skip the corresponding Rel-18 test case.

For CSI-RS based reporting test cases, the two Rel-19 measurement capabilities (early CSI-RS measurements and event-triggered L1 reporting based on CSI-RS measurements) are both optional UE capabilities. UE can either support both or only the CSI-RS measurement capability without event-triggered reporting. The corresponding test cases are to be defined under two separate sub agenda items.

We think event-triggered reporting test case can be considered as the prioritized test case, and the UE supporting both capabilities would need to only pass the event-triggered CSI-RS reporting test case. If UE only supports CSI-RS measurement capability without event-triggered reporting, the UE should pass that test case.

If the UE supports both Rel-19 CSI-RS measurement capability and Rel-19 event-triggered reporting capability for CSI-RS measurement, UE only needs to pass the event-triggered CSI-RS reporting test case.

Test configuration aspects were discussed under issue 4-1-2 in the last meeting:

Issue 4-1-2: test configuration for event-triggered L1 reporting Candidate options: - Proposal 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia) - Use LTM3 as the reporting event in the test cases. - Proposal 2: it is proposed to define test cases for LTM event 2/3/4/5. (CMCC) - Proposal 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW) - Proposal 4: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///) Agreement: - On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

RAN4 agreed the reporting delay to be the maximum measurement period of the cells involved in the event. RAN2 has agreed to define the following L1 events for LTM reporting:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In the test cases, it was agreed in the last meeting that on high level we define the test setup so that there is one serving cell and one candidate cell. One of the cells’ (e.g. serving cell) signal quality shall remain constant during the test. The signal level of the other cell (e.g. candidate cell) shall be changed during the test so that the reporting condition becomes fulfilled, and the UE shall send the report within the defined reporting delay. The supported events consider either just the serving cell (LTM2), just the candidate cell (LTM4), or both (LTM3 and LTM5). We propose to use LTM3 as the reporting event in the test cases, where signal level of both cells is considered in the event.

Use LTM3 as the reporting event in the test cases.

Conclusion

The following proposals were made:

Proposal 1: Introduce the following test cases for event-triggered LTM L1 reporting:

a. FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

b. FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

c. FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

d. At least 1 test case should test event-triggered periodic reporting (e.g. FR1 Intra-frequency SSB-based event-triggered L1 reporting)

Proposal 2: For SSB based measurement reporting, if the UE passes the Rel-19 event-triggered L1 reporting test case, the UE can skip the corresponding Rel-18 test case.

Proposal 3: If the UE supports both Rel-19 CSI-RS measurement capability and Rel-19 event-triggered reporting capability for CSI-RS measurement, UE only needs to pass the event-triggered CSI-RS reporting test case.

Proposal 4: Use LTM3 as the reporting event in the test cases.