3GPP TSG-RAN WG4 Meeting #116bis	R4-2514070

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for enhanced requirements and test methodology for NR and IoT NTN

Agenda Item:	7.6.4.2

Document for:	Discussion

# Introduction

The WI NR Enhanced requirements and test methodology for NR and IoT NTN Phase 2, which targets release Rel-20 was approved, which can be found in [1].

In this contribution, we present our initial views on the underlying impacts to RRM requirements mandated by the objectives in the WI.

# Discussion

In the WI, the objective of HD-FDD for NTN includes a clear request to update the RRM requirements.

Objective of Core part WI HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

With respect to the plan, RRM requirements to support HD-FDD VSATs in Ku bands with FR1 numerology shall be initiated first.

The Rel-20 WI specifies that the applicable UEs that are VSATs in FR1-NTN equipped with electronically steered antennas, covering fixed and mobile variants as well as GSO, LEO, and GSO+LEO support, in line with the definitions of VSAT types 2, 3, 5, and 6 as referenced from TS 38.101-5 and the related RF agreement, which are duplicated in the below.

As a reference, the scope of the applicable UEs in the WI Ku-band NTN in Rel-19included following: has below considerations:

- GSO and NGSO

- NTN capable UE type 4 and 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices)

- FDD Mode

- timing requirements for mobile VSAT served by NGSO

Rel-19 WI has covered GSO and NGSO, mobile/fixed, electronic steering antenna  already and same requirements (except for timing requirements) are applicable for the variants of VSATs. Therefore, put HD-FDD/FDD aside, it appears that VSATs in the Ku band with FR1 numerology, including VSAT types 2, 3, 5, 6 with electronically steered antenna, can be covered by the existing specification with same RRM requirement scope, expect for timing requirements, for all variants of VSATs.

And, furthermore, timing requirements for Ku-band NTN in FR1-NTN have achieved agreements already in Rel-19, as copied in the below:

Agreement: - Both 15kHz and 30kHz SCS for timing error requirements should be considered for Ku-band Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 30; 22*64*Tc. NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6].

To our understanding, the same timing error requirements are applicable for the applicable UEs in this Rel-20 WI.

Intuitively, a rational starting point is to take VSATs in the Ku band with FR1 numerology with FDD, as defined in Rel-19, as the baseline and assess the impacts of HD-FDD on all applicable requirements. And, only the requirements concerning electronically steered antennas shall be taken into account.

Observation 1: An intuitive startpoint is to take VSATs in the Ku band with FR1 numerology, as defined in Rel-19, as the baseline and assess the impacts of HD-FDD on all applicable requirements for VSATs in the Ku band with FR1 numerology.

In WI Ku-band NTN in Rel-19, it’s generally agreed that unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT operating in cells with FR1-NTN numerology, which may include the below requirements.

- Cell Re-selection

- UE transmit timing for Satellite Access

- Signaling, e.g., Radio Link Monitoring, Link Recovery Procedures etc.

- Blind HO

- Satellite switching with re-synchronization

Observation 2: In Rel-19, VSATs in Ku bands with FR1 numerology defined the below requirements without inter-satellite measurements:

- Cell Re-selection

- UE transmit timing for Satellite Access

- Signaling, e.g., Radio Link Monitoring, Link Recovery Procedures etc.

- Blind HO

- Satellite switching with re-synchronization

Accordingly, we understand that this WI shall not extend the applicable scenairos for VSATs in Ku bands with FR1 numerology.

Proposal 1: RAN4 to confirm that VSATs in Ku bands with FR1 numerology in Rel-20 WI does not extend applicable scenairos and corresponding requirements beyond the scope of WI Ku-band NTN in Rel-19.

Proposal 2: RAN4 to confirm that the existing timing requirements for FR1-NTN are applicable for VSATs in Ku bands with FR1 numerology in Rel-20 WI.

With resepct to WI NTN Ph3 in Rel-19, the  HD-FDD operation is introduced for RedCap in NTN, and it impacts the below requirements.

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- Minimum requirement for L1 indication for RLM and LR

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 3: RAN4 to consider the impacts of HD-FDD on the below requirements and, furthermore, take into account of the specific characteristics of VSAT UEs.

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- Minimum requirement for L1 indication for RLM and LR

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

# Conclusion

Observation 1: An intuitive startpoint is to take VSATs in the Ku band with FR1 numerology, as defined in Rel-19, as the baseline and assess the impacts of HD-FDD on all applicable requirements for VSATs in the Ku band with FR1 numerology.

Observation 2: In Rel-19, VSATs in Ku bands with FR1 numerology defined the below requirements without inter-satellite measurements:

- Cell Re-selection

- UE transmit timing for Satellite Access

- Signaling, e.g., Radio Link Monitoring, Link Recovery Procedures etc.

- Blind HO

- Satellite switching with re-synchronization

Proposal 1: RAN4 to confirm that VSATs in Ku bands with FR1 numerology in Rel-20 WI does not extend applicable scenairos and corresponding requirements beyond the scope of WI Ku-band NTN in Rel-19.

Proposal 2: RAN4 to confirm that the existing timing requirements for FR1-NTN are applicable for VSATs in Ku bands with FR1 numerology in Rel-20 WI.

Proposal 3: RAN4 to consider the impacts of HD-FDD on the below requirements and, furthermore, take into account of the specific characteristics of VSAT UEs.

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- Minimum requirement for L1 indication for RLM and LR

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

# References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN