3GPP TSG RAN4 Meeting #116bis																R4-2514073

Prague, Czhech 13th – 17th Oct, 2025

Title: 	On High power UE (HPUE) for NR single carrier operation

Agenda Item:	7.1.4

Source: 	Nokia

Document for:	Approval

# 1				Introduction

New Rel-19 WI for 5G-A UE RF was approved [1]. In this contribution we focus on High power UE (HPUE) for NR single carrier operation objectives

As a re-cap of the objctive:

High power UE (HPUE) for NR single carrier operation

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

  - This work is to start in 2026 Q4

# 2	Discussion

Purpose of the first objective single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations is to enable to build FWA device that can produce power class 1 level of output power using current off-the-self components. Currently power class 1 specifications assume that single PA Tx architecture is used and availability of these PC1 PAs and filters that can tolerate such a high-power level is limited.

26 dBm PAs are nowadays common for TDD bands and all example bands of the WI are TDD bands. Using four 26 dBm PA the output power would be 32 dBm if one does the math. Now power class 1 is 31 dBm. In RAN#109 there was discussion how to solve the discrepancy, but no conclusion was reached. One way is to reuse PC1 and rely to the fact that 32 dBm is still within tolerances. For this option a concearn was raised that in this case then a UE capability is needed to separate 1Tx and 4Tx UEs. Another proposed way was to define new power class 1bis which would be 32 dBm. With new power class there is no need for new UE capability to distinguish 1Tx and 4Tx.

We prefer to define new power class 1Bis for 32 dBm as it is a clean solution and there is no ambiquity about the UE Tx architecture. Also we think that it is important to highlight the fact that 4x26 dBm is 1 dB more than PC1, all dBs count.

Proposal 1: Define new power class 1Bis for 32 dBm.

A major task for this work is to do the co-existance studies for FWA use case for 32 dBm output power. Current PC1 specifications were done based on vehicular use case and the ACLR requirement for that used case is 37 dB for PC1.In NTN there is also PC1 defined for NTN use case and part of that work co-existance studies were also performed which shoved that 37 dBm ACLR can be relaxed.

Proposal 2: Discuss and aim to agree co-existance simulation assumptions in RAN4#117 for 32 dBm 4Tx FWA use case

The second objective PC1.5 with 2Tx for FDD bands for handheld and FWA UE is building on existing specifications as PC1.5 with 2Tx for TDD feature is already specified.

Observation 1: PC1.5 with 2Tx for TDD feature is already specified and those emission requirements can be extended into FDD operation.

It is quite likely that MPR requirements for PC1.5 with 2Tx for TDD are also applicable for FDD. In fact the PC1.5 TDD feature is reusing PC2 MPR as it is realized by using two 26 dBm PAs.

Observation 2: PC1.5 with 2Tx for TDD MPR maybe be extendedable for FDD operation.

However the example bands n1, n25 do need A-MPR study which is the major part of the work for PC1.5 with 2Tx for FDD.

Observation 3: A-MPR study is need for PC1.5 with 2Tx for FDD example bands n1, n25.

It is also anticipated that due to FDD operation and higher Tx power than before self-desentization issue needs a study.

Observation 4: self-desentization (MSD) needs to be studies for example bands n1 and n25.

# 3	Conclusion

In this contribution we have made following proposals and observations.

Proposal 1: Define new power class 1Bis for 32 dBm.

Proposal 2: Discuss and aim to agree co-existance simulation assumptions in RAN4#117 for 32 dBm 4Tx FWA use case

Observation 1: PC1.5 with 2Tx for TDD feature is already specified and those emission requirements can be extended into FDD operation.

Observation 2: PC1.5 with 2Tx for TDD MPR maybe be extendedable for FDD operation.

Observation 3: A-MPR study is need for PC1.5 with 2Tx for FDD example bands n1, n25.

Observation 4: self-desentization (MSD) needs to be studies for example bands n1 and n25.

# 4	References

[1] RP-252942, UE RF enhancements for NR FR1, Phase 5, RAN#109