# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514094

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements on Ambient-IoT

Agenda item:	6.14.3

Document for:	Approval

Introduction

RRM core requirements for Rel-19 Ambient-IoT are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our views on RRM performance requirements on Ambient-IoT.

Discussion

In last meeting, we have already confirmed that the random access and D2R transmit timing requirements will be tested in RAN4 test cases, the agreement could be concluded as below:

Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test.

For transmit timing requirements, we believe that it refers to legacy UL transmit timing error test cases. RF session has not studied the SFO impact on timing, only RRM session has this transmit timing error requirements. Consequently, this test case shall be studied in RRM session. In last meeting, we have reached the agreement as below:

- Issue 1-2-1: Device transmit timing For D2R transmission, the device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH of the corresponding R2D transmission to the starting time of D2R transmission. The reference point for the device transmission timing is TR→D after the end of R2D transmission intended for the device as define in 7.1.2 TS 38.291 TR→D is For CBRA MSG1 X=1: TR→D is Toffset1 after the end of corresponding R2D transmission For CBRA MSG1 X=2: TR→D is Toffset1+ Toffset2 after the end of corresponding R2D transmission For CBRA MSG3: TR→D is Toffset3 after the end of corresponding R2D transmission For CFRA first D2R: TR→D is Toffset3 after the end of corresponding R2D transmission For D2R transmission except for Msg1 and Msg3: TR→D is Toffset4 after the end of corresponding R2D transmission Note: considering that TR→D is in use in RAN1 spec (TS 38.291), above details about TR→Dwill not be captured in TS38.191 - Issue 1-2-2: The value for TD2R_min - The value of TD2R_min is max {3* TD2R_chip, 10 us}, where TD2R_chip is the chip length of D2R transmission

In my opinion, we shall distinguish several cases:

For CBRA, the following case shall be tested since they have different procedures on signal transmission:

- MSG1 X=1, Toffset1;

- MSG1 X=2. Toffset1+Toffset2.

- Except for MSG1 and MSG3, Toffset4.

For CFRA, one case shall be tested:

- First D2R transmission.

We also reached the agreement on TD2R_min，the timing for device Tx-Rx switching time shall also be tested.

Proposal 1: For device transmit timing, the following test cases shall be considered:

- For CBRA: MSG1 X=1, Toffset1; MSG1 X=2. Toffset1+Toffset2. Except for MSG1 and MSG3, Toffset4. - For CFRA: First D2R transmission. - For TD2R_min

For random access, we reached the core agreements as below:

Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message

We clarify the detailed random access procedure for CBRA and CFRA. We deem that the whole procedure shall be tested since this procedure is different from legacy test case and it doesn’t have too much workload. If the whole procedure is tested, the contents of Ambient-IoT in specification will be more complete.

Proposal 2: RAN4 shall define test cases one by one for the whole CBRA procedure and CFRA procedure.

Conclusion

In this paper we provided our views on RRM performance requirements on R19 Ambient-IoT.

Proposal 1: For device transmit timing, the following test cases shall be considered:

- For CBRA: MSG1 X=1, Toffset1; MSG1 X=2. Toffset1+Toffset2. Except for MSG1 and MSG3, Toffset4. - For CFRA: First D2R transmission. - For TD2R_min

- Proposal 2: RAN4 shall define test cases one by one for the whole CBRA procedure and CFRA procedure.

- 4 References

- TS38.133