# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514099

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for LP-WUS

Agenda item:	6.16.3

Document for:	Approval

Introduction

RRM core requirements for Rel-19 LP-WUS are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on LP-WUS.

Discussion

- In previous meeting, we mainly discuss the core part on LP-WUS and the agreements are stable. In this stage, we would like to do some analysis on performance part. The following analysis could be considered from our side.

Topic 1 Measurement accuracy requirements

- For LR accuracy requirements, RAN4 has reached one agreement in RAN4 #114bis meeting as below:

Agreement: No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement.

- It says the related accuracy requirements shall not be defined in performance part, it shall be reflected as a margin to the threshold in core requirements. Consequently, it is no need to define LR accuracy in performance part. For MR accuracy requirements, we deem that the legacy accuracy requirements in FR1 and FR2 shall be reused. Since in the last meeting, we have already defined the RRM requirements in FR2. Thus, we do think the accuracy requirements for both FR1 and FR2 shall be considered.

- Proposal 1: For LR accuracy, it is no need to be defined in performance part.

- Proposal 2: For MR accuracy, the legacy accuracy requirements shall be reused.

- Proposal 3: The accuracy requirements for FR1 and FR2 shall be both considered.

Topic 2 General Aspects

- The way forward could be seen as below:

- Proposals - P1: RAN4 to focus on time and delay behavior of RRM offloading, RRM relaxation and LP-WUR monitoring modes. (Apple vivo) - P2: Do not define a test mode for LP-WUS/WUR performance part. (Apple) - P3: Define test mode for LP-WUR serving cell measurement tests (oppo Nokia) - P4: Defining TCs for LPWUS cannot verify whether UE meet the new RRM relaxation requirements in R19 LPWUS or the legacy requirements without RRM relaxation (in both cases UE will pass the test). The TCs are redundant. (MTK)

- For proposal 1, we agree with RAN4 shall focus on time and delay behaviour of RRM offload, RRM relaxation and LP-WUR monitoring mode. In core part, RAN4 mainly discussed the core requirement on IDLE/INACTIVE state. The main time and delay is cell re-selection delay. The cell re-selection delay to a newly detectable cell can be expressed as Tdetect,NR_intra/inter + TSI-NR, and to an already detected cell can be expressed as: Tevaluate,NR_intra/inter + TSI-NR. The performance test shall focus on cell re-selection time and delay, which RAN4 has already defined the evaluation requirements in core part. Test cases should confirm the cell re-selection procedure shall be completed with the defined delay requirements.

For proposal 2 and proposal 3, the main difference is whether to introduce the test mode for entering LP-WUR coverage. Actually, we do not have strong view on this issue. We know that if a UE staying at legacy state would like to enter into the LP-WUS coverage, the MR measurement threshold shall be mandatory and LR measurement threshold is optional. However, whether to enter or not is up to UE implementation. The proponents for introducing test mode think if there is no new test mode, the UE will not be guaranteed to monitor LP-WUS signal with a large probability. The new test mode just wants to ensure the UE enters the right state before or during the test is performed in order to complete the whole test. Nonetheless, some companies say that the entry is up to UE implementation, which means at the beginning of a test case, the UE will already in the LP-WUR monitoring mode. To our understanding, we shall confirm one thing is that the test case for LP-WUR monitoring shall be considered, as for what method to use we could have some discussion in further meeting. If test mode is defined, it is OK for us since UE have to enter into the LP-WUR monitoring and perform related test. If not, the UE shall be initialed in LP-WUR monitoring mode and perform related test at the beginning of the whole tests.

Proposal 4: The whole test shall focus on time and delay behaviour of RRM offloading, RRM relaxation and LP-WUR monitoring modes.

Proposal 5: For whether to introduce new test mode,

- If new test mode is introduced, UE shall perform tests on LP-WUR monitoring at certain occasion.

- If not, UE shall be initialed in LP-WUR mode directly and perform test at the beginning of a test case.

Overall, the test case for LP-WUR monitoring shall be considered. RAN4 could further discuss the proper method to be utilized.

Topic 3 Test case scenarios

For test case scenarios, we have two proposals as below:

- Proposals - P1: Consider test case for the following scenarios: 1: LP-WUR monitoring related test case; 2: RRM offloading related test case; 3: RRM relaxation related test case (CATT China telecom vivo) - P2: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.(Ericsson Nokia)

In fact, it is evident that proposal 1 and proposal 2 tells the same thing. For RRM offloading related test case means MR RRM offloading scenarios. For RRM relaxation related test case equals to MR RRM relaxation scenarios. Thus, we prefer proposal 1, which contains all of scenarios.

Proposal 6: Consider test case for the following scenarios:1: LP-WUR monitoring related test case; 2: RRM offlaoding related test case; 3. RRM relaxation related test case.

Topic 4 Test case for OOK based LR and OFDM based LR

In the latest WID, we have two kinds of LP-WUR which are OOK based LR and OFDM based LR. For OOK based LR, the measurement RS is LP-SS. For OFDM based LR, the measurement RSs are SSS/PSSS and LP-SS. For these two different LR, the following test case could be seen:

- Proposals - P1: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR. (Apple oppo CMCC China telecom vivo Huawei Nokia) - P2: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE (Ericsson) - P3: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB.(Ericsson) - P4: Discuss how to define a test case / configuration for OFDM capable UE (FG-62-1a) which is only configured with LP-RSRP & LP-RSRQ (no SS-RSRP, SS-RSRQ configuration). (Nokia)

In previous meetings, we discussed the LR related evaluation requirements are:

112bis meeting: (LP-SS ) Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. 113 meeting:(SSB) If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. If LP-SS is configured, apply the same agreement as for when LP-SS is not configured 114 meeting:(OFDM-based receiver LP-SS/ OOK-based receiver LP-SS) - In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not. - Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based

We observe that different reference signals are configured, the measurement delay requirements are different. Hence, we would like to separate test cases between OOK based LR and OFDM based LR.

Proposal 7: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR.

Topic 5 Test case design for RRM relaxation

In previous clarification, we emphasize the test case shall also be defined on RRM relaxation. This RRM relaxation mainly refer to MR RRM measurement relaxation in case 3 including serving cell and neighbour cell measurement relaxation, which is 16x legacy delay. This test case is akin to R16 power saving. To our understanding, for RRM relaxation, the following tests can be considered:

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

- Proposal 8: For defining test cases on RRM relaxation, the following tests can be considered:

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

Topic 6 Test case design for higher priority frequency layer search

- In core part, we also defined the requirements for higher priority frequency layer search. The way forward for test cases on higher priority frequency layer search could be seen as below:

- Proposals - Option 1: Introduce test case for high priority frequency search in case 1. (Apple oppo Huawei ZTE) - Option 2: For RRM measurement offloading, not to introduce test case for higher priority carrier frequency measurement. (CMCC vivo)

- In legacy test cases for cell re-selection, the higher priority search has been tested as below:

- Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

- We deem that the UE behaviour of measurement higher priority frequency shall be tested as legacy. The detailed test procedure could be reused as starting point.

- Proposal 9: RAN4 shall introduce test case for high priority frequency search in case 1.

- Proposal 10: The legacy test procedure on high priority frequency measurement shall be reused as starting point.

Topic 7 Test case design for entry/exit conditions for case1/case3

- The latest way forward could be seen as below:

- Proposals - Entry/exit condition evaluation verification - Option 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions (Apple CMCC vivo Huawei QC) - Option 2: Define test case for entry scenario (oppo Nokia) - Option 2-1 (oppo) - Scenario 1: UE enter Case 1 from legacy and exit to legacy - Scenario 2: UE enter Case 3 from legacy and exit to legacy - Scenario 3: UE enter Case 1 from Case 3 - Option 2-2: Use test mode / procedure for LR entry measurements (Nokia) - Option 3: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).(MTK) - Detail on exit condition evaluation verification: - P1: Define following tests for exit criteria evaluation (CMCC) - LR exit criteria evaluation test when exit from LP-WUS monitoring - LR exit criteria evaluation test when exit from Case 1 - FFS exit criteria evaluation test when exit from Case 3, wait the RAN2’s conclusion of exit condition - P2: Define test cases for evaluation of exit conditions for the following cases. (Huawei) - Case 1: UE exist Case 1 to legacy - Case 2: UE exist Case 3 to legacy - P3: Introduce the following test cases where the exit conditions can be tested (QC) - UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode - UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode. Recommendations: Suggest to agree the following based on majority view: only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions

- There are two issues to discuss: 1. Whether to define test case for entry conditions; 2: Details on exit or entry condition evaluation verification.

- For the first issue, we find that the majority of companies think even if UE satisfies the entry condition, it is up to UE to enter another modes or not. On the contrary, if UE satisfies the exit threshold, UE must exit the current states to another states.Thus, defining test cases for exit conditions is basic. However, to our understanding, we deem that the test cases for entry conditions shall also be defined. If network configures the entry threshold and UE would like to change the current state, UE has to ensure the measurement results is good enough to satisfy entry condition, and then switches to another state. That means we would like to define test cases for the sake of verifying the current requirements not only for entry evaluation condition but for exit evaluation condition. Consequently, we do think the tests cases for entry evaluation condition and exit evaluation condition shall be both considered.

- Proposal 11: RAN4 shall defines test cases for entry conditions and exit conditions.

- Proposal 12: For entry condition:

- Scenario 1: UE enters Case 3 from legacy state.

- Scenario 2: UE enters Case 1 from Case 3.

- Scenario 3: UE enters Case 3 from Case 1.

- Proposal 13: For exit condition:

- Scenario 1: UE exits Case 1 to Case 3 or to legacy state.

- Scenario 2: UE exits Case 3 to legacy state.

Topic 8 Test case design for MR wake up delay

- In LP-WUS monitoring, if the UE receives LP-WUS, MR will be woke up and perform legacy paging monitoring. In previous meeting, we confirm the wake up delay defined in RAN1. To our understanding, the test case defined for verifying MR wake up delay shall be considered.

- Proposal 14: RAN4 shall define test case to verify the MR wake up delay once UE receives LP-WUS.

Conclusion

In this paper we provided our views on RRM requirements on R19 LP-WUS:

- Proposal 1: For LR accuracy, it is no need to be defined in performance part.

- Proposal 2: For MR accuracy, the legacy accuracy requirements shall be reused.

- Proposal 3: The accuracy requirements for FR1 and FR2 shall be both considered.

Proposal 4: The whole test shall focus on time and delay behaviour of RRM offloading, RRM relaxation and LP-WUR monitoring modes.

Proposal 5: For whether to introduce new test mode,

- If new test mode is introduced, UE shall perform tests on LP-WUR monitoring at certain occasion.

- If not, UE shall be initialed in LP-WUR mode directly and perform test at the beginning of a test case.

Overall, the test case for LP-WUR monitoring shall be considered. RAN4 could further discuss the proper method to be utilized.

Proposal 6: Consider test case for the following scenarios:1: LP-WUR monitoring related test case; 2: RRM offlaoding related test case; 3. RRM relaxation related test case.

Proposal 7: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR.

- Proposal 8: For defining test cases on RRM relaxation, the following tests can be considered:

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

- Proposal 9: RAN4 shall introduce test case for high priority frequency search in case 1.

- Proposal 10: The legacy test procedure on high priority frequency measurement shall be reused as starting point.

- Proposal 11: RAN4 shall defines test cases for entry conditions and exit conditions.

- Proposal 12: For entry condition:

- Scenario 1: UE enters Case 3 from legacy state.

- Scenario 2: UE enters Case 1 from Case 3.

- Scenario 3: UE enters Case 3 from Case 1.

- Proposal 13: For exit condition:

- Scenario 1: UE exits Case 1 to Case 3 or to legacy state.

- Scenario 2: UE exits Case 3 to legacy state.

- Proposal 14: RAN4 shall define test case to verify the MR wake up delay once UE receives LP-WUS.

- 4 References

- TS38.133