3GPP TSG-RAN WG4 Meeting #116bis	R4-2514129

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	Simulation results for UE demod performance requirements for LPWUS

Agenda item:	6.16.4

Document for:	Information

Introduction

This contribution contains Nokia’s simulation results related to defining demodulation requirements for LPWUS.

Simulation results

Agreed simulation configuration

Table 1 contains the agreed configurations from RAN4#115 & RAN4#116 with addition of L=24 for 5 info bits.

Table 1: Agreed configuration for simulation alignment

| Parameter | Value | Comments |
| --- | --- | --- |
| Waveform FR1 | OOK-4 OFDM |  |
| Waveform FR2 | OOK-1 |  |
| Receiver types | OOK OFDM |  |
| Duplex modes | FDD TDD |  |
| Number of RBs | 11 |  |
| CBW/SCS | FR1: 10MHz/15kHz for FDD 40MHz/30kHz for TDD FR2: 100MHz/120kHz for TDD |  |
| Carrier frequency | Fc=900 for FR1 FDD Fc=2.6GHz for FR1 TDD Fc=TBD for FR2 TDD |  |
| Antenna configuration | 1T1R for FR1 [1T1R] for FR2 |  |
| Channel model | FR1: TDLA30-10 and TDLC300-100 FR2: TDLA30-10 |  |
| Timeoffset | FR1: 5us for OOK4 and OOK-1 2us for OOK-4 with M=2 1us for OOK-4 with M=4 FR2: [0.5] us for OOK-1 |  |
| Coding/decoding algorithms | Small block coding for 2 info bits RM coding for 5 info bits |  |
| Number of coded bits after rate-matching | [12, 16] for 2 info bits 24, 32 for 5 info bits |  |
| M for OFDM receiver | M=1 M=4 |  |

FR1 Results for Missed Detection Rate (MDR)

OOK Receiver

Table 2 and Table 3 provides an overview of simulation results for FDD and TDD using OOK4 receiver. The results cover both envelope and sequence detector.

Table 2 MDR for OOK receiver (FR1 - FDD). Results are provided in dB@1% MDR

| Parameters | Channel profile |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency |  | 900MHz |  |  |  |  |  |  |  |
| Modulation |  | OOK4 |  |  |  |  |  |  |  |
| SCS (kHz) |  | 15 |  |  |  |  |  |  |  |
| Infobits |  | 2 |  |  |  | 5 |  |  |  |
| Symbols |  | 12 |  | 16 |  | 24 |  | 32 |  |
| M |  | 1 | 4 | 1 | 4 | 1 | 4 | 1 | 4 |
| Time offset (us) |  | 5 | 1 | 5 | 1 | 5 | 1 | 5 | 1 |
| Freq. offset (ppm) |  | 10 |  |  |  |  |  |  |  |
| Envelope detector | AWGN | -7,1 | -7,2 | -7,9 | -7,3 | -7,7 | -7 | -8,5 | -7,8 |
|  | TDLC300-100 | -0,7 | -0,7 | -1,4 | -0,4 | -0,3 | 0,4 | -0,9 | -0,4 |
|  | TDLA30-10 | 4,2 | 3,9 | 3,3 | 3,9 | 5,2 | 5,7 | 4,4 | 5,2 |
| Sequence detector | AWGN | -7,1 | -9,8 | -7,6 | -9,4 | -7,7 | -9,7 | -8,2 | <-10 |
|  | TDLC300-100 | 1,5 | -1,9 | 1 | -1,3 | 2,2 | -0,7 | 1,5 | -1,2 |
|  | TDLA30-10 | 5,7 | 1,6 | 4,7 | 2,1 | 6,7 | 3,5 | 6,2 | 3,3 |

Table 3 MDR for OOK receiver (FR1 - TDD). Results are provided in dB@1% MDR

| Parameters | Channel profile |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency |  | 2.6GHz |  |  |  |  |  |  |  |
| Modulation |  | OOK4 |  |  |  |  |  |  |  |
| SCS (kHz) |  | 30 |  |  |  |  |  |  |  |
| Infobits |  | 2 |  |  |  | 5 |  |  |  |
| symbols |  | 12 |  | 16 |  | 24 |  | 32 |  |
| M |  | 1 | 4 | 1 | 4 | 1 | 4 | 1 | 4 |
| Time offset (us) |  | 5 | 1 | 5 | 1 | 5 | 1 | 5 | 1 |
| Freq. offset (ppm) |  | 10 |  |  |  |  |  |  |  |
| Envelope detector | AWGN | -6,5 | -6,6 | -7,5 | -6,6 | -7,2 | -6,3 | -7,9 | -7,2 |
|  | TDLC300-100 | -1,4 | -1,2 | -2,5 | -1,3 | -1,2 | -0,3 | -1,7 | -1,3 |
|  | TDLA30-10 | 2,9 | 2,6 | 2 | 3,3 | 3,5 | 4,5 | 2,6 | 3,4 |
| Sequence detector | AWGN | -6,7 | -8,8 | -7,1 | -8,8 | -6,7 | -8,8 | -7,1 | -9,4 |
|  | TDLC300-100 | 2,6 | -1,7 | 2,3 | -1,4 | 3,1 | -0,2 | 2,6 | -1 |
|  | TDLA30-10 | 6,5 | 1,3 | 6,2 | 1,2 | 7,3 | 3,1 | 7 | 2,4 |

We generally observe <1 dB in difference between M=1 and M=4 results.

For OOK we see a significant difference in SNR between AWGN and TDLC/TDLA channel models. In addition, there is also significant difference between TLDA and TDLC models, hence it is reasonable to define requirements for both TDLA30-10 and TDLC300-100.

For reference, we have included our simulation results for FDD (yellow highlighted values only) in Figure 1.

Figure 1 Full simulation results for FDD for yellow highlighted values

OFDM Receiver

Table 4 and Table 5 provides an overview of simulation results for FDD and TDD using OFDM receiver. We have only shown results for M=1 as we do not expect significant difference with results for M=4 in case of OFDM receiver.

Table 4 MDR for OFDM receiver (FR1 - FDD). Results are provided in dB@1% MDR

| Parameters | Channel profile |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency |  | 900MHz |  |  |  |
| Modulation |  | OFDM |  |  |  |
| SCS (kHz) |  | 15 |  |  |  |
| Infobits |  | 2 |  | 5 |  |
| symbols |  | 12 | 16 | 24 | 32 |
| M |  | 1 |  |  |  |
| Time offset (us) |  | 0 |  |  |  |
| Freq. offset (ppm) |  | 5 |  |  |  |
| OFDMA | AWGN | <-10 | <-10 | <-10 | <-10 |
|  | TDLC300-100 | -2,5 | -3,2 | -1,9 | -2,4 |
|  | TDLA30-10 | 1,8 | 1 | 2,7 | 2,4 |

Table 5 MDR for OFDM receiver (FR1 - TDD). Results are provided in dB@1% MDR

| Parameters | Channel profile |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency |  | 2.6GHz |  |  |  |
| Modulation |  | OFDM |  |  |  |
| SCS (kHz) |  | 30 |  |  |  |
| Infobits |  | 2 |  | 5 |  |
| symbols |  | 12 | 16 | 24 | 32 |
| M |  | 1 |  |  |  |
| Time offset (us) |  | 0 |  |  |  |
| Freq. offset (ppm) |  | 5 |  |  |  |
| OFDMA | AWGN | -9,1 | -9,7 | -9,7 | -10 |
|  | TDLC300-100 | -2,6 | -3,2 | -2,3 | -3,2 |
|  | TDLA30-10 | 1,6 | 1,1 | 2,7 | 2 |

For OFDM we see a significant difference in SNR between AWGN and TDLC/TDLA channel models. In addition, there is also significant difference between TLDA and TDLC models, hence it is reasonable to define requirements for both TDLA30-10 and TDLC300-100.

FR2 Results for Missed Detection Rate (MDR)

OOK receiver

Table 6 provides an overview of simulation results for FR2 using OOK1 receiver. No results are shown for sequence detector as it is not available for FR2.

Table 6 MDR for OOK receiver (FR2). Results are provided in dB@1% MDR

| Parameters | Channel profile |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency |  | 28GHz |  |  |  |
| Modulation |  | OOK1 |  |  |  |
| SCS (kHz) |  | 120 |  |  |  |
| Infobits |  | 2 |  | 5 |  |
| symbols |  | 12 | 16 | 24 | 32 |
| M |  | 1 |  |  |  |
| Time offset (us) |  | 0,5 |  |  |  |
| Freq. offset (ppm) |  | 1 |  |  |  |
| Envelope detector | AWGN | -4,4 | -4,4 | -2 | -2 |
|  | TDLC300-100 | -0,5 | -0,5 | 2,2 | 2,2 |
|  | TDLA30-10 | 4 | 4 | 6,6 | 6,6 |
| Sequence detector | AWGN | NA | NA | NA | NA |
|  | TDLC300-100 | NA | NA | NA | NA |
|  | TDLA30-10 | NA | NA | NA | NA |

As previous agreement is to use TDLA30-10 and since we see reasonable SNR values, we see it feasible to define requirements for FR2 using TDLA30-10. We observe that there is no difference in SNR between number of symbols for the same number of info bits.

Conclusion

The following Observations were made:

Observation 1: We generally observe <1 dB in difference between M=1 and M=4 results.

Observation 2: For OOK we see a significant difference in SNR between AWGN and TDLC/TDLA channel models. In addition, there is also significant difference between TLDA and TDLC models, hence it is reasonable to define requirements for both TDLA30-10 and TDLC300-100.

Observation 3: For OFDM we see a significant difference in SNR between AWGN and TDLC/TDLA channel models. In addition, there is also significant difference between TLDA and TDLC models, hence it is reasonable to define requirements for both TDLA30-10 and TDLC300-100.

Observation 4: As previous agreement is to use TDLA30-10 and since we see reasonable SNR values, we see it feasible to define requirements for FR2 using TDLA30-10. We observe that there is no difference in SNR between number of symbols for the same number of info bits.

- References

- R4-2512526_WF for [116][330] NR_LPWUS_Demod