# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514143

# Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Discussion on Rel-19 ATG demodulation requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.7.4

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, the revised WID of enhancements for Air-to-ground network for NR was approved in [1], the objectives related to enhanced ATG are listed:

The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

In Rel-18, RAN4 defined demodulation requirements for PDSCH under AWGN channel model on FR1 single scenario. However, considering that ATG has the advantage of high throughput, low propagation delay, and low cost application, some operators and the aircraft industry have a strong request for the deployment of ATG. In R19, the following two aspects are considered:

- Enhance throughput to meet ATG capacity demand

- Fully utilize operators’ spectrum especially for the fragment spectrum

In this contribution, we give some initial discussion on the enhanced ATG demodulation requirements according to the last meeting agreements [2].

# 2 Discussion

DL CA requirements

TDD pattern

- Option 1: Use the legacy TDD pattern 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U.

In Rel-18 for ATG demodulation requirements, RAN4 defined two kinds of TDD pattern for UE side requirements, e.g., 7D1S2U and 30D4S6U TDD patterns. From the definition of CA demod requirements based on legacy rules, we think that define 30D4S6U requirements for baseband are reasonable.

Proposal 1. Propose to define TDD pattern 30D4S6U for R-19 enhanced ATG DL CA demodulation requirements.

Regarding how to define new TDD pattern requirements, currently according to Rel-18 results, different TDD patterns have the same requirements, which is based on normalization maximum throughput. From our understanding, RAN4 needs to define separate demodulation requirements for different TDD patterns, because different FRCs are introduced for different patterns caused by different maximum throughput.

Observation 1. Different maximum throughput brings Different FRCs for different TDD patterns.

Observation 2. The SNR operation point of 70% maximum throughput can be reused for different TDD patterns.

Proposal 2. Propose to define two separate demodulation requirements for different TDD patterns.

According to Rel-18 definition, only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes that UE can support 30D4S6U pattern. Therefore, for Rel-19 DL CA requirements, we think that the applicable rule is also needed for different patterns.

Proposal 3. Propose to define following applicable rule for Rel-19 CA demodulation requirements.

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 TDD | PDSCH | Tests listed in 5.2A.2.x | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
|  |  |  | Tests listed in 5.2A.2.x | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
| ATG UE with 4RX | FR1 TDD | PDSCH | Tests listed in 5.2A.3.x | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
|  |  |  | Tests listed in 5.2A.3.x | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |

On the other side, regarding HARQ process and K1 values, we have the following detailed analysis for different CA combinations based on legacy definition.

- FDD 15kHz Pcell + TDD 30kHz Scell

Based on our analysis, the number of HARQ process and K1 values could be 4 and {2} for FDD 15kHz PCell, and 13 and {2} for TDD 30kHz Scell.

- TDD 30kHz Pcell + FDD 15kHz Scell

Regarding TDD 30kHz Pcell + FDD 15kHz Scell, HARQ feedback of Scell should be transmitted on the UL slot of the Pcell. However, the HARQ feedback of TDD pattern has totally used 8 K1 values. Considering the restrictions of RAN1 specification on K1 value, the HARQ feedback of Scell also should be based on those values. However, according to our analysis, the HARQ feedback of Scell based on those values can’t meet the requirement of providing HARQ feedback for all FDD slots every 20ms. Therefore, one potential approach is to puncture some FDD slots to meet the HARQ values, but the total maximum throughput should be reviewed for FDD scenario, which means that RAN4 needs to redefine a new FRC table for FDD under TDD 30kHz Pcell + FDD 15kHz Scell CA scenario. However, compared to FDD 15kHz Pcell + TDD 30kHz Scell scenario, which is very different the legacy definition approach that one set of requirements can applicable for all CA test cases. Bases on above reason, we propose to do not define the TDD 30kHz Pcell + FDD 15kHz Scell test case for ATG CA requirements.

Observation 3. Based on our analysis, the HARQ feedback of Scell based on those values can’t meet the requirement of providing HARQ feedback for all FDD slots every 20m.

Proposal 4. Propose to do not define the TDD 30kHz Pcell + FDD 15kHz Scell test case for Rel-19 ATG CA requirements.

- TDD 30kHz Pcell + TDD 30kHz Scell

For TDD 30kHz Pcell + TDD 30kHz Scell case, from our analysis, the number of HARQ process and K1 values could be 13 and {34,30,26,22,18,14,11,10} for TDD 30kHz Pcell and TDD 30kHz Scell.

Therefore, we propose to define the TDD 30kHz Pcell + TDD 30kHz Scell and FDD 15kHz Pcell + TDD 30kHz Scell test cases for Rel-19 ATG CA requirements. And the number of HARFQ process, k1 values in the following tables.

Test parameters for number of HARQ processes for ATG CA requirements

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 13 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 13 | N/A |

Test parameters for K1 values for ATG CA requirements

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 | N/A |

Proposal 5. Propose to define the TDD 30kHz Pcell + TDD 30kHz Scell and FDD 15kHz Pcell + TDD 30kHz Scell test cases for Rel-19 ATG CA requirements.

Proposal 6. Propose to consider following the number of HARQ process and K1 values for CA test cases.

Test parameters for number of HARQ processes for ATG CA requirements

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 13 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 13 | N/A |

Test parameters for K1 values for ATG CA requirements

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 | N/A |

CA configuration selection rules

- Option 1: Define the CA configuration selection rules as CA capability; Step 1; Step 2; Step 3; Step 4. CA_C; Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1.; N/A; N/A. CA_AX; Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1.; Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3..

Based on the agreement from the last meeting, CA configuration selection rules are defined for CA process selection. According to the legacy approach, we think option 1 could be accepted.

Proposal 7. Propose to consider the following CA configuration selection rules.

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Test applicability rules

- Option 1: Define the test rules as Tests; CA capability where the tests apply; CA configuration from the selected CA capability where the tests apply; CA Bandwidth combination to be tested in priority order; PCell CC configuration. Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz）; CA_C, CA_AX; Table TBD; Largest aggregated CA bandwidth combination; Any of CCs. Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz); CA_AX; Table TBD; Largest aggregated CA bandwidth combination; FDD CC if supported, otherwise TDD CC.

Currently, RAN4 only defined TDD 30 kHz + TDD 30 kHz and FDD 15 kHz + TDD 30 kHz test cases for CA requirements. However, based on our previous analysis, TDD 30kHz Pcell + FDD 15kHz Scell test case can’t be supported caused by HARQ feedback. Therefore, we prefer to consider the following applicable and test rules.

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Propose 8. Propose to consider the following applicable and test rules

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

CA requirements

Regarding DL CA requirements, based on the above analysis and the previous meeting conclusion, we propose the following minimum performance for CA configuration.

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 30 kHz + TDD 30 kHz | As defined in Table x |
| 2 | FDD 15 kHz + TDD 30 kHz | As defined in Table x |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

Proposal 9. Propose to define the following CA minimum performance for Rel-19 ATG.

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 30 kHz + TDD 30 kHz | As defined in Table x |
| 2 | FDD 15 kHz + TDD 30 kHz | As defined in Table x |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

DL MIMO requirements

TDD pattern

- Option 1: Use the legacy TDD pattern 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U.

Currently, two TDD pattern demodulation requirements are defined in Rel-18. There is reasonable to define two requirements for different patterns. Based on previous discussion, the SNR point of new TDD pattern could be the same as the legacy TDD pattern, the only different is the FRC table, different maximum throughput for different TDD patterns.

Proposal 10. Propose to define TDD pattern 30D4S6U demodulation requirements for DL MIMO.

MCS

- Agreement: - 64QAM MCS 22. - FFS for 256QAM Table 2 MCS 24.

We evaluated 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 with 2T2R, the SNR point of 70% maximum throughput and curves can be found in the following. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher. Therefore, we prefer to consider MCS 13 or MCS 22 to define requirements.

Table 1. Simulation results for 2T2R Rank 2

| Propagation condition | Antenna configuration | MCS | Layer | SNR(70% Max TP) |
| --- | --- | --- | --- | --- |
| AWGN+220Hz | 2T2R | 16QAM Table 1 MCS13 | Rank 2 | 7.41 |
|  |  | 64QAM Table 1 MCS 22 | Rank 2 | 14.87 |
|  |  | 256QAM Table 2 MCS 24 | Rank 2 | 22.76 |

Table 2.  Throughput curves for 2T2R Rank 2

| Fig 1. FDD, MCS13, Rank 1 and Rank 2 | Fig 2. FDD, MCS22, Rank 1 and Rank 2 |
| --- | --- |
| Fig 3. FDD, MCS24, Rank 1 and Rank 2 |  |

Observation 4. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher.

Proposal 11. Propose to consider MCS 13 or MCS 22 defining demodulation requirements.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of ATG UE demodulation requirements, the conclusions are:

Observation 1. Different maximum throughput brings Different FRCs for different TDD patterns.

Observation 2. The SNR operation point of 70% maximum throughput can be reused for different TDD patterns.

Observation 3. Based on our analysis, the HARQ feedback of Scell based on those values can’t meet the requirement of providing HARQ feedback for all FDD slots every 20m.

Observation 4. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher.

Proposal 1. Propose to define TDD pattern 30D4S6U for R-19 enhanced ATG DL CA demodulation requirements.

Proposal 2. Propose to define two separate demodulation requirements for different TDD patterns.

Proposal 3. Propose to define following applicable rule for Rel-19 CA demodulation requirements.

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 TDD | PDSCH | Tests listed in 5.2A.2.x | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
|  |  |  | Tests listed in 5.2A.2.x | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
| ATG UE with 4RX | FR1 TDD | PDSCH | Tests listed in 5.2A.3.x | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |
|  |  |  | Tests listed in 5.2A.3.x | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |

Proposal 4. Propose to do not define the TDD 30kHz Pcell + FDD 15kHz Scell test case for Rel-19 ATG CA requirements.

Proposal 5. Propose to define the TDD 30kHz Pcell + TDD 30kHz Scell and FDD 15kHz Pcell + TDD 30kHz Scell test cases for Rel-19 ATG CA requirements.

Proposal 6. Propose to consider following the number of HARQ process and K1 values for CA test cases.

Test parameters for number of HARQ processes for ATG CA requirements

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 13 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 13 | N/A |

Test parameters for K1 values for ATG CA requirements

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 | N/A |

Proposal 7. Propose to consider the following CA configuration selection rules.

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Propose 8. Propose to consider the following applicable and test rules

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 9. Propose to define the following CA minimum performance for Rel-19 ATG.

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 30 kHz + TDD 30 kHz | As defined in Table x |
| 2 | FDD 15 kHz + TDD 30 kHz | As defined in Table x |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

Proposal 9. Propose to define TDD pattern 30D4S6U demodulation requirements for DL MIMO.

Proposal 10. Propose to consider MCS 13 or MCS 22 defining demodulation requirements.

# 4	Reference

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR. CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom.

- R4-2512602, Way Forward for [116][326] NR_ATG_enh_demod, ZTE Corporation, CMCC.