# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514144

# Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Simulation results on Rel-19 ATG

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.7.4

# Document for:	Discussion

# 1 Introduction

Based on the agreed simulation assumption approved in previous meetings [1], we give our simulation results and observations.

# 2 Discussion

2.1 Simulation assumptions and results for DL CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD TDD: 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U |
| Subcarrier spacing |  |  | FDD: 15kHz TDD: 30kHz |
| CHBW |  |  | FDD:5,10,15,20,25,30,40,50MHz TDD: 5,10,15,20,25,30,40,50,60,80,90,100MHz |
| MCS |  |  | 64QAM MCS22 |
| Antenna configuration |  |  | 2T2R and 2T4R |
| Propagation condition |  |  | FDD: AWGN+500Hz TDD: AWGN+220Hz |
| Rank |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Receiver type |  |  | MMSE-IRC |

The SNR corresponding to 70% maximum throughput is shown in the following.

| Test number | Duplex model and SCS | Propagation condition | MCS | Antenna configuration | CHBW | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz | AWGN+ 220Hz | 64QAM MCS22 | 2T2R | 5M | 10.4 |
| 2 |  |  |  |  | 10M | 10.4 |
| 3 |  |  |  |  | 15M | 10.4 |
| 4 |  |  |  |  | 20M | 10.4 |
| 5 |  |  |  |  | 25M | 10.4 |
| 6 |  |  |  |  | 30M | 10.4 |
| 7 |  |  |  |  | 40M | 10.4 |
| 8 |  |  |  |  | 50M | 10.4 |
| 9 | FDD 15kHz | AWGN+ 220Hz | 64QAM MCS22 | 2T4R | 5M | 7.8 |
| 10 |  |  |  |  | 10M | 7.9 |
| 11 |  |  |  |  | 15M | 7.8 |
| 12 |  |  |  |  | 20M | 7.8 |
| 13 |  |  |  |  | 25M | 7.8 |
| 14 |  |  |  |  | 30M | 7.8 |
| 15 |  |  |  |  | 40M | 7.8 |
| 16 |  |  |  |  | 50M | 7.8 |
| 17 | TDD 30kHz | AWGN+ 500Hz | 64QAM MCS22 | 2T2R | 5M | 10.8 |
| 18 |  |  |  |  | 10M | 10.8 |
| 19 |  |  |  |  | 15M | 10.8 |
| 20 |  |  |  |  | 20M | 10.8 |
| 21 |  |  |  |  | 25M | 10.4 |
| 22 |  |  |  |  | 30M | 10.8 |
| 23 |  |  |  |  | 40M | 10.4 |
| 24 |  |  |  |  | 50M | 10.8 |
| 25 |  |  |  |  | 60M | 10.8 |
| 26 |  |  |  |  | 80M | 10.8 |
| 27 |  |  |  |  | 90M | 10.4 |
| 28 |  |  |  |  | 100M | 10.4 |
| 29 | TDD 30kHz | AWGN+ 500Hz | 64QAM MCS22 | 2T4R | 5M | 7.8 |
| 30 |  |  |  |  | 10M | 7.8 |
| 31 |  |  |  |  | 15M | 7.8 |
| 32 |  |  |  |  | 20M | 7.8 |
| 33 |  |  |  |  | 25M | 7.8 |
| 34 |  |  |  |  | 30M | 7.8 |
| 35 |  |  |  |  | 40M | 7.8 |
| 36 |  |  |  |  | 50M | 7.8 |
| 37 |  |  |  |  | 60M | 7.8 |
| 38 |  |  |  |  | 80M | 7.8 |
| 39 |  |  |  |  | 90M | 7.8 |
| 40 |  |  |  |  | 100M | 7.8 |

2.2 Simulation assumptions and results for DL MIMO

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD TDD: 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U |
| Subcarrier spacing |  |  | FDD: 15kHz TDD: 30kHz |
| CHBW |  |  | FDD:10MHz TDD:40MHz |
| MCS |  |  | 64QAM MCS 22 FFS for 256QAM |
| Antenna configuration |  |  | 2T2R and 2T4R with XP-low |
| Propagation condition |  |  | FDD: AWGN+500Hz TDD: AWGN+220Hz |
| Rank |  |  | 2 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | DMRS port |  | {1000},{1001} |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Receiver type |  |  | MMSE-IRC |

| Test number | Duplex mode, SCS and CBW | Propagation condition | MCS | Antenna configuration | CHBW | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz | AWGN+ 220Hz | 64QAM MCS22 | 2T2R | 10M | 13.8 |
| 2 |  |  |  | 2T4R | 10M | 10.8 |
| 3 | TDD 30kHz | AWGN+ 500Hz | 64QAM MCS22 | 2T2R | 40M | 13.8 |
| 4 |  |  |  | 2T4R | 40M | 10.8 |

2.3 Simulation assumptions and results for UL MIMO

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15kHz SCS: N/A 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| CHBW |  | FDD: 5MHz TDD:10MHz |
| MCS |  | 64QAM Table 1 MCS 20 |
| Antenna configuration |  | 2T2R with X-pol low correlation |
| Propagation conditions |  | FDD 15kHz SCS: H= 1j1-j+200Hz TDD 30kHz SCS: H= 1j1-j+500Hz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0},{1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL pattern, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

| Test number | Duplex mode, SCS and CBW | Propagation condition | MCS | Antenna configuration | CHBW | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz | AWGN+ 200Hz | 64QAM MCS20 | 2T2R | 5M | 11.8 |
| 2 | TDD 30kHz | AWGN+ 500Hz | 64QAM MCS20 | 2T2R | 10M | 11.8 |

# 3 Conclusion

In this contribution, we provided the simulation results for Rel-19 ATG enhancement demodulation performance, including DL CA, DL MIMO, and UL MIMO.

# 4 References

- R4-2512602 Way Forward for [116][326] NR_ATG_enh_demod, ZTE Corporation, CMCC.