3GPP TSG-RAN4 Meeting #116-bis	R4-2514160

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on core requirements for Ku band NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability of RRM requirements for VSAT UE in FR1-NTN is captured in clause 3.6. However, in clause 9 there are similar applicability specified for measurement requirements. Defining applicability for individual requirements is redundant and it cause the confusion for interpreting applicability of other individual requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the requirement applicability specified in clause 9 for measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are confusion for interpreting applicability of other requirements than measurement requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2C.7, 9.2C.8, 9.3C.8, 9.3C.9, 9.3C.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 9.2C.7	Intra-frequency measurements without measurement gaps for NTN band above 10 GHz

<Texts without change are omitted>

<End of Change 1>

<Start of Change 2>

### 9.2C.8	Intra-frequency measurements with measurement gaps for NTN band above 10 GHz

<Texts without change are omitted>

<End of Change 2>

<Start of Change 3>

### 9.3C.8	Inter-frequency measurement with measurement gaps for NTN band above 10 GHz

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3C.8-1.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3C.8-2.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3C.9-1.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. When the UE is configured with concurrent measurement gaps and the two measurement gaps are fully overlapping with MGRP=160 ms, Kgap = 2. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in clause 9.1.8.3.

Kgap is only applicable for UE supporting parallelMeasurementGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0, or if one SMTC overlaps more than one MGs associated to the frequency layer.

Table 9.3C.8-1: Time period for PSS/SSS detection

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(8*1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 x Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

Table 9.3C.8-2: Time period for time index detection

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, Ceil(3 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(3 x Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

<End of Change 3>

<Start of Change 4>

### 9.3C.9	Inter-frequency measurements for NTN band above 10 GHz

When measurement gaps are provided for inter-frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1.5C, 10.1.10C and 10.1.15C, respectively, as shown in table 9.3C.5-1.

Table 9.3C.9-1: Measurement period for inter-frequency measurements with gaps

| Condition NOTE1 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(8 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(8 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 x Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

<End of Change 4>

<Start of Change 5>

### 9.3C.10	Inter-frequency measurements without measurement gaps for NTN band above 10 GHz

<Texts without change are omitted>

<End of Change 5>