3GPP TSG RAN WG4 Meeting #116	bis 																R4-2514299

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.12.2.1

Source: 	Ericsson

Title:	On RRM performance aspects and test cases for MIMO Ph 5

Document for:	Discussion

# Introduction

In last meeting WF, following WF is agreed.

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

In this contribution, we provide our views on the test cases and configuration.

# Discussion

## UE-initiated/event-driven beam management

RAN4 discussed the following aspects as part of the core requirements design:

- Measurement delay

- Measurement accuracy

- TCI state switching

We discuss the performance requirements for the above aspects.

Measurement delay:

RAN1 defined three events: Event-1, Event-2, and Event-7.

- Event-1: The quality of the current beam is worse than a certain threshold.

- Event-2: The quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: The quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured and subject to UE capability signalling. The UE may only indicate a single candidate value or not support Event-7.

RAN4 discussed the requirements for Event-2 with the assumption that the same framework would be extended to other events. From our perspective, Event-2 and Event-7 are the important events as they resemble the existing TCI state switching framework from the legacy. We believe that at least these events should be tested.

- RAN4 to define the test cases for Event-2 and Event-7

Furthermore, RAN1 defined two different reporting frameworks called Mode-A and Mode-B. At least Mode-A should be tested as this mode is a basic UE capability (i.e., all UEs supporting UE-initiated/event-driven beam reporting should support this feature). The descriptions of Mode-A and Mode-B from the RAN1 agreements are as follows:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit) to request a resource for a second UL channel to carry the beam report.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry the beam report.

  - Step 3: Beam report is transmitted in the second UL channel.

  - This mode is a basic UE capability (i.e., all UEs supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for the second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry the beam report.

  - Step 2: UE transmits the beam report in the second UL channel.

  - The notification in Step 1 is in a separate reporting instance from the beam report in Step 2.

RAN4 to consider at least Mode-A in the test case design.

Time window based:

Based on the RAN1 agreement, we believe its functionality is similar to that of the TTT timer, with one key difference: events do not need to persist throughout the entire window duration but must occur M number of times. In the L3 reporting, the UE can assess the event at the end of the TTT to determine if the event criteria have been still met and trigger the report.

In the UEI BM context, the UE must evaluate the event N number of times to confirm that it occurs M number of times. This functionality appears more stringent than the TTT for L3, where the UE can simply check at the end of the TTT to see if the event condition remains satisfied. Therefore, we suggest defining RAN4 testing to ensure that UE measurement behaviour is tested accurately.

- Define test cases with time window-based and non-time window-based configurations.

Test configuration:

For Event-2, we have the following definitions for Scheme-1 and Scheme-2:

- Scheme-1: RS for the current beam is the QCL RS in the indicated TCI state.

- Scheme-2: RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Generally, the QCL RS in the indicated TCI state could be TRS, and the quality of the TRS may not be fair to compare with the quality of the new beam (which could be SSB). Hence, we believe that at least Scheme-2 should be considered for SSB based event triggered reporting testing. Once the scheme for Event-2 is agreed upon, the scheme for Event-7 can follow the same principle. This is further supported by RAN1 agreement shown below.

- Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

Another important test configuration is the PUCCH configuration for the new UCI. One relevant agreement regarding the first PUCCH from RAN1 is listed below. Among the first PUCCH configurations, we believe that at least periodicityAndOffset should be specified in the test configuration. This ensures that the UE can send the indication to the NW node when the event-triggered condition is met during the first PUCCH occasion.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline FFS: overlapping with SR/LRR or PUSCH

- RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset).

Measurement accuracy:

No filtering is agreed upon, and hence we expect the measurement accuracy to be the same as the legacy. We do not see a need for testing measurement accuracy or introducing new accuracy requirements.

Unified TCI state switching delay:

As discussed in the core part, we do not see MAC CE-based requirements changing, and we do not see a need for testing. For DCI-based TCI state switching, we understand that RAN1 is discussing enhancements for delay reduction in the case of DCI-based TCI state switching. We can revisit this after RAN1 reaches an agreement.

- No MAC CE-based TCI state switching test case is needed with event-driven reporting.

In the context of event-triggered L1-RSRP reporting, for M>1 case, 3GPP RAN1 has introduced a new reporting quantity: the Condition Met Indicator (CMI). This 1-bit indicator is designed to provide an efficient and reliable mechanism for the UE to inform the gNodeB about whether a specific beam has met the event-triggering conditions within a defined time window. The interpretation of the CMI is as follows:

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

As Xiaomi points out CMI is new reporting quantity, and it would be good to include in the test configuration.

- Consider CMI as part of the test reporting configuration.

# Summary

The following have been proposed in this contribution.

- RAN4 to define the test cases for Event-2 and Event-7

- Define test cases with time window-based and non-time window-based configurations.

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

- RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset).

- No MAC CE-based TCI state switching test case is needed with event-driven reporting.

- Consider CMI as part of the test reporting configuration.

# References

[1] 		xxx