3GPP TSG-RAN WG4 Meeting #116bis	R4-2514351

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	6.11.3

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. In this paper, we address the performance monitoring and life-cycle management requirements including reporting delay for AI/ML-based CSI prediction use cases.

Discussion

Performance monitoring

Reporting Requirements for Performance monitoring

The WI on AI/ML-based CSI prediction use case is specifying the reporting of performance. First, the delay and accuracy of performance monitoring metric is discussed in RAN4#115 [1].

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

Also, RAN4#116 meeting has made the agreements on the performance monitoring [2].

Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

An open issue is whether it is feasible to define and test a reporting accuracy requirement for performance monitoring in AI/ML-based CSI prediction. A potential test framework can be specified in RAN4, where the UE calculates and reports SGCS values under static and controlled radio conditions. Since the environment remains unchanged, any variation in reported values indicates potential inaccuracies in the UE’s reporting mechanism.

By evaluating the consistency of SGCS reports across multiple monitoring instances, the test equipment (TE) can determine whether the UE reliably reports the performance monitoring metric. This approach avoids the need for ground-truth measurements or dynamic environmental changes and still provides a valid basis for defining accuracy requirements. Although this framework differs from conventional RAN4 test designs, it is suitable for verifying the stability of UE-reported metrics. Under static conditions, reported values should remain stable, and significant deviations may signal issues in the AI/ML-based prediction or reporting function.

Since RAN1 has already defined the performance monitoring mechanism and SGCS reporting structure, RAN4 must define corresponding test procedures to ensure the reliability of UE-reported metrics. Without such testing, performance monitoring reports used for AI/ML life-cycle management may be unreliable, potentially leading to degraded network performance. RAN4 can define an accuracy requirement using a statistical consistency-based approach. For example, it may require that in at least 90% of test iterations, reported SGCS values remain within a defined tolerance range under static conditions. This ensures the reported values are stable and suitable for performance monitoring.

Observation: RAN4 can define reporting accuracy requirements for AI/ML-based CSI prediction by using a test framework that monitors SGCS consistency under static radio conditions.

Proposal: RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- RAN4 can define reporting accuracy requirements for AI/ML-based CSI prediction by using a test framework that monitors SGCS consistency under static radio conditions.

- RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment.

- References

- R4-2508080, “WF on AI/ML air interface part 1,” RAN4#115, Malta.

- R4-2511890, “WF on AI/ML air interface for [116][131],” RAN4#116, Bengaluru, India.