3GPP TSG-RAN WG4 Meeting #116bis	R4-2514375

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Nokia

Title:	Discussion on remaining issues on Rel-19 LP-WUS core requirements

Agenda item:	4.22.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, most of the core part issues critical to the closure of the core part were agreed. However, some of the issues related to LP-WUS / WUR operation remains unclear. This contribution addresses the remaining issues for the core part.

Discussion

RAN2 entry definition agreement

In RAN2#131 the following definition for entry was agreed:

<RAN2 agreement>: The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

The above means that there is no configuration for relaxation exit threshold from the network side.

Figure 1: Only entry threshold is configured based on RAN2 agreement for Relaxation.

The above only applies for relaxation scenario, for MR offloading RAN2 has both entry and exit thresholds. RAN4 has had the working assumption that there are both entry and exit thresholds configured for MR relaxation. Hence, the RAN4 has specified the following:

The UE shall evaluate and consider an entry or exit criteria is fulfilled within Tevaluate-LP-WUR-PSS/SSS, provided that the corresponding criteria is met by a margin of 6 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR1 and by a margin of 7.5 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR2 when SSB Ês/Iot ≥ -3dB

Discuss how not configuring MR relaxation exit threshold impacts RAN4 requirements.

  - LP-WUR and SDT

Related to SDT, RAN2 agreed the following in the last meeting:

RAN2 agreement: - Confirm that SDT can be initiated while UE is monitoring LP-WUS, and there is no impact to the SDT procedure. Can check if any spec change is needed.

Small data transmission (SDT) was specified in R17 and R18. SDT is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT can be initiated either by the UE or by the network. MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. SDT was specified for UE power saving purposes like LP-WUS and therefore it can be assumed that they should work together.

The following is capture in the TS 38.304 running CR:

5.2.4.x.3 Serving cell measurement offloading rules LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement is fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

If the UE evaluates conditions for SDT initiation when LP-WUS UE is performing serving cell measurement offloading, then MR RSRP measurement is not available since measurements are performed by LR. It needs to be decided whether LP-WUS UE can use RSRP measurement for SDT condition evaluation in this case. Also, MR RSRP measurement can be considered for this case, but using it causes delay (2 times DRX cycle), since RSRP needs to be measured according to RAN4 requirements.

SDT requirements apply also for UE supporting LP-WUS

The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading (i.e., no MR results available).

LP-WUR and Redcap

Issue 1-1-15 LP-WUR operation with RedCap

RAN4 116 agreement: Specify LP-WUR related idle/inactive requirements including requirement on serving cell offloading, RRM relaxation and higher priority frequency layer search for Redcap UE. - Existing requirements for MR offloading, RRM relaxation and higher priority frequency layer search will be reused for Redcap UE - Confirm the MR wake up delay will apply for 2 Rx Redcap

In the last meeting support for RedCap with LP-WUS was agreed. What was left open was the wake-up delay for 1Rx RedCap. The wake-up delay is the time difference between LP-WUS and network guaranteed PO. Currently the UE can select with capability signalling one value from three different values, namely 10ms, 400ms and 800ms. The RAN4 delay is currently baked in into the RAN1 capability values meaning that the number of samples that RAN4 assumes is not visible in the delay other than as an extra time. Furthermore, there are no RAN4 requirements defined for the wake up and first PO. Defining new values for 1Rx Redcap would require that RAN1 would need to define a new value for 1Rx RedCap.

1Rx RedCap UE can have 800ms wake up time and take more samples based on implementation if it needs to.

Differentiating wake-up delay between 1Rx RedCap and 2Rx RedCap would mean that RAN1 would need to add a separate capability for 1Rx RedCap wake-up delay.

If the delay is to be differentiated, the clearest option is to define RAN4 part in 38.133 for this and refer in the RAN1 capability to RAN4 defined values.

Use the same wake-up delay for 1Rx RedCap, 2Rx Redcap and legacy devices. Leave any measurement optimisations up to UE implementation: 800ms wake-up delay with 20ms SSB periodicity assumption is sufficient for this.

  - If the wake-up delay is to be differentiated for 1Rx RedCap, define RAN4 delay separately from RAN1 delay and send LS to RAN1 to request to remove the RAN4 part (I.e,. number of SSBs) from RAN1 delay.

Issue 1-1-16: LP-WUR operation with EMR

In the last meeting, it was agreed that when T331 is running, UE shall not use LP-WUS if R16 EMR and LP-WUS are configured simultaneously.

There was also online discussion and concerns about EMR R18 behaviour. The following highlights the relevant section in 38.133 for EMR R18 feature:

4.7.3 Measurement Report Requirements A UE shall perform validity check and report valid measurement results: - if the UE supports measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, - if the UE supports measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

The following figure illustrates the highlighted cases. If ValidityDuration is configured, it is performed retrospectively. If the validityDuration is not configured, UE may still report.

Figure 2: EMR R-18 operation, UE checks measurements retrospectively

If the UE has been woken up with LP-WUS, at the MSG-1 there may not be measurements. For this case RAN4 may define that if the is woken up by the LP-WUS signal, it may skip the validityCheck.

For Rel-18 eEMR if the UE has been woken up by LP-WUS from fully offloading scenario, UE may skip the validityCheck for the configured carriers (measIdleCarrierListNR or measReselectionCarrierListNR).

MR wake up delay after exiting

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met

Related to this issue, the following agreement was made:

RAN4#116 Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

What was left unclear is whether the same wake-up delay applies when the UE is exiting MR relaxation, for instance due to LR or MR measurement-based exit. Since the DRX cycle of the MR can be quite long, and the relaxation factor is very large, the MR may “go to sleep state” in between the MR measurements. Therefore it should be ok to allow the UE to have the same MR wake-up delay even when coming out from the relaxation.

For requirements point of view, it should be clearest if after the UE wakes up MR for measurement-based exit, the requirements only use a single value for wake-up. This should simplify the RAN4 requirements in this case.

Since this is a new procedure and not following the legacy measurements in MR we should expect MR to measure during this time with SSB periodicity.

Use MR wake up delay after exiting relaxation based on MR or LR exit.

  - Select a single value for wake-up time from 10ms, 100ms or 800ms.

  - During the wake-up time UE uses SSB periodicity for the measurements

Conclusion

The following Observations and Proposals were made:

Proposal 1: Discuss how not configuring MR relaxation exit threshold impacts RAN4 requirements.

Proposal 2: SDT requirements apply also for UE supporting LP-WUS

Proposal 3: The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading (i.e., no MR results available).

Observation 1: 1Rx RedCap UE can have 800ms wake up time and take more samples based on implementation if it needs to.

Observation 2: Differentiating wake-up delay between 1Rx RedCap and 2Rx RedCap would mean that RAN1 would need to add a separate capability for 1Rx RedCap wake-up delay.

Proposal 4: Use the same wake-up delay for 1Rx RedCap, 2Rx Redcap and legacy devices. Leave any measurement optimisations up to UE implementation: 800ms wake-up delay with 20ms SSB periodicity assumption is sufficient for this.

a.	If the wake-up delay is to be differentiated for 1Rx RedCap, define RAN4 delay separately from RAN1 delay and send LS to RAN1 to request to remove the RAN4 part (I.e,. number of SSBs) from RAN1 delay.

Proposal 5: For Rel-18 eEMR if the UE has been woken up by LP-WUS from fully offloading scenario, UE may skip the validityCheck for the configured carriers (measIdleCarrierListNR or measReselectionCarrierListNR).

Proposal 6: Use MR wake up delay after exiting relaxation based on MR or LR exit.

-	Select a single value for wake-up time from 10ms, 100ms or 800ms.

-	During the wake-up time UE uses SSB periodicity for the measurements