3GPP TSG-RAN WG4 Meeting #116bis	R4-2514382

# Prague, Czech Republic, 13th – 17th October 2025

Agenda item:		4.25.1

Source: 		Qualcomm Inc.

Title:	 	NTN (e)RedCap release independence

Document for:		Approval

## 1	Introduction

The core part for the work item for NR NTN Phase 3 was completed in previous meeting. While new requirements were included, changes to release independence were forgotten. This contribution discusses the release independence for NTN (e)RedCap

## 2	Discussion

When NR NTN Phase 3 was discussed in earlier meeting, release independence discussion was postponed until there is more clarity on requirements and related capabilities. Unfortunately the discussion of release independence never took place. This document discusses release independence for NTN eRedCap.

In TN side release independence for RedCap is as follows

Table 5.9-1: NR (e)RedCap operating band

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| (e)Redcap operating band | FDD | Rel-171 Rel-182 | Table B.4.18-1 |
| NOTE 1: This is applied to RedCap UE. NOTE 2: This is applied to eRedCap UE. |  |  |  |

In NTN side there are some differences to consider

- Changes to PHY-layer were made for HD-FDD

- Power boosting with a capability bit was specified in RAN4

- RRM session introduced a capability for RedCap support

The feature list related to NR NTN PH3 therefore is as follows [1]:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. NR_NTN_Ph3 | 61-1 | Tx output power enhancement for NR-NTN UE | 1. Support of NTN (e) RedCap UE power boosting when applicable as defined in 6.2 of TS 38.101-5. |  | Yes | N/A | NTN Legacy output power limitation still apply | Per band | No | FR1 only | N/A | The feature can be supported in below scenarios: NTN (e) RedCap UE in FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. Additional RRC IE is used to enable/disable power boosting. | Optional with capability signalling |
|  | 61-2 | Support of (e)RedCap UE with FR1-NTN | Indicates whether the UE supports (e)RedCap UE with FR1-NTN. | Layer-1 FG: 28-1, 48-1 Layer-2 and Layer-3 FG: 34-1 | Yes | N/A | The requirements defined for (e)RedCap UE with FR1-NTN in Rel-19 may not apply. | Per UE | No | FR1 only | N/A | If the UE indicates this capability, the UE shall support (e)RedCap UE with FR1-NTN. | Optional with capability signalling |

For power boosting, situation is analogous to NR when power boosting was added in rel-18. Even though new capabilities were added in rel-18, for earlier release UE it is still possible to support e.g. rel-18 bands without impact from the power boosting. As earlier release UE does not support ASN.1 of the newer release, it just cannot take advantage of the power boosting feature.

For the support of (e)RedCap UE with FR1-NTN, it would be necessary to make the signalling early implementable to allow no ambiguity what the UE supports.

Therefore, we should send an LS to RAN2 for asking early implementability for the capability signalling. To avoid parallel LSs, it is suggested to coordinate with feature list thread.

Proposal 1: NTN (e)RedCap is release independent:

- FD-FDD RedCap from rel-17

- FD-FDD eRedCap from rel-18

- HD-FDD RedCap and eRedCap from rel-19

Proposal 2: Send LS to RAN2 asking to make signalling of support of (e)RedCap UE with FR1-NTN early implementable, if possible.

Proposal 3: Further coordinate with feature list thread on whether a separate LS is needed.

Proposal 4: Consider draftLS in the appendix as baseline for the LS.

## 3	Conclusion

In this contribution release independence of NTN (e)RedCap. Following observations and proposals were made.

Proposal 1: NTN (e)RedCap is release independent:

- FD-FDD RedCap from rel-17

- FD-FDD eRedCap from rel-18

- HD-FDD RedCap and eRedCap from rel-19

Proposal 2: Send LS to RAN2 asking to make signalling of support of (e)RedCap UE with FR1-NTN early implementable, if possible.

Proposal 3: Further coordinate with feature list thread on whether a separate LS is needed.

Proposal 4: Consider draftLS in the appendix as baseline for the LS.

## 4	References

[1]	R4-2511883, R19 RAN4 UE feature list for NR, CMCC

# Appendix

Title:	[draft] LS on UE release independence of NTN RedCap and related capability signalling

Response to:	-

Release:	Rel-19

Work Item:	NR_IoT_NTN_req_test_enh

Source:	RAN4

To:	RAN2

Cc:	-

Contact Person:

Name:		Toni Lähteensuo

E-mail Address:	tlaehtee (at) qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 has discussed the release independence for NTN RedCap and eRedCap for which RF requirements were introduced in release 19. RAN4 concluded that NTN (e)RedCap is release independent as follows:

- FD-FDD RedCap from rel-17

- FD-FDD eRedCap from rel-18

- HD-FDD RedCap and eRedCap from rel-19

At the same time, RAN4 recognizes that capability signalling to indicate support for (e)RedCap was agreed in Rel-19 in FG 61-2, as sent to RAN2 in LS R4-2511884. RAN4 respectfully asks if it is possible to make this capability early implementable so that also earlier release UEs are enabled to indicate support.

# 2	Actions

To RAN2 group:

ACTION:  RAN4 respectfully requests RAN2 to take the provided information into account and introduce necessary updates to UE capabilities, if possible.

# 3	Dates of Next RAN WG4 Meetings

TSG-RAN4 Meeting #117	Nov 17 - 21, 2025	Dallas, USA

TSG-RAN4 Meeting #118	Feb 09 - 13, 2026	Gothenburg, Sweden