3GPP TSG RAN WG4 Meeting #116-bis	R4-2514397

Prague, CZ 13th – 17th Oct. 2025

Agenda Item:	7.4.3.2

Source: 	Ericsson

Title:	Simulation assumptions for(e)RedCap UE enhancement Study

Document for:	Approval

# Introduction

In September's Plenary, NR RRM Phase 6 got approved with the following objective:

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4]

According to the Objectives, RAN4 should extend support of less than 5 MHz CBW for RedCap. Also, relaxed RLM/BFD requirements should be extended to (e)RedCap UEs. This contribution lists the simulation parameters for a 1 Rx RedCap UE case, studying the 1 Rx impact on RedCap performance for less than 5 MHz and RedCap RLM/BDF relaxation.

# Simulation Assumptions for 1 RX RedCap Less than 5 MHz

Table 1: Simulation Assumptions for SSB index reading and MIB reading Evaluation for RedCap UE with less than 5 MHz

| Parameter | Unit | Value |
| --- | --- | --- |
| - Number of PRBs for PBCH |  | 12 PRBs for 3MHz CBW 15 PRBs for 3MHz CBW |
| - Carrier frequency | - GHz | - 900MHz |
| - Subcarrier spacing | - kHz | - 15 kHz |
| - Number of Tx antennas | - - | - 1 - 2 |
| - Number of Rx antennas | - - | - 1 |
| - DMRS | - - | - 3GPP RedCap PBCH DMRS |
| - Other assumptions |  | - Tx BW and SSB puncturing are known at the Rx side |
| - CP Length | - - | - Normal |
| - Number of transmitted SS block within a SS burst set period (K) | - - | - 1 |
| - SS burst set periodicity | - ms | - 20 |
| - Frequency Offset relative to UE frequency reference | - Hz | - 0 |
| - PBCH symbols within the SS block |  | - PSS-PBCH-SSS-PBCH |
| - Data and Control Power offset with respect to PSS and SSS | - dB | - Baseline 0 |
| - PBCH power offset with respect to PBCH-DMRS | - dB | - 0 |
| - PBCH-DMRS power offset with respect to PSS and SSS | - dB | - 0 |
| - PSS and SSS sequences | - - | - No changes expected |
| - PBCH-DMRS sequences | - - | - No changes expected |
| - PBCH-DMRS RE positions within the PBCH resource | - - | - No changes expected except for puncturing impact |
| - PBCH Channel coding |  | - No changes expected to actual Channel coding - (Polar code with 512 length and 24bit CRC) |
| - PBCH Modulation | - - | - QPSK |
| - PBCH Payload (including the CRC) | - bits | - 56bit (CRC 24bit) |
| - PBCH SNR | - dB | - -10 : 0 dB, with 1 dB spacing |
| - Propagation Condition / Channel models | - - | - For 3 km/h UE speed - TDL-C 300ns - Additional scenarios can be considered |
| - UE speed |  | - 3 km/h, |
| - Detection Method |  | - Baseline: One shot detection (i.e. no combination for different PBCHs), Soft Combine |
| - Metrics |  | - See below |
| - NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned. |  |  |

RAN4 to agree on the simulation configuration of SSB index reading MIB reading for 1 Rx RedCap UE with less than 5 MHz.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for RedCap Less than 5 MHz and made the following observations and proposals.

No table of figures entries found.Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 to agree on the simulation configuration of SSB index reading MIB reading for 1 Rx RedCap UE with less than 5 MHz.

# References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple

- R4-2410395, BigCR for NR_FR1_lessthan_5MHz_BW, Nokia

- R4-2304589, Discussion on Less than 5MHz RRM requirements, Nokia