3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514402

Prague, CZ, October 13 – 17, 2025

Agenda Item:	6.10.2.1

Source:	MediaTek Inc.

Title:	Discussion on perf part for FR2-1 SSB based L3 measurement delay reduction by optimizing BSF

Document for:	Discussion

# Introduction

In this paper we discuss the performance part for R19 RRM phase 5 WI, specifically for Rx beam sweeping factor optimization for L3 measurement delay reduction. The progress in the last meeting of RAN4 is captured in the WF [1].

# Discussion

RAN4 previously agreed on the TC list to be defined for BSF optimization as shown below:

RAN4#115 agreement: Index; Scenario; Number of TC; Configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1; - Non-DRX - In TC1/2/3/4, UE is only required to pass TC1+TC4 or TC2+TC3.; CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; 1; CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1; - With DRX, DRX.1 for sub-test1 and DRX.7 for sub-test2 (same as in table A.7.6.1.2.1-2) - In TC1/2/3/4, UE is only required to pass TC1+TC4, or TC2+TC3.; Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; 1; Samsung. 5; Scenario 5: Handover; 1; - Intra-frequency handover from FR2 to FR2 - Unknown target cell; OPPO. 1; - Inter-frequency handover from FR2 to FR2 - Unknown target cell. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1; - Inter-frequency RRC Re-establishment; - Unknown target cell - FR2 to FR2; Intel. 1; - RRC Connection release with redirection - FR2 to FR2.

Issue 2-1-2: whether or not to test FBS quitting - Option 1(CMCC): it is proposed to discuss whether to test FBS quiting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition. - [Way forward] - Continue discussion in RAN4#116bis.

For issue 2-1-2, some companies proposed to test FBS quitting (i.e., after UE activate multi-Rx for fast beam sweeping). Quitting FBS can be due to 1) T1 timer of 90s has expired, 2) UAI indication to switch to single-Rx, or 3) the conditions are not satisfied anymore after UE activate FBS. In any case, UE is not required to be tested whether UE is not using multi-Rx anymore when these quitting cases could occur since this will be completely up to UE. In other words, UE is only required to be tested when UE is required to activate multi-Rx for FBS, not the other way around.

Proposal 1: Not need to test the quitting of FBS (i.e., after UE activate multi-Rx for fast beam sweeping).

Proposal 2: The TCs are only required to test when UE is required to apply FBS for L3 measurement delay reduction.

This can be achieved by using serving cell’s quality as the triggering condition for FBS. The TE’s power can be adjusted to be higher than a threshold at T1, then lower power than a threshold at T2. The UE need to send measurement report based on FBS within the time period of T2.

Proposal 3: Use two different power levels of the serving cell to trigger FBS in the TCs.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Not need to test the quitting of FBS (i.e., after UE activate multi-Rx for fast beam sweeping).

Proposal 2: The TCs are only required to test when UE is required to apply FBS for L3 measurement delay reduction.

Proposal 3: Use two different power levels of the serving cell to trigger FBS in the TCs.

# References

- R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.