3GPP TSG-CT WG1 Meeting #157	C1-256404

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7042 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the 5GS mobile identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the 5GS mobile identity IE, the Type of identity field is in bits 1 to 3 of octet 4, and other bits in octet 4 is defined for different fields based on the value of Type of identity field, for the Type of identity field is set to "IMEI" or "IMEISV", the bit 4 of octet 4 is defined for the "odd/even indic". However, in Table 9.11.3.4.1, the whole octet 4 is specified for the Type of identity field and "odd/even indic" field for the Type of identity field is set to "IMEI" or "IMEISV" which needs to be fixed. And the name of the "odd/even indication" field used which is not aligned with the figure 9.11.3.4.2. And "All other values are reserved" is not needed since there is no other values left. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Fix the bit used for "Type of identity" field and the "odd/even indication" field; 2) Align the field name for the "odd/even indication" field; 3) remove "All other values are reserved" for "Type of identity" field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong bits specified for for "Type of identity" field and the "odd/even indication" field; inconsistent name used for "odd/even indication" field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.11.3.4	5GS mobile identity

The purpose of the 5GS mobile identity information element is to provide either the SUCI, the 5G-GUTI, the IMEI, the IMEISV, the 5G-S-TMSI, the MAC address or the EUI-64.

The 5GS mobile identity information element is coded as shown in figures 9.11.3.4.1, 9.11.3.4.2, 9.11.3.4.3, 9.11.3.4.4, 9.11.3.4.5, 9.11.3.4.6, 9.11.3.4.8 and 9.11.3.4.7, and table 9.11.3.4.1.

The 5GS mobile identity is a type 6 information element with a minimum length of 4.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 1 | 1 | 1 | 1 | 0 spare | Type of identity |  |  | octet 4 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 5 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 7 |
| AMF Region ID |  |  |  |  |  |  |  | octet 8 |
| AMF Set ID |  |  |  |  |  |  |  | octet 9 |
| AMF Set ID (continued) |  | AMF Pointer |  |  |  |  |  | octet 10 |
| 5G-TMSI |  |  |  |  |  |  |  | octet 11 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 12 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 13 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 14 |

Figure 9.11.3.4.1: 5GS mobile identity information element for type of identity "5G-GUTI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Identity digit 1 |  |  |  | Odd/even indication | Type of identity |  |  | octet 4 |
| Identity digit p+1 |  |  |  | Identity digit p |  |  |  | octet 5* |

Figure 9.11.3.4.2: 5GS mobile identity information element for type of identity "IMEI" or "IMEISV"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 spare | SUPI format |  |  | 0 spare | Type of identity |  |  | octet 4 |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet 5 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet 6 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet 7 |
| Routing indicator digit 2 |  |  |  | Routing indicator digit 1 |  |  |  | octet 8 |
| Routing indicator digit 4 |  |  |  | Routing indicator digit 3 |  |  |  | octet 9 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Protection scheme Id |  |  |  | octet 10 |
| Home network public key identifier |  |  |  |  |  |  |  | octet 11 |
| Scheme output |  |  |  |  |  |  |  | octet 12 - x |

Figure 9.11.3.4.3: 5GS mobile identity information element for type of identity "SUCI" and SUPI format "IMSI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSIN digit 2 |  |  |  | MSIN digit 1 |  |  |  | octet 12 |
| … |  |  |  |  |  |  |  |  |
| MSIN digit n+1 |  |  |  | MSIN digit n |  |  |  | octet x |

Figure 9.11.3.4.3a: Scheme output for type of identity "SUCI", SUPI format "IMSI" and Protection scheme Id "Null scheme"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 Spare | SUPI format |  |  | 0 Spare | Type of identity |  |  | octet 4 |
| SUCI NAI |  |  |  |  |  |  |  | octet 5 - y |

Figure 9.11.3.4.4: 5GS mobile identity information element for type of identity "SUCI" and SUPI format "Network specific identifier", "GCI" or "GLI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 1 | 1 | 1 | 1 | 0 spare | Type of identity |  |  | octet 4 |
| AMF Set ID |  |  |  |  |  |  |  | octet 5 |
| AMF Set ID (continued) |  | AMF Pointer |  |  |  |  |  | octet 6 |
| 5G-TMSI |  |  |  |  |  |  |  | octet 7 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 8 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 9 |
| 5G-TMSI (continued) |  |  |  |  |  |  |  | octet 10 |

Figure 9.11.3.4.5: 5GS mobile identity information element for type of identity "5G-S-TMSI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 | 0 | 0 | 0 | 0 | Type of identity |  |  | octet 4 |
| spare |  |  |  |  |  |  |  |  |

Figure 9.11.3.4.6: 5GS mobile identity information element for type of identity "No identity"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 spare | 0 spare | 0 spare | 0 spare | MAURI | Type of identity |  |  | octet 4 |
| MAC address |  |  |  |  |  |  |  | octet 5 octet 10 |

Figure 9.11.3.4.7: 5GS mobile identity information element for type of identity "MAC address"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | Type of identity |  |  | octet 4 |
| EUI-64 |  |  |  |  |  |  |  | octet 5 octet 12 |

Figure 9.11.3.4.8: 5GS mobile identity information element for type of identity "EUI-64"

Table 9.11.3.4.1: 5GS mobile identity information element

| Type of identity (octet 4 bits 1 to 3) Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 2 | 1 |  |  |  |  |  |  |  |
| 0 | 0 | 0 | No identity (see NOTE 1) |  |  |  |  |  |  |
| 0 | 0 | 1 | SUCI |  |  |  |  |  |  |
| 0 | 1 | 0 | 5G-GUTI |  |  |  |  |  |  |
| 0 | 1 | 1 | IMEI |  |  |  |  |  |  |
| 1 | 0 | 0 | 5G-S-TMSI |  |  |  |  |  |  |
| 1 | 0 | 1 | IMEISV |  |  |  |  |  |  |
| 1 | 1 | 0 | MAC address |  |  |  |  |  |  |
| 1 | 1 | 1 | EUI-64 |  |  |  |  |  |  |
| Odd/even indication (octet 4 bit 4) Bit |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |
| 0 |  |  | even number of identity digits |  |  |  |  |  |  |
| 1 |  |  | odd number of identity digits |  |  |  |  |  |  |
| For the 5G-GUTI, then bits 5 to 8 of octet 4 are coded as "1111", octet 5 through 7 contain the MCC and MNC values as specified below, octet 8 through 10 contain the AMF Region ID, the AMF Set ID and the AMF Pointer values and octet 11 through 14 contain the 5G-TMSI as defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| MCC, Mobile country code (octet 5, octet 6 bits 1 to 4) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |  |  |  |  |
| MNC, Mobile network code (octet 6 bits 5 to 8, octet 7) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 6 shall be coded as "1111". The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |  |  |  |  |  |  |  |  |
| AMF Region ID (octet 8) This field contains the binary encoding of the AMF Region ID. Bit 8 of octet 7 is the most significant bit and bit 1 of octet 7 is the least significant bit. AMF Set ID (octet 9, octet 10 bits 7 to 8) This field contains the binary encoding of the AMF Set ID. Bit 8 of octet 9 is the most significant bit and bit 7 of octet 10 is the least significant bit. AMF Pointer (octet 10 bits 1 to 6) This field contains the binary encoding of the AMF Pointer. Bit 6 of octet 9 is the most significant bit and bit 1 of octet 9 is the least significant bit. 5G-TMSI (octet 11 to 14) Bit 8 of octet 11 is the most significant bit and bit 1 of octet 14 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| Identity digit (octet 4 bits 5 to 8, octet 5 etc.) |  |  |  |  |  |  |  |  |  |
| For the IMEI, Identity digit field is coded using BCD coding. If the number of identity digits is even then bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". The format of the IMEI is described in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| For the IMEISV, Identity digit field is coded using BCD coding. Bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". The format of the IMEISV is described in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| For the SUCI, bit 8 of octet 4 is spare and shall be coded as zero. Bits 5-7 of octet 4 contain the SUPI format and are coded as shown below. |  |  |  |  |  |  |  |  |  |
| SUPI format (octet 4, bits 5-7) Bits |  |  |  |  |  |  |  |  |  |
| 7 | 6 | 5 |  |  |  |  |  |  |  |
| 0 | 0 | 0 |  | IMSI |  |  |  |  |  |
| 0 | 0 | 1 |  | Network specific identifier |  |  |  |  |  |
| 0 | 1 | 0 |  | GCI |  |  |  |  |  |
| 0 | 1 | 1 |  | GLI |  |  |  |  |  |
| All other values are interpreted as IMSI by this version of the protocol. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format "IMSI", octets 5 through 7 contain the MCC and MNC values as specified below. For subsequent fields, bit 8 of octet 8 is the most significant bit and bit 1 of the last octet the least significant bit. The required fields for the SUCI are as defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |  |
| MCC, Mobile country code (octet 5, octet 6 bits 1 to 4) The MCC field is coded as in ITU-T Recommendation E.212 [42], annex A. |  |  |  |  |  |  |  |  |  |
| MNC, Mobile network code (octet 6 bits 5 to 8, octet 7) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 6 shall be coded as "1111". The MCC and MNC digits are coded as octets 6 to 8 of the Temporary mobile group identity IE in figure 10.5.154 of 3GPP TS 24.008 [12]. |  |  |  |  |  |  |  |  |  |
| Routing indicator (octets 8-9) Routing Indicator shall consist of 1 to 4 digits. The coding of this field is the responsibility of home network operator but BCD coding shall be used. If a network operator decides to assign less than 4 digits to Routing Indicator, the remaining digits shall be coded as "1111" to fill the 4 digits coding of Routing Indicator (see NOTE 2). If no Routing Indicator is configured in the USIM or the ME, the UE shall code bits 1 to 4 of octet 8 of the Routing Indicator as "0000" and the remaining digits as "1111". |  |  |  |  |  |  |  |  |  |
| Protection scheme identifier (octet 10 bits 1 to 4) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | Null scheme |  |  |  |  |  |
| 0 | 0 | 0 | 1 | ECIES scheme profile A |  |  |  |  |  |
| 0 | 0 | 1 | 0 | ECIES scheme profile B |  |  |  |  |  |
| 0 | 0 | 1 | 1 |  |  |  |  |  |  |
| to |  |  |  | Reserved |  |  |  |  |  |
| 1 | 0 | 1 | 1 |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 |  |  |  |  |  |  |
| to |  |  |  | Operator-specific protection scheme |  |  |  |  |  |
| 1 | 1 | 1 | 1 |  |  |  |  |  |  |
| Bits 5-8 of octet 10 are spare and shall be coded as zero. |  |  |  |  |  |  |  |  |  |
| Home network public key identifier (octet 11) |  |  |  |  |  |  |  |  |  |
| The Home network public key identifier (PKI) field is coded as defined in 3GPP TS 23.003 [4]. Home network public key identifier shall be coded as "00000000" when Protection scheme identifier is set to "0000" (i.e. Null scheme). |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | Home network PKI value 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |
| to |  |  |  |  |  |  |  |  | Home network PKI value (1-254) |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Reserved |
| Scheme output (octets 12 to x) The Scheme output field consists of a string of characters with a variable length or hexadecimal digits as specified in 3GPP TS 23.003 [4]. If Protection scheme identifier is set to "0000" (i.e. Null scheme), then the Scheme output consists of the MSIN and is coded using BCD coding with each digit of the MSIN coded over 4 bits. If the MSIN includes an odd number of digits, bits 5 to 8 of octet x shall be coded as "1111". If Protection scheme identifier is not "0000" (i.e. ECIES scheme profile A, ECIES scheme profile B or Operator-specific protection scheme), then Scheme output is coded as hexadecimal digits. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format set to "Network specific identifier", the SUCI NAI field contains an NAI constructed as specified in subclause 28.7.3 of 3GPP TS 23.003 [4] and encoded as UTF-8 string. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format set to "GCI", the SUCI NAI field contains an NAI constructed as specified in subclause 28.15.5 of 3GPP TS 23.003 [4] and encoded as UTF-8 string. |  |  |  |  |  |  |  |  |  |
| For the SUCI with SUPI format set to "GLI", the SUCI NAI field contains an NAI constructed as specified in subclause 28.16.5 of 3GPP TS 23.003 [4] and encoded as UTF-8 string. |  |  |  |  |  |  |  |  |  |
| For the 5G-S-TMSI, bits 5 to 8 of octet 4 are coded as "1111". The coding of the 5G-S-TMSI is left open for each administration. |  |  |  |  |  |  |  |  |  |
| AMF Set ID (octet 5, octet 6 bits 7 to 8) This field contains the binary encoding of the AMF Set ID. Bit 8 of octet 5 is the most significant bit and bit 7 of octet 6 is the least significant bit. AMF Pointer (octet 6 bits 1 to 6) This field contains the binary encoding of the AMF Pointer. Bit 6 of octet 6 is the most significant bit and bit 1 of octet 6 is the least significant bit. 5G-TMSI (octet 7 to 10) Bit 8 of octet 7 is the most significant bit and bit 1 of octet 10 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| For Type of identity "No identity", the length of mobile identity contents parameter shall be set to 1 and the bits 4-8 of octet 4 are spare and shall be coded as zero. |  |  |  |  |  |  |  |  |  |
| MAC address usage restriction indication (MAURI) (octet 4 bit 4) |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  | No restrictions |  |  |  |  |  |
| 1 |  |  |  | MAC address is not usable as an equipment identifier |  |  |  |  |  |
| MAC address (octets 5 to 10) This field contains the MAC address as defined in subclause 8 of IEEE Std 802 [43]. Bit 8 of octet 5 is the most significant bit and bit 1 of octet 10 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| EUI-64 (octets 5 to 12) This field contains an EUI-64 as defined in [48]. Bit 8 of octet 5 is the most significant bit and bit 1 of octet 12 is the least significant bit. |  |  |  |  |  |  |  |  |  |
| NOTE 1: This can be used when the requested identity is not available at the UE during the identification procedure. NOTE 2: For a 3-digit Routing Indicator, e.g "567", bits 1 to 4 of octet 8 are coded as "0101", bits 5 to 8 of octet 8 are coded as "0110", bits 1 to 4 of octet 9 are coded as "0111", bits 5 to 8 of octet 9 are coded as "1111". |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *