3GPP TSG-CT WG1 Meeting #157	C1-256244

Sophia-Antipolis, France 13-17 October 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.545 | CR | 0185 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN for sidelink CDDL document |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an EN: “Editor's note (WIC: eLSAPP, CR 0178): CCDL document updates for the sidelink positioning / ranging management procedure is FFS.” It is proposed to update location report configuration for the sidelink positioning / ranging management procedure in CDDL document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Result the EN for the sidelink positioning / ranging management procedure in CDDL document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved EN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.3.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

B.3.1.5.2	CDDL document

;;; LocationAreaQuery

LocationAreaQuery = {

geoArea: GeographicArea

? geofencArea: GeographicArea

}

;;; LocationAreaInfo

LocationAreaInfo = {

? valReqUe: ValTargetUe

? ueList: [* UeInfo]

}

;;; UeInfo

UeInfo = {

? ueId: ValTargetUe

? ueLoc: LocationInfo

}

;;; LocationReportConfiguration

;;+ Represents Location reporting configuration information.

LocationReportConfiguration = {

valTgtUes: [* ValTargetUe]

locationType: Accuracy

? triggeringCriteria: [* TriggeringCriteriaType]

? minimumIntervalLength: Uinteger

? immediateReportInd: boolean

? endpointId: EndpointId

? timestampInd: boolean

? requestedLocAccessType: [* LocationType]

? requestedPosMethod: [* PositioningMethod]

? LocationReuse: [* LocationReuse]

? currentCoordinate: GeographicalCoordinates

? slPosMgmtParams: SlPosMgmtParams

}

;;; Accuracy

Accuracy = "CURRENT_SERVING_NCGI" / "NEIGHBOURING_NCGI" / "MBMS_SA" / "MBSFN_AREA" / "CURRENT_GEOGRAPHICAL_COORDINATE"

;;; TriggeringCriteriaType

TriggeringCriteriaType = {

? cellChange: CellChange

? trackingAreaChange: TrackingAreaChange

? plmnChange: PlmnChange

? mbmsSaChange: MbmsSaChange

? mbsfnAreaChange: MbsfnAreaChange

? periodicReport: PeriodicReport

? travelledDistance: TravelledDistance

? verticalAppEvent: VerticalAppEvent

? geographicalAreaChange: GeographicalAreaChange

? validPeriod: [+ ScheduledCommunicationTime]; The list of the scheduled time intervals for the reporting in form of day of the week and/or time period.

}

;;; CellChange

CellChange = {

? anyCellChange: BaseTrigger

? enterSpecificCells: SpecificCells

? exitSpecificCells: SpecificCells

}

;;; SpecificCells

SpecificCells = {

triggerId: TriggerId

cells: [* CellId]

}

;;; TrackingAreaChange

TrackingAreaChange = {

? anyTrackingAreaChange: BaseTrigger

? enterSpecificTrackingAreas: SpecificTrackingAreas

? exitSpecificTrackingAreas: SpecificTrackingAreas

}

;;; SpecificTrackingAreas

SpecificTrackingAreas = {

triggerId: TriggerId

trackingAreas: [* TaId]

}

;;; PlmnChange

PlmnChange = {

? AnyPlmnChange: BaseTrigger

? EnterSpecificPlmns: SpecificPlmns

? ExitSpecificPlmns: SpecificPlmns

}

;;; SpecificPlmns

SpecificPlmns = {

triggerId: TriggerId

plmns: [* PlmnId]

}

;;; MbmsSaChange

MbmsSaChange = {

? anyPlmnChange: BaseTrigger

? enterSpecificPlmns: SpecificMbmsSas

? exitSpecificPlmns: SpecificMbmsSas

}

;;; SpecificMbmsSas

SpecificMbmsSas = {

triggerId: TriggerId

mbmsSas: [* MbmsSaId]

}

;;; MbsfnAreaChange

MbsfnAreaChange = {

? anyPlmnChange: BaseTrigger

? enterSpecificMbsfnAreas: SpecificMbsfnAreas

? exitSpecificPlmn: SpecificMbsfnAreas

}

;;; SpecificMbsfnAreas

SpecificMbsfnAreas = {

triggerId: TriggerId

mbsfnAreas: [* MbsfnAreaId]

}

;;; PeriodicReport

PeriodicReport = {

triggerId: TriggerId

interval: Uinteger

}

;;; TravelledDistance

TravelledDistance = {

triggerId: TriggerId

distance: Uinteger

}

;;; VerticalAppEvent

VerticalAppEvent = {

? initialLogOn: BaseTrigger

? locConfigReceived: BaseTrigger

? anyOtherEvent: BaseTrigger

}

;;; GeographicalAreaChange

GeographicalAreaChange = {

? AnyGeoAreaChange: BaseTrigger

? EnterSpecificGeoAreas: SpecificGeoAreas

? ExitSpecificGeoAreas: SpecificGeoAreas

}

;;; SpecificGeoAreas

SpecificGeoAreas = {

triggerId: TriggerId

geoAreas: [* GeographicArea]

}

;;;LocationAccessTypeType

LocationAccessType = "3GPP_ACCESS" / "NON_3GPP_ACCESS" / text

;;;PositioningMethod

PositioningMethod = "CELLID" / "ECID" / "OTDOA" / "BAROMETRIC_PRESSURE" / "WLAN" / "BLUETOOTH" / "MBS" / "MOTION_SENSOR" / "DL_TDOA" / "DL_AOD" / "MULTI-RTT" / "NR_ECID" / "UL_TDOA" / "UL_AOA" / "NETWORK_SPECIFIC" / "RL_TDOA" / "RL_TOA" / "RL_AOA" / "RL_RT" / text

;;; EndpointId

;;+ Unique identifier of an endpoint of the selected VAL server.

EndpointId = text

;;; ScheduledCommunicationTime

;;+ Represents the scheduled time interval.

ScheduledCommunicationTime = {

? daysOfWeek: [1*6 DayOfWeek]   ; Identifies the day(s) of the week. If absent, it indicates every day of the week.

? timeOfDayStart: TimeOfDay

? timeOfDayEnd: TimeOfDay

}

;;; DayOfWeek

;;+ Integer between and including 1 and 7 denoting a weekday. Value 1 shall indicate Monday, and the subsequent weekdays shall be indicated with the next higher numbers, so value 7 shall indicate Sunday.

DayOfWeek = 1..7

;;; TimeOfDay

;;+ String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

TimeOfDay = text

;;; RequestedLocation

RequestedLocation = {

valTgtUes: [* ValTargetUe]

? locationType: Accuracy

? requestedLocAccessType: [* LocationAccessType]

? requestedPosMethod: [* PositioningMethod]

}

;;; LocationReport

LocationReport = {

valTgtUe: ValTargetUe

triggerIds: [* TriggerId]

locInfo: LocationInfo

? timestamp: TimeOfDay

? velocityInfo: VelocityInfo

? locDataStatistic: LocDataStatistic

? subscriptionIdentifier: string

}

;;; LocationInfo

LocationInfo = {

? cellId: CellId

? neighbouringCellIds: [* CellId]

? mbmsSaId: MbmsSaId

? mbsfnAreaId: MbsfnAreaId

? currentCoordinate: GeographicalCoordinates

? subscriptionIdentifier: string

}

;;; BaseTrigger

BaseTrigger = {

triggerId: TriggerId

}

;;; TriggerId

;;+ Unique identifier of a trigger.

TriggerId = text

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; GeographicArea

;;+ Geographic area specified by different shape.

GeographicArea = Point / PointUncertaintyCircle / PointUncertaintyEllipse / Polygon / PointAltitude / PointAltitudeUncertainty / EllipsoidArc

;;; GADShape

;;+ Common base type for GAD shapes.

GADShape = {

shape: SupportedGADShapes

}

;;; Point

;;+ Ellipsoid Point.

Point = {

~GADShape

point: GeographicalCoordinates

}

;;; PointUncertaintyCircle

;;+ Ellipsoid point with uncertainty circle.

PointUncertaintyCircle = {

~GADShape

point: GeographicalCoordinates

uncertainty: Uncertainty

}

;;; PointUncertaintyEllipse

;;+ Ellipsoid point with uncertainty ellipse.

PointUncertaintyEllipse = {

~GADShape

point: GeographicalCoordinates

uncertaintyEllipse: UncertaintyEllipse

confidence: Confidence

}

;;; Polygon

;;+ Polygon.

Polygon = {

~GADShape

pointList: PointList

}

;;; PointAltitude

;;+ Ellipsoid point with altitude.

PointAltitude = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

}

;;; PointAltitudeUncertainty

;;+ Ellipsoid point with altitude and uncertainty ellipsoid.

PointAltitudeUncertainty = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

uncertaintyEllipse: UncertaintyEllipse

uncertaintyAltitude: Uncertainty

confidence: Confidence

}

;;; EllipsoidArc

;;+ Ellipsoid Arc.

EllipsoidArc = {

~GADShape

point: GeographicalCoordinates

innerRadius: InnerRadius

uncertaintyRadius: Uncertainty

offsetAngle: Angle

includedAngle: Angle

confidence: Confidence

}

;;; GeographicalCoordinates

;;+ Geographical coordinates.

GeographicalCoordinates = {

lon: -180.0..180.0

lat: -90.0..90.0

}

;;; UncertaintyEllipse

;;+ Ellipse with uncertainty.

UncertaintyEllipse = {

semiMajor: Uncertainty

semiMinor: Uncertainty

orientationMajor: Orientation

}

;;; PointList

;;+ List of points.

PointList = [3*15 GeographicalCoordinates]

;;; Altitude

;;+ Indicates value of altitude.

Altitude = -32767.0..32767.0

;;; Angle

;;+ Indicates value of angle.

Angle = 0..360

;;; Uncertainty

;;+ Indicates value of uncertainty.

Uncertainty = float32 .ge 0

;;; Orientation

;;+ Indicates value of orientation angle.

Orientation = 0..180

;;; Confidence

;;+ Indicates value of confidence.

Confidence = 0..100

;;; InnerRadius

;;+ Indicates value of the inner radius.

InnerRadius = (0..327675) .and int32

;;; SupportedGADShapes

;;+ Indicates supported GAD shapes.

SupportedGADShapes = "POINT" / "POINT_UNCERTAINTY_CIRCLE" / "POINT_UNCERTAINTY_ELLIPSE" / "POLYGON" / "POINT_ALTITUDE" / "POINT_ALTITUDE_UNCERTAINTY" / "ELLIPSOID_ARC" / "LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE" / "LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID" / text

;;; CellId

;;+ Unique identifier of a cell.

CellId = text

;;; TaId

;;+ Unique identifier of a tracking area.

TaId = text

;;; PlmnId

;;+ Unique identifier of a PLMN.

PlmnId = text

;;; MbmsSaId

;;+ Unique identifier of a MBMS serving area.

MbmsSaId = text

;;; MbsfnAreaId

;;+ Unique identifier of a MBSFN area.

MbsfnAreaId = text

;;; AdaptiveReportConfiguration

;;+ Represents Adaptive reporting configuration information.

AdaptiveReportConfiguration = {

valTgtUes: [* ValTargetUe]

triggeringCriteria: [* TriggeringCriteriaType]

}

;;; AdaptiveResult

;;+ Represents the result for the Adaptive reporting configuration information.

AdaptiveResult = "ACCEPT" / "REJECT"

;;; VelocityInfo

;;+ The velocity of the target UE for which the location information is requested.

VelocityInfo = text

;;; LocDataStatistic

;;+ The statistic result of target UE location data per temporal or spatial granularity as requested.

LocDataStatistic = text

;;; LocationReuse

LocationReuse = {

valTgtUes: [* ValTargetUe]

LocReuseInd: boolean

? CurLocation: [* LocationReport]

? validPerio: [+ ScheduledCommunicationTime]; The list of the scheduled time intervals for the reporting in form of day of the week and/or time period.

}

;;; SlPosMgmtParams

LocationReuse = {

? distanceInd: boolean

? directionInd: boolean

}

* * * End of Changes * * * *