3GPP TSG-CT WG1 Meeting #157	C1-256430

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.545 | CR | 0189 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the procedures using the failure-cause element |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification was updated in Rel-19 by adding a new element called "failure-cause" to reflect new requirements added to TS 24.433 (see CR in S6-245509). However, the stage-3 requirements in TS 24.545 are not correct for the procedures describing the failure-cause. Note that in case of HTTP 403 response what it is provided is the failure-cause element but not the value itself as indicated in clauses 6.2.3.1 and 6.2.23.1. Also, there is misalignment with other parts of the specification (data semantics). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The inclusiong of the failure-cause element is made correct as per the reason of change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong requirements for implementations on what the HTTP 403 response contais remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, 6.2.23.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.2.3.1	SLM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <request> element included in the <location-info> root element;

the SLM-C:

a)	if the SLM-C does not obtain the requested UE’s location information due to the target VAL UE has moved away from the SLM-C, the SLM-C shall send the HTTP 403 (Forbidden) response including the <failure-cause> element set to "MOVED_AWAY" in the <anyExt> element in the <location-info> element and not continue the remaining steps;

b)	if the positioning method in the <requested-pos-method> in the <anyExt> element in the <request> element in the <location-info> element is not supported by SLM-C, the SLM-C shall send the HTTP 403 (Forbidden) response including the <failure-cause> set to "UNSUPPORTED_POSITIONING_METHOD" and not continue the remaining steps; or

c)	otherwise, may send a location report as specified in clause 6.2.2.2.2.

NOTE:	The SLM-C can determine that the VAL UE moved away by an optional surrounding UE retrieval method (e.g. ProSe, BT, WiFi) as described in clause 9.3.7 of 3GPP TS 23.434 [4] or by using the location report procedure as described in clause 6.3.2.2.

* * * Next Change * * * *

#### 6.2.23.1	SLM client HTTP procedure

Upon receiving of an HTTP GET request where the Request-URI identifies the SLM-C as specified in the specific vertical application, the SLM-C:

a)	shall initiate ranging-based services and sidelink positioning in 5G (see clause 4A) as described in 3GPP TS 24.514 [37], off-network procedures (see clause 6.3) or both, and:

1)	if the SLM-C does not receive any result for the short-range based positioning information, the SLM-C shall send the HTTP 403 (Forbidden) response including the in the <failure-cause> set to "NO RESULT" and skip the rest of the steps;

b)	shall generate an HTTP 200 (OK) response according to IETF RFC 9110 [16]. In the HTTP 200 (OK) response message, the SLM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

2)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body;

3)	shall include at least one of <report> element in the <location-info> root element and in each <report> element:

i)	shall include <identity> element identity the VAL user who acts as the reference UE; and

ii)	shall include at least one of the following sub-elements:

A)	a <current-coordinate> in the <current-location> element to indicate the coordinates of the reference UE with origin corresponding to SLM-C UE;

B)	shall include <velocity-info> to indicate velocity of the reference UE with origin corresponding to SLM-C UE; and

C)	shall include <range-direction> to the distance and direction between the reference UE and SLM-C UE; and

c)	shall send the HTTP 200 (OK) response towards the SLM-C.

* * * End of Changes * * * *