3GPP TSG-CT WG1 Meeting #157	C1-256431

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.545 | CR | 0190 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the XML schema and data semantics for the <failure-cause> element |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification was updated in Rel-19 by adding a new element called "failure-cause" to reflect new requirements added to TS 24.433 (see CR in S6-245509). However, the stage-3 requirements in TS 24.545 are not correct for the procedures describing the failure-cause. Note that in case of HTTP 403 response what it is provided is the failure-cause element but not the value itself as indicated in clauses 6.2.3.1 and 6.2.23.1. Also, there is misalignment with other parts of the specification (i.e., data semantics and XML schema) as follows: - the failure-cause element is not defined under the <AnyExt> element of the <location-info> element; - the XML coding is wrong as it requires to have first a <failure> element, and the inside the <failure-cause> one, and this is not align with the related procedures; - the data semantics are then incorrect and not aligned with the procedures either; and - also the data semantics fails to describe the case of “NO RESULT” value as a valid one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The failure-cause element is made correct as per the reason of change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong requirements for implementations on what the HTTP 403 response contais remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.4.2	XML schema

Editor's note (WIC: eLSAPP, CR 0170):	XML updates for the location positioning subscription procedure are FFS.

Editor's note (WIC: eLSAPP, CR 0171):	XML updates for the location positioning notification procedure are FFS.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:sealLocationInfo:1.0"

xmlns:sealloc="urn:3gpp:ns:sealLocationInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<xs:import namespace="http://www.w3.org/XML/1998/namespace"

schemaLocation="http://www.w3.org/2001/xml.xsd"/>

<xs:element name="location-info" id="loc">

<xs:annotation>

<xs:documentation>Root element, contains all information related to location configuration, registration,location request and location reporting for the SEAL service</xs:documentation>

</xs:annotation>

<xs:complexType>

<xs:choice maxOccurs="unbounded">

<xs:element name="identity" type="sealloc:tIdentityType"/>

<xs:element name="configuration" type="sealloc:tConfigurationType"/>

<xs:element name="report" type="sealloc:tReportType"/>

<xs:element name="location-based-query" type="sealloc:tLocationBasedQueryType"/>

<xs:element name="location-based-response" type="sealloc:tLocationBasedResponseType"/>

<xs:element name="notification" type="sealloc:tNotificationType"/>

<xs:element name="request" type="sealloc:tRequestType"/>

<xs:element name="requested-identity" type="sealloc:tRequestedIDType"/>

<xs:element name="subscription" type="sealloc:tSubscriptionType"/>

<xs:element name="report-request" type="sealloc:tReportRequestType"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="LocationCapability" type="sealloc:tLocationCapabilityType"/>

<xs:element name="GeofencBasedQuery" type="sealloc:tLocationBasedQueryType"/>

<xs:element name="AdaptiveConfiguration" type="sealloc:TriggeringCriteriaType"/>

<xs:element name="AdaptiveInd" type="sealloc:tAdaptiveIndType"/>

<xs:element name="confirm-loc-report" type="sealloc:tconfirmLocReportType"/>

<xs:element name="off-network-loc-pos-configuration-req" type="sealloc:tOffNetworkLocPosConfigurationReqType"/>

<xs:element name="off-network-loc-pos-configuration-res" type="sealloc:tOffNetworkLocPosConfigurationResType"/>

<xs:element name="history-loc-result-report" type="sealloc:tHistoryLocResulReportType"/>

<xs:element name="failure-cause" type="xs:string"/>

<!-- The following element is added for extensibility and to be placed in the anyExt element above or to be used in the anyExt in the tSubscriptionType -->

<xs:element name="LocationQoS" type="sealloc:tLocationQoSType"/>

<xs:complexType name="tIdentityType">

<xs:choice>

<xs:element name="VAL-user-id" type="sealloc:contentType" minOccurs="0"/>

<xs:element name="VAL-group-id" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tConfigurationType">

<xs:sequence>

<xs:element name="location-information" type="sealloc:tRequestedLocationType" minOccurs="0"/>

<xs:element name="triggering-criteria" type="sealloc:TriggeringCriteriaType" minOccurs="0"/>

<xs:element name="minimum-interval-length" type="xs:positiveInteger" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="config-scope">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="Full"/>

<xs:enumeration value="Update"/>

</xs:restriction>

</xs:simpleType>

</xs:attribute>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="requested-loc-access-type" type="sealloc:tLocationAccessTypeType"/>

<xs:element name="requested-pos-method" type="xs:string"/>

<xs:element name="requested-velocity-info" type="sealloc:tEmptyType" minOccurs="0"/>

<xs:element name="verification" type="sealloc:tVerificationType"/>

<xs:element name="srpos-info-req" type="xs:SrPosInfoReqType"/>

<xs:complexType name="tReportType">

<xs:sequence>

<xs:element name="trigger-id" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="current-location" type="sealloc:tCurrentLocationType"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="report-id" type="xs:string" use="optional"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="timestamp" type="xs:dateTime"/>

<xs:annotation>

<xs:documentation>timestampt can only be present once in a single report</xs:documentation>

</xs:annotation>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="velocity-info" type="sealloc:tVelocityInfoType" minOccurs="0"/>

<xs:element name="VAL-user-id" type="sealloc:contentType" minOccurs="0"/>  <xs:element name="range-direction" type="sealloc:tRangeDirectionType"/>

<xs:complexType name="tLocationBasedQueryType">

<xs:sequence>

<xs:element name="polygon-area" type="sealloc:tPolygonAreaType" minOccurs="0"/>

<xs:element name="ellipsoid-arc-area" type="sealloc:tEllipsoidArcType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLocationBasedResponseType">

<xs:sequence>

<xs:element name="identities-list" type="sealloc:tIDsListType"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tNotificationType">

<xs:sequence>

<xs:element name="identities-list" type="sealloc:tIDsListType"/>

<xs:element name="reports" type="sealloc:tReportsType"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="subscription-identifier" type="xs:string"/>

<xs:element name="velocity-info" type="xs:string"/>

<xs:element name="loc-data-statistic" type="xs:string"/>

<xs:element name="confirm" type="sealloc:tEmptyType"/>

<xs:complexType name="tRequestType">

<xs:sequence>

<xs:element name="location-information" type="sealloc:tRequestedLocationType" minOccurs="0"/>

<xs:element name="requested-loc-access-type" type="sealloc:tLocationAccessTypeType" minOccurs="0"/>

<xs:element name="requested-pos-method" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="request-id" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements is added for extensibility and to be placed in the anyExt element above or to be used in the anyExt in the tSubscriptionType -->

<xs:element name="confirm" type="sealloc:tEmptyType"/>

<xs:element name="location-reuse-request" type="sealloc:tLocationReuseRequestType"/>

<xs:complexType name="tRequestedIDType">

<xs:choice>

<xs:element name="VAL-user-id" type="sealloc:contentType" minOccurs="0"/>

<xs:element name="VAL-group-id" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="tSubscriptionType">

<xs:sequence>

<xs:element name="identities-list" type="sealloc:tIDsListType"/>

<xs:element name="time-interval-length" type="xs:positiveInteger"/>

<xs:element name="subscription-identifier" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="expiry-time" type="xs:nonPositiveInteger"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="suppl-loc-info-ind" type="xs:boolean"/>

<xs:element name="loc-data-statistic-ind" type="xs:boolean"/>

<xs:element name="req-time-info" type="sealloc:tVelocityInfoType"/>

<xs:element name="req-loc-info" type="xs:string"/>

<xs:complexType name="tReportRequestType">

<xs:sequence>

<xs:element name="immediate-report-indicator" type="xs:boolean"/>

<xs:element name="current-location" type="sealloc:tCurrentLocationType"/>

<xs:element name="triggering-criteria" type="sealloc:TriggeringCriteriaType"/>

<xs:element name="minimum-interval-length" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="endpoint-info" type="sealloc:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="adaptive-report" type="sealloc:tAdaptiveReportType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="timestamp-indicator" type="xs:boolean"/>

<xs:simpleType name="tAdaptiveReportType">

<xs:restriction base="xs:string">

<xs:enumeration value="DIRECT UPDATE"/>

<xs:enumeration value="SUGGESTIVE UPDATE"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tLocationCapabilityType">

<xs:sequence>

<xs:element name="location-access-type" type="sealloc:tLocationAccessTypeType" minOccurs="0"/>

<xs:element name="positioning-method" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLocationQoSType">

<xs:sequence>

<xs:element name="hAccuracy" type="sealloc:tAccuracyType" minOccurs="0"/>

<xs:element name="vAccuracy" type="sealloc:tAccuracyType" minOccurs="0"/>

<xs:element name="vertRequested" type="xs:boolean" minOccurs="0"/>

<xs:element name="responseTime" type="sealloc:tResponseTimeType" minOccurs="0"/>

<xs:element name="minorLocQoses" type="sealloc:tMinorLocationQoSType" minOccurs="0"/>

<xs:element name="lcsQosClass" type="sealloc:tLcsQosClassType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMinorLocationQoSType">

<xs:choice>

<xs:element name="hAccuracy" type="sealloc:tAccuracyType" minOccurs="0"/>

<xs:element name="vAccuracy" type="sealloc:tAccuracyType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tRequestedLocationType">

<xs:sequence>

<xs:element name="current-serving-NCGI" type="sealloc:tEmptyType" minOccurs="0"/>

<xs:element name="neighbouring-NCGI" type="sealloc:tEmptyType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="mbms-service-area-id" type="sealloc:tEmptyType" minOccurs="0"/>

<xs:element name="mbsfn-area-id" type="sealloc:tEmptyType" minOccurs="0"/>

<xs:element name="current-geographical-coordinate" type="sealloc:tEmptyType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="VAL-user-id" type="sealloc:contentType" minOccurs="0"/>

<xs:element name="slpos-mgmt-params" type="sealloc:SlposMgmtParamsType" minOccurs="0"/>

<xs:complexType name="TriggeringCriteriaType">

<xs:sequence>

<xs:element name="cell-change" type="sealloc:tCellChange" minOccurs="0"/>

<xs:element name="tracking-area-change" type="sealloc:tTrackingAreaChangeType" minOccurs="0"/>

<xs:element name="plmn-change" type="sealloc:tPlmnChangeType" minOccurs="0"/>

<xs:element name="mbms-sa-change" type="sealloc:tMbmsSaChangeType" minOccurs="0"/>

<xs:element name="mbsfn-area-change" type="sealloc:tMbsfnAreaChangeType" minOccurs="0"/>

<xs:element name="periodic-report" type="sealloc:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="travelled-distance" type="sealloc:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="vertical-application-event" type="sealloc:tVerticalAppEventType" minOccurs="0"/>

<xs:element name="geographical-area-change" type="sealloc:tGeographicalAreaChange"/>

<xs:element name="valid-period" type="sealloc:tValidPeriod"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEmptyType"/>

<xs:complexType name="tCellChange">

<xs:sequence>

<xs:element name="any-cell-change" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-cell" type="sealloc:tSpecificCellType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-cell" type="sealloc:tSpecificCellType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tNcgi">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}[0-1]{28}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tSpecificCellType">

<xs:simpleContent>

<xs:extension base="sealloc:tNcgi">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tEmptyTypeAttribute">

<xs:complexContent>

<xs:extension base="sealloc:tEmptyType">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="tTrackingAreaChangeType">

<xs:sequence>

<xs:element name="any-tracking-area-change" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-tracking-area" type="sealloc:tTrackingAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-tracking-area" type="sealloc:tTrackingAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tTrackingAreaIdentityFormat">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}[0-1]{16}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tTrackingAreaIdentity">

<xs:simpleContent>

<xs:extension base="sealloc:tTrackingAreaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tPlmnChangeType">

<xs:sequence>

<xs:element name="any-plmn-change" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-plmn" type="sealloc:tPlmnIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-plmn" type="sealloc:tPlmnIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tPlmnIdentityFormat">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tPlmnIdentity">

<xs:simpleContent>

<xs:extension base="sealloc:tPlmnIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tMbmsSaChangeType">

<xs:sequence>

<xs:element name="any-mbms-sa-change" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-mbms-sa" type="sealloc:tMbmsSaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-mbms-sa" type="sealloc:tMbmsSaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tMbmsSaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMbmsSaIdentity">

<xs:simpleContent>

<xs:extension base="sealloc:tMbmsSaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tMbsfnAreaChangeType">

<xs:sequence>

<xs:element name="any-mbsfn-area-change" type="sealloc:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:element name="enter-specific-mbsfn-area" type="sealloc:tMbsfnAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-mbsfn-area" type="sealloc:tMbsfnAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tMbsfnAreaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMbsfnAreaIdentity">

<xs:simpleContent>

<xs:extension base="sealloc:tMbsfnAreaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tIntegerAttributeType">

<xs:simpleContent>

<xs:extension base="xs:integer">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tVerticalAppEventType">

<xs:sequence>

<xs:element name="initial-log-on" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="location-configuration-received" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="any-other-event" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tLocationAccessTypeType">

<xs:restriction base="xs:string">

<xs:enumeration value="3GPP_ACCESS"/>

<xs:enumeration value="NON_3GPP_ACCESS"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tCurrentLocationType">

<xs:sequence>

<xs:element name="current-serving-NCGI" type="sealloc:tLocationType" minOccurs="0"/>

<xs:element name="neighbouring-NCGI" type="sealloc:tLocationType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="mbms-service-area-id" type="sealloc:tLocationType" minOccurs="0"/>

<xs:element name="mbsfn-area-id" type="sealloc:tLocationType" minOccurs="0"/>

<xs:element name="current-geographical-coordinate" type="sealloc:tPointCoordinate" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tVelocityInfoType">

<xs:restriction base="xs:string">

<xs:pattern value="^[A-Fa-f0-9]{14}$"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tLocationType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="Ncgi" type="sealloc:tNcgi" minOccurs="0"/>

<xs:element name="SaId" type="sealloc:tMbmsSaIdentity" minOccurs="0"/>

<xs:element name="MbsfnAreaId" type="sealloc:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attribute name="type" type="sealloc:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tGeographicalAreaChange">

<xs:sequence>

<xs:element name="any-area-change" type="sealloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-area" type="sealloc:tSpecificAreaType" minOccurs="0"/>

<xs:element name="exit-specific-area" type="sealloc:tSpecificAreaType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tValidPeriod">

<xs:sequence>

<xs:element name="days-of-week" type="sealloc:tDayOfWeek" minOccurs="0" maxOccurs="6"/>

<xs:element name="time-of-day-start" type="xs:string" minOccurs="0"/>

<xs:element name="time-of-day-end" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tDayOfWeek">

<xs:restriction base="xs:string">

<xs:enumeration value="monday"/>

<xs:enumeration value="tuesday"/>

<xs:enumeration value="wednesday"/>

<xs:enumeration value="thursday"/>

<xs:enumeration value="friday"/>

<xs:enumeration value="saturday"/>

<xs:enumeration value="sunday"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tSpecificAreaType">

<xs:sequence>

<xs:element name="geographical-area" type="sealloc:tGeographicalAreaDef"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPointCoordinate">

<xs:sequence>

<xs:element name="longitude" type="sealloc:tCoordinateType"/>

<xs:element name="latitude" type="sealloc:tCoordinateType"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following element is added for extensibility and to be placed in the anyExt element above -->

<xs:element name="altitude" type="sealloc:tCoordinateType"/>

<xs:complexType name="tCoordinateType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="threebytes" type="sealloc:tThreeByteType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attribute name="type" type="sealloc:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tThreeByteType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="16777215"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tGeographicalAreaDef">

<xs:sequence>

<xs:element name="polygon-area" type="sealloc:tPolygonAreaType" minOccurs="0"/>

<xs:element name="ellipsoid-area" type="sealloc:tEllipsoidArcType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPolygonAreaType">

<xs:sequence>

<xs:element name="Corner" type="sealloc:tPointCoordinate" minOccurs="3" maxOccurs="15"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEllipsoidArcType">

<xs:sequence>

<xs:element name="Center" type="sealloc:tPointCoordinate"/>

<xs:element name="Radius" type="xs:nonNegativeInteger"/>

<xs:element name="OffsetAngle" type="xs:unsignedByte"/>

<xs:element name="IncludedAngle" type="xs:unsignedByte"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tReportsType">

<xs:sequence>

<xs:element name="loc-info-report" type="sealloc:tlocInfoReportType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="current-geographical-area" type="sealloc:tGeographicalAreaDef" minOccurs="0" maxOccurs="1"/>

<xs:element name="timestamp" type="xs:dateTime" minOccurs="0" maxOccurs="1"/>

<xs:complexType name="tlocInfoReportType">

<xs:sequence>

<xs:element name="VAL-user-id" type="sealloc:contentType" minOccurs="0"/>

<xs:element name="latest-location" type="sealloc:tLatestLocationType"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLatestLocationType">

<xs:sequence>

<xs:element name="latest-serving-NCGI" type="sealloc:tLocationType" minOccurs="0"/>

<xs:element name="neighbouring-NCGI" type="sealloc:tLocationType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="mbms-service-area-id" type="sealloc:tLocationType" minOccurs="0"/>

<xs:element name="mbsfn-area" type="sealloc:tLocationType" minOccurs="0"/>

<xs:element name="latest-coordinate" type="sealloc:tPointCoordinate" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="latest-geographical-area" type="sealloc:tGeographicalAreaDef" minOccurs="0" maxOccurs="1"/>

<xs:element name="timestamp" type="xs:dateTime" minOccurs="0" maxOccurs="1"/>

<xs:complexType name="contentType">

<xs:choice>

<xs:element name="sealURI" type="xs:anyURI"/>

<xs:element name="sealString" type="xs:string"/>

<xs:element name="sealBoolean" type="xs:boolean"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attribute name="type" type="sealloc:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tIDsListType">

<xs:choice maxOccurs="unbounded">

<xs:element name="VAL-user-id" type="sealloc:contentType" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tAccuracyType">

<xs:restriction base="xs:float">

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tResponseTimeType">

<xs:restriction base="xs:string">

<xs:enumeration value="LOW_DELAY"/>

<xs:enumeration value="DELAY_TOLERANT"/>

<xs:enumeration value="NO_DELAY"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tLcsQosClassType">

<xs:restriction base="xs:string">

<xs:enumeration value="BEST_EFFORT"/>

<xs:enumeration value="ASSURED"/>

<xs:enumeration value="MULTIPLE_QOS"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tAdaptiveIndType">

<xs:restriction base="xs:string">

<xs:enumeration value="ACCEPT"/>

<xs:enumeration value="REJECT"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tLocationReuseRequestType">

<xs:sequence>

xs:element name="target-VAL-user-id" type="sealloc:tIdentityType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="loc-reuse-ind" type="xs:boolean"/>

<xs:element name="current-location" type="sealloc:tReportType"/>

<xs:element name="valid-period" type="sealloc:tValidPeriod" minOccurs="0"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tVerificationType">

<xs:choice>

<xs:element name="current-coordinate" type="sealloc:tPointCoordinate" minOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tconfirmLocReportType">

<xs:choice>

<xs:element name="confirm-loc-status" type="xs:string" minOccurs="1"/>

<xs:element name="timestamp" type="xs:dateTime"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tOffNetworkLocPosConfigurationReqType">

<xs:sequence>

<xs:element name="identities-list" type="sealloc:tIDsListType"/>

<xs:element name="triggering-criteria" type="sealloc:TriggeringCriteriaType"/>

<xs:element name="history-loc-report-triggers" type="sealloc:tHistoryLocReportTriggersType"/>

<xs:element name="location-qos" type="sealloc:tLocationQoSType" minOccurs="0"/>

<xs:element name="requested-pos-method" type="sealloc:tPositioningMethodType" minOccurs="0"/>

<xs:element name="valid-period" type="sealloc:tValidPeriod" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:attribute name="requested-loc-type" type="xs:string" use="required" />

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tHistoryLocReportTriggersType">

<xs:sequence>

<xs:element name="history-loc-report-trigger " type="xs:string" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tOffNetworkLocPosConfigurationResType">

<xs:sequence>

<xs:element name="result" type="sealloc:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tResultType">

<xs:sequence>

<xs:element name="operation-result" type="sealloc:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="subscription-identifier" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tOperationResultType">

<xs:restriction base="xs:string">

<xs:enumeration value="success"/>

<xs:enumeration value="failure"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tHistoryLocResulReportType">

<xs:sequence>

<xs:element name="subscription-identifier" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="reports" type="sealloc:tReportType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSrPosInfoReqType">

<xs:sequence>

<xs:element name="identities-list" type="sealloc:tIDsListType"/>

<xs:element name="srpos-filter" type="sealloc:tSrPosFilterType" minOccurs="1"/>

<xs:element name="location-QoS" type="sealloc:tLocationQoSType" minOccurs="0"/>

<xs:element name="expiry-time" type="xs:nonPositiveInteger" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tSrPosFilterType">

<xs:restriction base="xs:string">

<xs:enumeration value="RANGE"/>

<xs:enumeration value="DIRECTION"/>

<xs:enumeration value="RELATIVE_POS"/>

<xs:enumeration value="RELATIVE_VEL"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tRangeDirectionType">

<xs:sequence>

<xs:attribute name="distance" type="xs:string" minOccurs="0"/>

<xs:attribute name="azimuth-direction" type="unsignedByte" minOccurs="0"/>

<xs:attribute name="elevation-direction" type="unsignedByte" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SlposMgmtParamsType">

<xs:sequence>

<xs:element name="distanceInd" type="xs:boolean" minOccurs="0"/>

<xs:element name="directionInd" type="xs:boolean" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

* * * Next Change * * * *

## 7.5	Data semantics

Editor's note (WIC: eLSAPP, CR 0170):	Data semantics updates for the location positioning subscription procedure are FFS.

Editor's note (WIC: eLSAPP, CR 0171):	Data semantics updates for the location positioning notification procedure are FFS.

The <location-info> element is the root element of the XML document. The <location-info> element contains the <identity>, <subscription>, <report>, <request>, <request-identity>, <configuration>, <report-request>, <location-based-query>, <location-based-response>, <location-capability>, <location-QoS>, <adaptive-configuration> <adaptive-ind>, <failure-cause>, <off-network-loc-pos-configuration-req>, <off-network-loc-pos-configuration-res>, <history-loc-result-report>, <verification> and <confirm-loc-report> sub-elements

<identity> is a mandatory element used to include the identity of a VAL user, a VAL client or a VAL group. The <identity> element contains one of following sub-elements:

a)	<VAL-user-id>, an element contains the identity of the VAL user. This element contains an optional <VAL-client-id> attribute that contains the identity of the VAL client; or

b)	<VAL-group-id>, an element contains the group identity of a set of VAL users or VAL clients according to the VAL service.

<subscription> contains the following sub-elements:

a)	<identities-list>, an element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user whose location information is requested;

b)	<time-interval-length>, an element specifying the interval time the SLM-S needs to wait before sending location reports. The value is given in seconds;

c)	<subscription-identifier>, an element specifying the value to uniquely identify the subscription;

d)	<expiry-time>, an element specifying expiry time for subscription in seconds;

e)	<location-QoS>, an element specifying the location QoS as specified in TS 29.572 [33] clause 6.1.6.2.13 contains the following sub-elements:

1)	a <hAccuracy> element;

2)	a <vAccuracy> element;

3)	a <vertRequested> element;

4)	a <responseTime> element;

5)	a <minorLocQoses> element; or

6)	a <lcsQosClass> element;

f)	<suppl-loc-info-ind>, an element specifying that supplementary location information is required;

g)	<requested-velocity-info> in an <anyExt> element, an optional element if the velocity information is required;

h)	<loc-data-statistic-ind>, an optional element specifying whether the statistic or calculation of target UE location data is needed per time or location;

i)	<req-time-info>, an optional element set to the time information when the target UE location data is calculated per time;

j)	<req-loc-info>, an optional element set to the location information when the target UE location data is calculated per location; and

k)	<confirm>, an optional element if the confirm location service is requested.

<notification> contains the following sub-elements:

a)	<identities-list>, an element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user whose location information needs to be notified;

b)	<trigger-id>, an element which can occur multiple times that contains the value of the <trigger-id> attribute associated with a trigger that has fired;

c)	<reports>, an element may contain one or more <loc-info-report> elements. Each <loc-info-report> element contains the following sub-elements:

1)	<VAL-user-id>, an element contains the identity of a VAL user in the identities list; and

2)	<latest-location >, an element contains at least one of the following sub-elements:

i)	<latest-serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

ii)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

iii)	<mbms-service-area-id>, an optional element containing the MBMS service area id the SLM-C is using coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI);

iv)	<mbsfn-area> element, an optional element specifying that the MBSFN area Id needs to be reported;

v)	<latest-coordinate>, an optional element containing the longitude, latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3] and altitude coded as specified in clause 6.3 in 3GPP TS 23.032 [3];

vi)	<latest-geographical-area>, an optional element containing containing the following two subelements:

A)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [2]; and

B)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [2]; and

3)	<timestamp>, an option element set to the timestamp in date and time of the location report with an offset from the UTC time; and

d)	<subscription-identifier> an optional element set to the subscription identifier value which uniquely identifies the subscription against which the notification shall be processed;

e)	<velocity-info>, an optional element in the <anyExt> element set to the velocity of the target UE for which the location information is requested, consisting of a 7 byte-long string of 14 hexadecimal digits which encode the binary content of the bearing, horizontal velocity and vertical velocity, as well as horizontal and vertical speed uncertainties of the SLM-C, according to clause 8.15 of 3GPP TS 23.032 [3], where the spare bits are set to 0;

f)	<loc-data-statistic>, an optional element set to the statistic result of target UE location data per temporal or spatial granularity as requested; and

g)	<confirm>, an optional element if the confirm location service is requested.

<report> is a mandatory element used to include the location report. It contains a <report-id> attribute. The <report-id> attribute is used to return the value in the <request-id> attribute in the <request> element. If the <report> element is used for the short-range based positioning information procedure, the <report> element shall include at least one of the following sub-elements: <current-coordinate>, <velocity-info> or <range-direction> element. The <report> element contains the following sub-elements:

a)	<trigger-id>, a mandatory element which can occur multiple times that contain the value of the <trigger-id> attribute associated with a trigger that has fired; and

b)	<current-location>, a mandatory element that contains the location information. The <current-location> element contains the following sub-elements:

1)	<current-serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

2)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

3)	<mbms-service-area-id>, an optional element containing the MBMS service area id the SLM-C is using coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI);

4)	<current-coordinate>, an optional element containing the longitude, latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3] and altitude coded as as specified in clause 6.3 in 3GPP TS 23.032 [3];

5)	<current-geographical-area>, an optional element containing a <trigger-id> attribute and the following two subelements:

A)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [2]; and

B)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [2]; and

c)	<timestamp>, an option element set to the timestamp in date and time of the location report with an offset from the UTC time;

d)	<velocity-info>, an optional element in the <anyExt> element set to the velocity of the target UE for which the location information is requested, consisting of a 7 byte-long string of 14 hexadecimal digits which encode the binary content of the bearing, horizontal velocity and vertical velocity, as well as horizontal and vertical speed uncertainties of the SLM-C, according to clause 8.15 of 3GPP TS 23.032 [3], where the spare bits are set to 0.

e)	<range-direction>, an optional element in the <anyExt> element set to the distance and direction between the reference UE and SLM client UE. The <range-direction> element contains at least one of the following sub-elements:

1)	<distance>, an element set to the distance between the requested UE and SLM client UE;

2)	<azimuth-direction>, an element set to the angle of azimuth direction angle between the requested UE and SLM client UE; and

3)	<elevation-direction>, an element set to the angle of elevation direction angle between the requested UE and SLM client UE.

NOTE 1:	The <azimuth-direction> element and the <elevation-direction> element are according to clause 7.10.10.6.2.10 of 3GPP TS 29.549 [18].

<request> is an element with a <request-id> attribute. The <request> element is used to request a location report. The value of the <request-id> attribute is returned in the corresponding <report-id> attribute in order to correlate the request and the report. The <request> element may include the <requested-velocity-info> in an <anyExt> element if the velocity of the VAL users/UEs is requested. The <request> may include the following sub-elemtents:

- <location-reuse-request> in an <anyExt> element to enable/disable the reuse of location of the UE for another UE.

- <requested-loc-access-type> in the <anyExt> element, an optional element specifying the identities of the location access type for which the location information is requested;

- <requested-pos-method> in the <anyExt> element, an optional element specifying the identities of the positioning method for which the location information is requested;

d)	<requested-velocity-info> in the <anyExt> element, an optional element specifying the velocity of the target UE for which the location information is requested;

e)	<verification> element in the <anyExt> element, an optional element including the location information which needs to be verified; and

f)	<srpos-info-req>, an optional element in an <anyExt> element to request the short-range based positioning information. The <srpos-info-req> element has the sub-elements:

1)	<identities-list>, an element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user whose short-range based positioning information is requested;

2)	<expiry-time>, an optional element specifying expiry time for subscription in seconds;

3)	<location-QoS>, an optional element specifying the location QoS; and

4)	<srpos-filter>, an element to indicate the requested short-range based positioning information.

<requested-identity> is a mandatory element used to include the identity of a VAL user, a VAL client or a VAL group for which a location report is requested. The <requested-identity> element contains one of following sub-elements:

a)	<VAL-user-id>, an element contains the identity of the VAL user. This element contains an optional <VAL-client-id> attribute that contains the identity of the VAL client; or

b)	<VAL-group-id>, an element contains the group identity of a set of VAL users or VAL clients according to the VAL service.

The <request> element may include the <verification> in an <anyExt> element to include the location information which needs to be verified. The <verification> element contains the following sub-elements:

a)	<current-coordinate> element, a mandatory element containing the longitude, latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3] and altitude coded as as specified in clause 6.3 in 3GPP TS 23.032 [3].

<configuration> is an element with a <configuration-scope> attribute that can have the value "Full" or "Update" . The value "Full" means that the <configuration> element contains the full location configuration which replaces any previous location configuration. The value "Update" means that the location configuration is an addition to any previous location configuration. To remove configuration elements a "Full" configuration is needed. The <configuration> element contains the following sub-elements:

a)	<location-information>, an optional element that specifies the location information. The <location-information> has the sub-elements:

1)	<serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

2)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

3)	<mbms-service-area-id>, an optional element containing the MBMS service area id that the SLM-C is using. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI);

4)	<mbsfn-area-id>, an optional element specifying that the MBSFN area id that needs to be reported;

5)	<current-geographical-coordinate>, an optional element containing the longitude, latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3] and altitude coded as specified in clause 6.3 in 3GPP TS 23.032 [3];

6)	<VAL-user-id> in an <anyExt> element, an optional element contains the identity of the VAL user; and

b)	<triggering-criteria>, an optional element specifying the triggers for the SLM-C to request a location report of a VAL user, a VAL client or a VAL group. The <triggering-criteria> element contains at least one of the following sub-elements:

1)	<cell-change>, an optional element specifying what cell changes trigger the request for a location report. This element consists of the following sub-elements:

i)	<any-cell-change>, an optional element. The presence of this element specifies that any cell change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-cell>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-cell>, an optional element specifying an NCGI which when exited triggers a request for a location report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<tracking-area-change>, an optional element specifying what tracking area changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-tracking-area-change>, an optional element. The presence of this element specifies that any tracking area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when exited triggers a request for alocation report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

3)	<plmn-change>, an optional element specifying what PLMN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-plmn-change>, an optional element. The presence of this element specifies that any PLMN change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when exited triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

4)	<mbms-sa-change>, an optional element specifying what MBMS changes trigger location reporting. This element consists of the following sub-elements:

i)	<any-mbms-sa-change>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbms-sa>, an optional element specifying an MBMS service area id which when entered triggers a request for a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbms-sa>, an optional element specifying an MBMS service area id which when exited triggers a request a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

5)	<mbsfn-area-change>, an optional element specifying what MBSFN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-mbsfn-area-change>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbsfn-area>, an optional element specifying an MBSFN area which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbsfn-area>, an optional element specifying an MBSFN area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

6)	<periodic-report>, an optional element specifying that periodic request for a location report shall be sent. The value in seconds specifies the reporting interval. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

7)	<travelled-distance>, an optional element specifying that the travelled distance shall trigger a request for a location report. The value in metres specified the travelled distance. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

8)	<vertical-application-event>, an optional element specifying what application signalling events triggers a request for a location report. The <vertical-application-event> element has the following sub-elements:

i)	<initial-log-on>, an optional element specifying that an initial log on triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<location-configuration-received>, an optional element specifying that a received location configuration triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<any-other- event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	<geographical-area-change>, an optional element specifying what geographical are changes trigger a request for a location reporting. This element consists of the following sub-elements:

i)	<any-area-change>, an optional element. The presence of this element specifies that any geographical area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-area>, an optional element specifying a geographical area which when entered triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <enter-specific-area> element has the following sub-elements:

A)	<geographical-area>, an optional element containing a <trigger-id> attribute and the following two subelements:

I)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [2]; and

II)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [2]; and

iii)	<exit-specific-area-type>, an optional element specifying a geographical area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

10)	<valid-period>, an optional element specifying the scheduled time intervals for the reporting in form of day of the week or time period or both. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <valid-period> element has the following sub-elements:

i)	<days-of-week>, an optional element containing a <day-of-week> attribute indicating the day(s) of the week. If absent, it indicates every day of the week;

ii)	<time-of-day-start>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35]; and

iii)	<time-of-day-end>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35];

c)	<minimum-interval-length>, a mandatory element specifying the minimum time the SLM-C needs to wait between sending location reports. The value is given in seconds;

d)	<requested-loc-access-type>, an optional element specifying the identities of the location access type for which the location information is requested;

e)	<requested-pos-method>, an optional element specifying the identities of the positioning method for which the location information is requested. This element is a string. Table 7.5-1 provides the valid values of identities of the positioning method; and

Table 7.5-1: Requested positioning method

| Value | Description |
| --- | --- |
| CELLID | Cell ID positioning method |
| ECID | Enhanced cell ID methods based on LTE signals |
| OTDOA | Observed time difference of arrival positioning based on LTE signals |
| BAROMETRIC_PRESSURE | Positioning method based on barometric Pressure Sensor |
| WLAN | WLAN positioning |
| BLUETOOTH | Bluetooth positioning |
| MBS | Terrestrial Beacon System (TBS) positioning based on MBS signals |
| MOTION_SENSOR | Positioning method based on motion Sensor |
| DL_TDOA | Downlink Time Difference of Arrival (DL-TDOA) based on NR signals |
| DL_AOD | Downlink Angle-of-Departure (DL-AoD) based on NR signals |
| MULTI-RTT | Multi-Round Trip Time Positioning (Multi-RTT based on NR signals). |
| NR_ECID | NR enhanced cell ID methods (NR E-CID) based on NR signals. |
| UL_TDOA | Uplink Time Difference of Arrival (UL-TDOA) based on NR signals |
| UL_AOA | Uplink Angle of Arrival (UL-AoA), including the Azimuth of Arrival (A-AoA) and the Zenith of Arrival (Z-AoA) based on NR signals. |
| Other value shall be ignored. |  |

NOTE 2:	The values of identities of the positioning method correspond to the values specified in 3GPP TS 29.572 [33] clause 6.1.6.3.6.

The recipient of the XML ignores any unknown element and any unknown attribute.

f)	<requested-velocity-info>, an optional element if velocity of the target UE is requested; and

g)	<slpos-mgmt-params> in an <anyExt> element, an optional element contains at least one of the following sub-elements:

1)	<distanceInd>, an optional element to indicate whether the distance between target UEs and reference UEs is reported or not; and

2)	<directionInd>, an optional element to indicate whether the direction between target UEs and reference UEs is reported or not.

<report-request> is a mandatory element used to include the requested location report. The <report-request> element contains the following sub-elements:

a)	<immediate-report-indicator>, indicates whether an immediate location report is required;

b)	<current-location>, an optional element that contains the location information. The <current-location> element contains the following sub-elements:

1)	<current-serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

2)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

3)	<mbms-service-area-id>, an optional element containing the MBMS service area id that the SLM-C is using. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI); and

4)	<current-coordinate>, an optional element containing the longitude, latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3] and altitude coded as as specified in clause 6.3 in 3GPP TS 23.032 [3].

c)	<triggering-criteria>, a mandatory element specifying the triggers for the SLM-C to request a location report of a VAL user, a VAL client or a VAL group. The <triggering-criteria> element contains at least one of the following sub-elements:

1)	<cell-change>, an optional element specifying what cell changes trigger the request for a location report. This element consists of the following sub-elements:

i)	<any-cell-change>, an optional element. The presence of this element specifies that any cell change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-cell>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-cell>, an optional element specifying an NCGI which when exited triggers a request for a location report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<tracking-area-change>, an optional element specifying what tracking area changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-tracking-area-change>, an optional element. The presence of this element specifies that any tracking area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when exited triggers a request for alocation report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

3)	<plmn-change>, an optional element specifying what PLMN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-plmn-change>, an optional element. The presence of this element specifies that any PLMN change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when exited triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

4)	<mbms-sa-change>, an optional element specifying what MBMS changes trigger location reporting. This element consists of the following sub-elements:

i)	<any-mbms-sa-change>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbms-sa>, an optional element specifying an MBMS service area id which when entered triggers a request for a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbms-sa>, an optional element specifying an MBMS service area id which when exited triggers a request a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

5)	<mbsfn-area-change>, an optional element specifying what MBSFN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-mbsfn-area-change>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbsfn-area>, an optional element specifying an MBSFN area which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbsfn-area>, an optional element specifying an MBSFN area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

6)	<periodic-report>, an optional element specifying that periodic request for a location report shall be sent. The value in seconds specifies the reporting interval. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

7)	<travelled-distance>, an optional element specifying that the travelled distance shall trigger a request for a location report. The value in metres specified the travelled distance. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

8)	<vertical-application-event>, an optional element specifying what application signalling events triggers a request for a location report. The <vertical-application-event> element has the following sub-elements:

i)	<initial-log-on>, an optional element specifying that an initial log on triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<location-configuration-received>, an optional element specifying that a received location configuration triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<any-other- event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	<geographical-area-change>, an optional element specifying what geographical are changes trigger a request for a location reporting. This element consists of the following sub-elements:

i)	<any-area-change>, an optional element. The presence of this element specifies that any geographical area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-area>, an optional element specifying a geographical area which when entered triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <enter-specific-area> element has the following sub-elements:

A)	<geographical-area>, an optional element containing a <trigger-id> attribute and the following two subelements:

I)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [3]; and

II)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [3]; and

iii)	<exit-specific-area-type>, an optional element specifying a geographical area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

10)	<valid-period>, an optional element specifying the scheduled time intervals for the reporting in form of day of the week or time period or both. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <valid-period> element has the following sub-elements:

i)	<days-of-week>, an optional element containing a <day-of-week> attribute indicating the day(s) of the week. If absent, it indicates every day of the week;

ii)	<time-of-day-start>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35]; and

iii)	<time-of-day-end>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35];

d)	<minimum-interval-length>, an optional element has a that default value of 0 if absent, otherwise it indicates the interval time between consecutive reports. The value is given in seconds;

e)	<endpoint-info>, an optional element specifying information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if the immediate report indicator is set to required;

f)	<timestamp-indicator>, an optional element which presence indicates that timetamp of the location report is required; and

g)	<adaptive-report> , an optional element specifying the request for an adaptive location reporting by dynamically adjusting the configuration and may indicate "DIRECT UPDATE" or "SUGGESTIVE UPDATE".

At least one of the bullet a to d above shall be present in the <report-request> element.

<location-based-query> contains at least one of the following sub-elements:

a)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [3]; and

b)	<ellipsoid-arc-area>, an optional element specifying the area as an Ellipsoid Arc specified in clause 5.7 in 3GPP TS 23.032 [3].

<geofenc-based-query>, an optional element contains at least one of the following sub-elements:

a)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [3]; and

b)	<ellipsoid-arc-area>, an optional element specifying the area as an Ellipsoid Arc specified in clause 5.7 in 3GPP TS 23.032 [3].

<location-based-response> contains the following sub-elements:

a)	<identities-list>, an optional element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user to be queried.

<location-capability> contains the following sub-elements:

a)	<access-type>, an optional element specifying the identity of the available access type of the VAL UE; and

b)	<positioning-method>, an optional element specifying the identity of the available positioning methods of the VAL UE.

<location-QoS> element is an optionally element used to indicate the location Quality of Service as specified in TS 29.572 [33] clause 6.1.6.2.13 for which the location information is requested and it may contain the following sub-elements:

a)	a <hAccuracy> element;

b)	a <vAccuracy> element;

c)	a <vertRequested> element;

d)	a <responseTime> element;

e)	a <minorLocQoses> element; or

f)	a <lcsQosClass> element.

<adaptive-configuration> element is an optionally element used to provide the proposed adaptive location configuration for the SLM-C to request a location report and it shall include:

a)	<cell-change>, an optional element specifying what cell changes trigger the request for a location report. This element consists of the following sub-elements:

1)	<any-cell-change>, an optional element. The presence of this element specifies that any cell change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-cell>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-cell>, an optional element specifying an NCGI which when exited triggers a request for a location report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

b)	<tracking-area-change>, an optional element specifying what tracking area changes trigger a request for a location report. This element consists of the following sub-elements:

1)	<any-tracking-area-change>, an optional element. The presence of this element specifies that any tracking area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when exited triggers a request for alocation report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

c)	<plmn-change>, an optional element specifying what PLMN changes trigger a request for a location report. This element consists of the following sub-elements:

1)	<any-plmn-change>, an optional element. The presence of this element specifies that any PLMN change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when exited triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

d)	<mbms-sa-change>, an optional element specifying what MBMS changes trigger location reporting. This element consists of the following sub-elements:

1)	<any-mbms-sa-change>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-mbms-sa>, an optional element specifying an MBMS service area id which when entered triggers a request for a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-mbms-sa>, an optional element specifying an MBMS service area id which when exited triggers a request a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

e)	<mbsfn-area-change>, an optional element specifying what MBSFN changes trigger a request for a location report. This element consists of the following sub-elements:

1)	<any-mbsfn-area-change>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-mbsfn-area>, an optional element specifying an MBSFN area which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-mbsfn-area>, an optional element specifying an MBSFN area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

f)	<periodic-report>, an optional element specifying that periodic request for a location report shall be sent. The value in seconds specifies the reporting interval. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

g)	<travelled-distance>, an optional element specifying that the travelled distance shall trigger a request for a location report. The value in metres specified the travelled distance. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

h)	<vertical-application-event>, an optional element specifying what application signalling events triggers a request for a location report. The <vertical-application-event> element has the following sub-elements:

1)	<initial-log-on>, an optional element specifying that an initial log on triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<location-configuration-received>, an optional element specifying that a received location configuration triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<any-other- event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

i)	<geographical-area-change>, an optional element specifying what geographical are changes trigger a request for a location reporting. This element consists of the following sub-elements:

1)	<any-area-change>, an optional element. The presence of this element specifies that any geographical area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-area>, an optional element specifying a geographical area which when entered triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <enter-specific-area> element has the following sub-elements:

i)	<geographical-area>, an optional element containing a <trigger-id> attribute and the following two subelements:

A)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [2]; and

B)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [2]; and

3)	<exit-specific-area-type>, an optional element specifying a geographical area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

j)	<valid-period>, an optional element specifying the scheduled time intervals for the reporting in form of day of the week or time period or both. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <valid-period> element has the following sub-elements:

1)	<days-of-week>, an optional element containing a <day-of-week> attribute indicating the day(s) of the week. If absent, it indicates every day of the week;

2)	<time-of-day-start>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35]; and

3)	<time-of-day-end>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35].

<adaptive-ind>, indicates whether the adaptive configuration is accepted.

<location-reuse-request> contains the following sub-elements:

a)	<target-VAL-user-id> element contains the identity(s) of the VAL user(s) whose current location shall be reused with location information of the VAL user triggering the location reuse procedure;

b)	<loc-reuse-ind> element indicating whether to enable or disable location reuse or whether the location can be sharing;

c)	<current-location>, an optional element includes current location of the UE; and

d)	<valid-period>, an optional element specifying the scheduled time intervals for the reporting in form of day of the week or time period or both. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <valid-period> element has the following sub-elements:

1)	<days-of-week>, an optional element containing a <day-of-week> attribute indicating the day(s) of the week. If absent, it indicates every day of the week;

2)	<time-of-day-start>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35]; and

3)	<time-of-day-end>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35].

<failure-cause>, an optional element indicating the reason of failure. The <failure-cause> element is a strng. Table 7.5-2

provides the valid reasons of failure.

Table 7.5-2: Failure

| Value | Description |
| --- | --- |
| MOVED_AWAY | The target VAL UE has moved away. |
| UNSUPPORTE_POSITIONING_METHOD | The required positioning method is not supported by target VAL UE. |
| NO RESULT | No result is obtained for the target VAL UE after performing the short-range based positioning information procedure. |
| Other value shall be ignored. |  |

<off-network-loc-pos-configuration-req> contains the following sub-elements:

a)	<identities-list>, an element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user whose history location result report is requested after they have been off network (i.e., loss of network connectivity);

b)	<requested-loc-type>, a mandatory element indicating the type of location which is requested. This element is a string. Table 7.5-3 provides the valid values of the triggers for reporting the history location results:

Table 7.5-3: requested-loc-type

| Value | Description |
| --- | --- |
| ABSOLUTE | The location requested is absolute (i.e., geographical coordinates). |
| RELATIVE | The location requested is relative (i.e., geographical area). |
| Other values shall be ignored. |  |

c)	<triggering-criteria>, a mandatory element specifying the triggers for the SLM-C to initiate location positioning of a VAL user, a VAL client or a VAL group when off network (i.e., loss of network connectivity);

d)	<history-loc-report-triggers>, a mandatory element that contains one or more <loc-report-trigger> elements. Each <loc-report-trigger> element is a string. Table 7.5-4 provides the valid values of the triggers for reporting the history location results:

Table 7.5-4: loc-report-trigger

| Value | Description |
| --- | --- |
| UU CONNECTIVITY IS BACK | The target VAL UE/VAL user regains Uu connectivity. |
| RESTART | The target VAL UE/VAL user restarts. |
| Other values shall be ignored. |  |

e)	<location-QoS>, an optional element specifying the location QoS and contains the following sub-elements:

1)	a <hAccuracy> element;

2)	a <vAccuracy> element;

3)	a <vertRequested> element;

4)	a <responseTime> element;

5)	a <minorLocQoses> element;or

6)	a <lcsQosClass> element;

f)	<requested-pos-method>, an optional element specifying the identities of the positioning method for which the location positioning information is requested; and

g)	<valid-period>, an optional element specifying the scheduled time intervals for the initiation of location positioning when in off network in form of day of the week or time period or both. The <valid-period> element has the following sub-elements:

1)	<days-of-week>, an optional element containing a <day-of-week> attribute indicating the day(s) of the week. If absent, it indicates every day of the week; and

2)	<time-of-day-start>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35].

3)	<time-of-day-end>, an optional element containing a <time-of-day> attribute that shall be a string with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [35];

<off-network-loc-pos-configuration-res> contains the following sub-elements:

a)	<result>, a mandatory element set to either "success" or "failure" " indicating success or failure of the operation. If the result is "success", this element shall contain the following sub-element:

1)	a <subscription-identifier>, an element specifying the value to uniquely identify the subscription.

<history-loc-result-report> contains the following sub-elements:

a)	<subscription-identifier>, a mandatory element specifying the value to uniquely identify the subscription; and

b)	<reports>, a mandatory element specifying location information.

<confirm-loc-report>, an optional element contains the following sub-elements:

a)	<confirm-loc-status> element see table 7.5-5, an optional element containing the result of comparing location information from the network; and

Table 7.5-5: Confirm-loc-status

| Value | Description |
| --- | --- |
| UNKNOWN | The server doesn’t know the result of comparing location information for the VAL UE |
| MISMATCH IN SAME COUNTRY | The comparing location information for the VAL UE is not mached in same country |
| MISMATCH IN OTHER COUNTRY | The comparing location information for the VAL UE is not mached in other country |
| ROAMING COUNTRY MISMATCH | The comparing location information for the VAL UE is not mached in roaming country. |
| ERROR UNAUTHORIZED | The comparing location information server is not authorized |
| ERROR OTHER | Other error |
| Other value shall be ignored. |  |

b)	shall include a <timestamp> element set to the timestamp in date and time of the confirm location report with an offset from the UTC time.

The recipient of the XML ignores any unknown element and any unknown attribute.

* * * End of Changes * * * *