3GPP TSG-CT WG1 Meeting #157	C1-256827

# Sophia Antipolis, France 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0104 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fixing the CDDL extensibility issue for Sdd_TransmissionQualityManagement API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current CDDL document for TxQualityManagementRequest and TxQualityManagementResponse part of Sdd_TransmissionQualityManagement API is missing the extensibility feature as explained in RFC 8610 section 3.5.1. CBOR document compliant to Rel-19 CDDL schema with new optional parameters (batOffsetUl and periodicityUl) does not validate against Rel-18 CCDL schema. The extensibility issue is resolved if we add "* tstr => any" into CDDL schema. A.5.3 states: ----------- Interoperability considerations: Applications must ignore any key-value pairs that they do not understand. This allows backwards-compatible extensions to this specification. ----------- However, the current CDDL schema requires the parser in the receiver to generate an error while parsing and validating CBOR document with a not understood key-value pair |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding "* tstr => any" to provide extensibility mechanism to the existing TxQualityManagementRequest and TxQualityManagementResponse part of Sdd_TransmissionQualityManagement API. Backward compatibility analysis: This CR introduces backward incompatible correction to the Sdd_TransmissionQualityManagement API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Extensiblity mechanism will not be added. Interoperability issues with other releases. Not possible to add additional parameters to the CDDL schema. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### A.3.3.5.2	CDDL document

;;; TxQualityManagementRequest

;;+ Represents a request for performing SDDM data transmission quality management.

TxQualityManagementRequest = {

sealddFlowId: Uinteger

txQualityManagementAction: string

? batOffsetUl: Uinteger

? periodicityUl: Uinteger

* tstr => any

}

;;; TxQualityManagementResponse

;;+ Represents the response of a request for performing SDDM data transmission quality management.

TxQualityManagementResponse = {

result: ResultOp

? cause: Cause

* tstr => any

}

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

# *** End of Changes ***