3GPP TSG-CT WG1 Meeting #157	C1-256670

# Sophia-Antipolis, France 13–17 October 2025									 (was C1-256339)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4562 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TAU update in MINT-EPS. |
| --- | --- |
| Source to WG: | Nokia, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new code point of the EPS update type IE: However, there is no IE defined for UE sending “UE determined PLMN with disaster condition” in the TAU request message - the message definition for the IE is added in 8.2.29 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New IE for UE determined PLMN with disaster condition in the TAU request message. The new message definition for the IE is added in 8.2.29.x |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The EPS-type update code point will remain unused in TAU procedure - Ambiguity shall remain without the changes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.29.1, 8.2.29.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Reworded first change Reverted changes to disaster wait range Updated cover page Rev 2: - Included UE determined PLMN with disaster condition in the TAU request message IE in 8.2.29.1, - updated changes in 5.5.3.2.4 for better clarification - Added co-signer Rev 3: Removed 5.5.3.2.4 changes completely, updated cover page |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 8.2.29.1	Message definition

The purposes of sending the tracking area update request by the UE to the network are described in clause 5.5.3.1. See table 8.2.29.1.

Message type:	TRACKING AREA UPDATE REQUEST

Significance:	dual

Direction:	UE to network

Table 8.2.29.1: TRACKING AREA UPDATE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | Tracking area update request message identity | Message type 9.8 | M | V | 1 |
|  | EPS update type | EPS update type 9.9.3.14 | M | V | 1/2 |
|  | NAS key set identifier | NAS key set identifier 9.9.3.21 | M | V | 1/2 |
|  | Old GUTI | EPS mobile identity 9.9.3.12 | M | LV | 12 |
| B- | Non-current native NAS key set identifier | NAS key set identifier 9.9.3.21 | O | TV | 1 |
| 8- | GPRS ciphering key sequence number | Ciphering key sequence number 9.9.3.4a | O | TV | 1 |
| 19 | Old P-TMSI signature | P-TMSI signature 9.9.3.26 | O | TV | 4 |
| 50 | Additional GUTI | EPS mobile identity 9.9.3.12 | O | TLV | 13 |
| 55 | NonceUE | Nonce 9.9.3.25 | O | TV | 5 |
| 58 | UE network capability | UE network capability 9.9.3.34 | O | TLV | 4-15 |
| 52 | Last visited registered TAI | Tracking area identity 9.9.3.32 | O | TV | 6 |
| 5C | DRX parameter | DRX parameter 9.9.3.8 | O | TV | 3 |
| A- | UE radio capability information update needed | UE radio capability information update needed 9.9.3.35 | O | TV | 1 |
| 57 | EPS bearer context status | EPS bearer context status 9.9.2.1 | O | TLV | 4 |
| 31 | MS network capability | MS network capability 9.9.3.20 | O | TLV | 4-10 |
| 13 | Old location area identification | Location area identification 9.9.2.2 | O | TV | 6 |
| 9- | TMSI status | TMSI status 9.9.3.31 | O | TV | 1 |
| 11 | Mobile station classmark 2 | Mobile station classmark 2 9.9.2.4 | O | TLV | 5 |
| 20 | Mobile station classmark 3 | Mobile station classmark 3 9.9.2.5 | O | TLV | 2-34 |
| 40 | Supported Codecs | Supported Codec List 9.9.2.10 | O | TLV | 5-n |
| F- | Additional update type | Additional update type 9.9.3.0B | O | TV | 1 |
| 5D | Voice domain preference and UE's usage setting | Voice domain preference and UE's usage setting 9.9.3.44 | O | TLV | 3 |
| E- | Old GUTI type | GUTI type 9.9.3.45 | O | TV | 1 |
| D- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| C- | MS network feature support | MS network feature support 9.9.3.20A | O | TV | 1 |
| 10 | TMSI based NRI container | Network resource identifier container 9.9.3.24A | O | TLV | 4 |
| 6A | T3324 value | GPRS timer 2 9.9.3.16 | O | TLV | 3 |
| 5E | T3412 extended value | GPRS timer 3 9.9.3.16B | O | TLV | 3 |
| 6E | Extended DRX parameters | Extended DRX parameters 9.9.3.46 | O | TLV | 3 |
| 6F | UE additional security capability | UE additional security capability 9.9.3.53 | O | TLV | 6 |
| 6D | UE status | UE status 9.9.3.54 | O | TLV | 3 |
| 17 | Additional information requested | Additional information requested 9.9.3.55 | O | TV | 2 |
| 32 | N1 UE network capability | N1 UE network capability 9.9.3.57 | O | TLV | 3-15 |
| 34 | UE radio capability ID availability | UE radio capability ID availability 9.9.3.58 | O | TLV | 3 |
| 35 | Requested WUS assistance information | WUS assistance information 9.9.3.62 | O | TLV | 3 |
| 36 | DRX parameter in NB-S1 mode | NB-S1 DRX parameter 9.9.3.63 | O | TLV | 3 |
| 38 | Requested IMSI offset | IMSI offset 9.9.3.64 | O | TLV | 4 |
| 29 | UE request type | UE request type 9.9.3.65 | O | TLV | 3 |
| 28 | Paging restriction | Paging restriction 9.9.3.66 | O | TLV | 3-5 |
| 30 | Unavailability information | Unavailability information 9.9.3.69 | O | TLV | 3-9 |
| 26 | UE determined PLMN with disaster condition | PLMN identity 9.9.3.77 | O | TLV | 5 |

* * * Next Change * * *

#### 8.2.29.x	UE determined PLMN with disaster condition

The UE shall include this IE only if the UE initiates a tracking area update procedure with update type "disaster roaming update" when the UE needs to indicate a UE determined PLMN with disaster condition to the network.

* * * End of Change * * *