3GPP TSG-CT WG4 Meeting #131	C4-254316

Sophia Antipolis, France; 13th – 17th October 2025	was C4-254189

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0913 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Monitoring Info for inter-I/V-SMF mobility |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  |  | 2025-10-15 |  |
| Category: | A |  |  |  |  |  |  | Release: |  |  |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |
| Reason for change: | The information element QosMonitoringInfo is only included in the VsmfUpdateData sent from the (H-)SMF to V/I-SMF and this IE is used to tell if QoS monitoring is performed using end to end accumulated packet delay reporting in UL GTP-U packets (see clause 5.33.3.3 of 3GPP TS 23.501 [2]) or QoS monitoring is performed using UPF and RAN time information in GTP-U packets (see clause 5.33.3.2 of 3GPP TS 23.501 [2]). 6.1.6.2.64 Type: QosMonitoringInfo Table 6.1.6.2.64-1: Definition of type QosMonitoringInfo Attribute name; Data type; P; Cardinality; Description. qosMonitoringInd; boolean; C; 0..1; This IE shall be present and set to "true" if QoS monitoring activated for QoS flows of the PDU session is performed using end to end accumulated packet delay reporting in UL GTP-U packets (see clause 5.33.3.3 of 3GPP TS 23.501 [2]). - True: QoS monitoring is performed using end to end accumulated packet delay reporting in UL GTP-U packets (see clause 5.33.3.3 of 3GPP TS 23.501 [2]). - False (default): QoS monitoring is performed using UPF and RAN time information in GTP-U packets (see clause 5.33.3.2 of 3GPP TS 23.501 [2]). When this attribute is present and set to "true", the I-SMF shall provision the I-UPF to report an end to end accumulated packet delay in UL GTP-U packets as specified in 3GPP TS 29.244 [29].. However, assuming this IE is set to true, after an I/V-SMF change or insertion procedure triggered, the target I/V-SMF has no idea that GTP-U Path QoS monitoring shall be activated in the target I/V-UPF and shall use GTP-U path based QoS monitoring for the concerning QoS flow. So, the IE "" shall be populated to the target I/V-SMF during an I/V-SMF insertion or change procedure in the SmContext, so that when the QosMonitoringInfo is set to true, the target I/V-SMF shall: - Select an I/V-UPF supporting QoS Monitoring for packet delay for a QoS flow using GTP-U based solution, i.e. the selected I/V-UPF shall support the UP function feature for GPQM; - Instruct the selected I/V-UPF to activate GTP-U path monitoring as specified in clause 5.24.5.2 of 3GPP TS 29.244 if the GTP-U path QoS monitoring towards the target NG-RAN has not activated; - instruct the selected I/V-UPF to activate GTP-U path based QoS monitoring for the QoS flow(s) of the PDU session indicated in the qosFlowsList as specified in clause 5.24.5.3 of 3GPP TS 29.244. |
| Summary of change: | Introduce the IE "QosMonitoringInfo" in the SmContext. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | GTP-U path based QoS monitoring for packet delay for QoS flow(s) of a PDU session will not work after an I/V-SMF insertion or change procedure due to UE mobility. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.2.2.6.1, 6.1.6.2.39, A.2 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: | This CR introduces backwards compatible corrections to the following APIs: TS29502_Nsmf_PDUSession.yaml |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: | Rev1: Change WI code to 5G_URLLC, the proposed changes are applied to Rel-17, so the CAT is changed to "A". Remove the changes in PduSessionCreatedData and HsmfUpdatedData. A few editorial changes. |  |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.2.2.6	Retrieve SM Context service operation

##### 5.2.2.6.1	General

The Retrieve SM Context service operation shall be used to retrieve an individual SM context, for a given PDU session, from the (H-)SMF, from the V-SMF during change or removal of V-SMF, or from the I-SMF during change or removal of I-SMF.

It is used in the following procedures:

-	5GS to EPS handover using N26 interface (see clause 4.11.1.2.1 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	5GS to EPS Idle mode mobility using N26 interface (see clause 4.11.1.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	UE Triggered Service Request with I-SMF insertion/change/removal or with V-SMF insertion/change/removal (see clause 4.23.4.3 of of 3GPP TS 23.502 [3]);

-	Xn based inter NG-RAN handover with insertion of intermediate SMF (see clause 4.23.11 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	Inter NG-RAN node N2 based handover, preparation phase, with I-SMF or V-SMF insertion/change (see clause 4.23.7.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions (see clause 4.3.5.2 of 3GPP TS 23.502 [3]);

-	Network triggered EAS change in HR-SBO context (see clause 6.7.3.2 of 3GPP TS 23.548 [74]);

-	N2 Handover with V-SMF insertion/change/removal in HR-SBO case (see clause 6.7.2.6 of 3GPP TS 23.548 [39]);

-	Inter V-SMF mobility registration update procedure in HR-SBO case (see clause 6.7.2.7 of 3GPP TS 23.548 [39]);

-	Xn Handover with V-SMF change in HR-SBO case (see clause 6.7.2.9 of 3GPP TS 23.548 [39]).

The NF Service Consumer (e.g. AMF or SMF) shall retrieve an SM context by using the HTTP POST method (retrieve custom operation) as shown in Figure 5.2.2.6.1-1.

Figure 5.2.2.6.1-1: SM context retrieval

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context to be retrieved. The POST request may contain a content with the following parameters:

-	target MME capabilities, if available, to allow the SMF to determine whether to include EPS bearer contexts for Ethernet PDN Type, non-IP PDN type, or requiring UP integrity protection or not;

-	SM context type:

-	indicating that this is a request to retrieve the complete SM context (i.e. 5G SM context including EPS context information as defined in clause 6.1.6.2.39), during scenarios with an I-SMF or V-SMF insertion/change/removal or SMF Context Transfer procedure; or

-	indicating that this is a request to retrieve the AF Coordination Information as defined in clause 6.1.6.2.69, during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled.

-	serving core network operator PLMN ID of the new V-SMF, when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change or insertion. Or the serving core network operator PLMN ID of the new I-SMF during the procedure with an I-SMF insertion;

-	notToTransferEbiList IE, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, to request the SMF to not transfer EPS bearer context(s) corresponding to EBIs in the list, during an 5GS to EPS mobility when the target MME does not support 15 EPS bearers;

-	ranUnchangedInd IE, if the NG-RAN Tunnel info is required in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover, I-SMF selection/removal per DNAI or V-SMF selection per DNAI, when the UE is in CM-CONNECTED state as specified in clauses 5.2.2.2.7 and 5.2.2.2.12;

-	hrsboSupportInd IE, if the procedure is triggered by a target V-SMF supporting the HR-SBO feature;

-	storedOffloadIds IE, if the target V-SMF has a list of stored offload identifiers from the previous procedures for the HPLMN of the PDU session.

2a.	On success, "200 OK" shall be returned; the content of the POST response shall contain the mapped EPS bearer contexts if this is a request for the UE EPS PDN connection, or the complete SM context if this is a request for retrieving the complete SM context, or the AF Coordination Information if this is a request for retrieving the AF Coordination Information.

If this is a request for the UE EPS PDN connection and the target MME capabilities were provided in the request parameters:

-	if the target MME supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Unstructured", an EPS bearer context with the "non-IP" PDN type;

-	if the target MME supports the Ethernet PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "Ethernet" PDN type;

-	if the target MME does not support the Ethernet PDN type but supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "non-IP" PDN type.

If the notToTransferEbiList IE was included in the request, the SMF shall not provide EPS bearer context(s) corresponding to EBIs in the list.

If this is a request for retrieving the complete SM context and there are downlink data packets buffered at I-UPF, the SMF shall include the "forwardingInd" attribute with value "true" in the response body to indicate downlink data packets are buffered at the I-UPF. The NF Service Consumer receiving the "forwardingInd" attribute with the value "true" shall setup a forwarding tunnel for receiving the buffered downlink data packets.

If this is a request for retrieving the complete SM context for an inter-PLMN V-SMF change, i.e. if the request contains the serving core network operator PLMN ID indicating a different PLMN than the PLMN of the SMF (acting as the old V-SMF), the latter shall not include the chargingInfo IE and the roamingChargingProfile IE in the SM context returned in the response.

During a procedure with an I-SMF or V-SMF insertion, the anchor SMF should use the servingNetwork IE received in the Retrieve SM Context Request to determine whether the inserted entity is an I-SMF or V-SMF, and if so, encode in the SM Context returned in the response the applicable set of attributes (e.g. hsmfUri, hSmfInstanceId, hSmfServiceInstanceId to a V-SMF, or smfUri, smfInstanceId, smfServiceInstanceId to an I-SMF) and the applicable URI in the pduSessionRef if different URIs are used for intra-PLMN and inter-PLMN signaling requests targeting the PDU session context.

NOTE 1:	During an inter-PLMN procedure with an I-SMF or V-SMF change, the old V-SMF or I-SMF returns the attributes of the SM context as were received from the anchor SMF.

If the UE, target MME and AMF support User Plane integrity protection with EPS, the SMF shall include the UP Security Policy IE in the UE EPS PDN connection context if User Plane integrity protection has been enabled by the SMF as specified in clauses 4.11.1.2.1 and 4.11.1.3.2 of 3GPP TS 23.502 [3].

NOTE 2:	During a 5GS to EPS handover, if User Plane Integrity Protection is used in 5GS and QoS flows are associated with EPS bearer IDs, the I-SMF/V-SMF considers that the UE supports User Plane Integrity Protection in EPS.

If this is a request for retrieving the complete SM context for an I-SMF or V-SMF insertion, and the smfUri IE or hSmfUri IE is provided by the AMF in the Create SM Context request and is different from the smfUri IE or hSmfUri IE in the SM context returned in the Retrieve SM Context response, the latter (i.e. the IEs received in the Retrieve SM Context response) shall prevail and be used by the I-SMF or V-SMF to trigger the create service operation to the (H-)SMF.

If the target V-SMF supports the HR-SBO feature, for a HR-SBO session, the content of the POST response message may also contain the HR-SBO related information, e.g., hrsboAuthResult, hDnsAddr, hPlmnAddr, vplmnOffloadingInfoList.

The source V-SMF shall determine if the target V-SMF has already received the corresponding VPLMN Offloading Information for an offload identifier based on the storedOffloadIds for the HPLMN of the PDU session provided by the target V-SMF in the Retrieve SM context request message for an intra PLMN V-SMF change, the source V-SMF shall include:

-	the Offload Identifier(s) only, if the Offload Identifier(s) are known by the target V-SMF, i.e., the Offload Identifier(s) are part of the storedOffloadIds; otherwise

-	a list of vplmnOffloadingInfo attributes where each vplmnOffloadingInfo contains an offload identifier and the corresponding VPLMN Offloading Information.

If this is a request for retrieving the complete SM context during an I/V-SMF insertion or change procedures, and if the qosMonitoringInd is set to true, the target I/V-SMF shall:

-	select an I/V-UPF supporting QoS Monitoring for packet delay for a QoS flow using GTP-U based solution, i.e. the selected I/V-UPF shall support the UP function feature for GPQM as specified in clause 5.24.5.1 of 3GPP TS 29.244 [29], if possible;

-	instruct the selected I/V-UPF to activate GTP-U path monitoring as specified in clause 5.24.5.2 of 3GPP TS 29.244 [29] if the GTP-U path QoS monitoring towards the target NG-RAN has not been activated yet;

-	instruct the selected I/V-UPF to activate GTP-U path based QoS monitoring for the QoS flow(s) of the PDU session indicated in the QosFlowSetupItem as specified in clause 5.24.5.3 of 3GPP TS 29.244 [29].

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.4.4.2-2.

If the EBI value of the QoS Flow associated with the default QoS Rule is included in the notToTransferEbiList IE, the SMF shall set the "cause" attribute in the ProblemDetails structure to "DEFAULT_EBI_NOT_TRANSFERRED".

If a request for the UE EPS PDN connection is rejected due to the target MME not being capable to support the PDU session, e.g. if the PDU session requires UP integrity protection but the target MME does not support User Plane Integrity Protection with EPS, the SMF shall return a 403 Forbidden response with the "cause" attribute in the ProblemDetails structure set to " TARGET_MME_CAPABILITY".

# *** Next Change ***

##### 6.1.6.2.39	Type: SmContext

Table 6.1.6.2.39-1: Definition of type SmContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionId | PduSessionId | M | 1 | This IE shall contain the PDU Session ID. |  |
| dnn | Dnn | M | 1 | This IE shall contain the UE requested DNN of the PDU session. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | M | 1 | This IE shall contain the S-NSSAI for the serving PLMN. |  |
| altSnssai | Snssai | C | 0..1 | This IE shall be present if the network slice for the serving PLMN (as indicated in the sNssai IE) was replaced by an alternative S-NSSAI. When present, this IE shall indicate the alternative S-NSSAI for the serving PLMN. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a HR PDU session, or a non-roaming PDU session if received from the AMF. When present, it shall contain the S-NSSAI for the HPLMN. |  |
| altHplmnSnssai | Snssai | C | 0..1 | This IE shall be present if the network slice for the HPLMN (as indicated in the hplmnSnssai IE) was replaced by an alternative S-NSSAI for the HPLMN. When present, this IE shall indicate the alternative S-NSSAI for the HPLMN. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| pduSessionType | PduSessionType | M | 1 | This IE shall indicate the PDU session type. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios. When present, it shall contain the API URI of the Nsmf_PDUSession service of the H-SMF. The API URI shall be formatted as specified in clause 6.1.1. |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the API URI of the Nsmf_PDUSession service of the SMF. The API URI shall be formatted as specified in clause 6.1.1. |  |
| pduSessionRef | Uri | C | 0..1 | This IE shall be present for a HR PDU session or a PDU session with an I-SMF. When present, this IE shall include the absolute URI of the PDU Session in H-SMF or SMF, including apiRoot (see clause 6.1.3.6.2) |  |
| interPlmnApiRoot | Uri | C | 0..1 | This IE shall be present, if available. When present, it shall contain the apiRoot of the PDU session context to be used in inter-PLMN signalling request targeting the PDU session context. (NOTE 2) |  |
| intraPlmnApiRoot | Uri | C | 0..1 | This IE shall be present, if available. When present, it shall contain the apiRoot of the PDU session context to be used in intra-PLMN signalling request targeting the PDU session context. (NOTE 2) |  |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |  |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if the pcfId IE is present. When present, it shall contain the NF Set ID of the PCF indicated by the pcfId IE. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 1) |  |
| sessionAmbr | Ambr | M | 1 | This IE shall contain the Session AMBR granted to the PDU session. |  |
| qosFlowsList | array(QosFlowSetupItem) | M | 1..N | This IE shall contain the set of QoS flow(s) established for the PDU session. It shall contain at least the Qos flow associated to the default Qos rule. The qosRules attribute of each QosFlowSetupItem shall be set to an empty string. |  |
| hSmfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the home SMF. |  |
| smfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the SMF. |  |
| pduSessionSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the home SMF as identified by hSmfInstanceId, or the SMF as identified by the smfInstanceId. |  |
| pduSessionSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this PDU session) in the home SMF or the SMF. |  |
| pduSessionSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the PDU session resource in the home SMF or the SMF. |  |
| enablePauseCharging | boolean | C | 0..1 | This IE shall be present for a HR PDU session, if available. When present, it shall indicate whether the use of Pause of Charging is enabled for the PDU session (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: enable Pause of Charging; - false (default): disable Pause of Charging. |  |
| ueIpv4Address | Ipv4Addr | C | 0..1 | This IE shall be present if a UE IPv4 address to the PDU session. |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if a UE IPv6 prefix to the PDU session. |  |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| maxIntegrityProtectedDataRate | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for uplink. If the maxIntegrityProtectedDataRateDl IE is absent, this IE applies to both uplink and downlink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE may be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for downlink. |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether this is an always On PDU session and it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. |  |
| hSmfServiceInstanceId | string | O | 0..1 | This IE may be present for a HR PDU session. When present, this IE shall contain the serviceInstanceId of the H-SMF service instance serving the PDU session. This IE may be used by the V-SMF to identify PDU sessions affected by a failure or restart of the H-SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| smfServiceInstanceId | string | O | 0..1 | This IE may be present for a PDU session with an I-SMF. When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session. This IE may be used by the I-SMF to identify PDU sessions affected by a failure or restart of the SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| recoveryTime | DateTime | O | 0..1 | This IE may be present if available. When present, this IE shall indicate the timestamp (in UTC) when the H-SMF or SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| forwardingInd | boolean | C | 0..1 | This IE shall be present, when downlink data packets are buffered at I-UPF. The SMF or I-SMF shall use this IE to inform the NF service consumer that a forwarding tunnel is needed for receiving the buffered downlink data packets, as specified in clause 4.23.4 of 3GPP TS 23.502 [3]. When present, this IE shall be set as follows: - true: a forwarding tunnel is needed for sending buffered downlink data packets; - false (default): forwarding tunnel is not needed |  |
| psaTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the N9 tunnel information of PDU Session Anchor UPF controlled by SMF or H-SMF. |  |
| chargingId | string | C | 0..1 | This IE shall be present for a HR PDU session, in scenarios with a V-SMF insertion/change/removal. When present, it shall contain the Charging ID of the PDU session (see 3GPP TS 32.255 [25]). The string shall encode the Charging ID (32-bit unsigned integer value, with maximum value "4294967295") in decimal representation. Pattern: '^(0¦([1-9]{1}[0-9]{0,9}))$' (NOTE 4) |  |
| smfChargingId | SmfChargingId | C | 0..1 | This IE shall be present if available for a HR PDU session, in scenarios with a V-SMF insertion/change/removal. When present, it shall contain the String based Charging ID of the PDU session (see 3GPP TS 32.255 [25]). | SCID |
| chargingInfo | ChargingInformation | C | 0..1 | This IE shall be present for a HR PDU session, if available and if the NF Service Consumer requesting the SM Context pertains to the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID). When present, it shall contain the addresses of the V-CHF used for the PDU session. |  |
| roamingChargingProfile | RoamingChargingProfile | C | 0..1 | This IE shall be present for a HR PDU session, if available and if the NF Service Consumer requesting the SM Context pertains to the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID). When present, it shall contain the Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| nefExtBufSupportInd | boolean | C | 0..1 | This IE shall be present with value "true", if the anchor NEF has indicated support of Extended Buffering for mobile terminated data during SMF-NEF connection establishment. When present, this IE shall be set as following: - true: Extended Buffering supported by NEF - false (default): Extended Buffering not supported by NEF |  |
| ipv6Index | IpIndex | C | 0..1 | This IE shall be present during I-SMF change scenarios, if IPv6 Index has previously been received by old I-SMF. |  |
| dnAaaAddress | IpAddress | O | 0..1 | When present, this IE shall contain the address of DN-AAA server for UE IP Address allocation previously received by old I-SMF. |  |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if this information has been received previously from the UE, the anchor SMF or the old I-SMF. |  |
| ranTunnelInfo | QosFlowTunnel | C | 0..1 | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the N2 tunnel information of NG-RAN with associated QoS flows (see "DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| addRanTunnelInfo | array(QosFlowTunnel) | C | 1..N | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for split PDU session (see "Additional DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| redRanTunnelInfo | QosFlowTunnel | C | 0..1 | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for Redundant QoS Flow(s) (see "Redundant DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| addRedRanTunnelInfo | array(QosFlowTunnel) | C | 1..N | This IE shall be present if the ranUnchangedInd IE is set to "true" in the SM context retrieve request. When present, this IE shall contain the additional N2 tunnel information of NG-RAN together with associated QoS flows for Redundant QoS Flow(s) with split PDU session (see "Additional Redundant DL QoS Flow per TNL Information" in clause 9.3.4.2 of 3GPP 38.413 [9]). |  |
| nspuSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the enablePauseCharging in the SmContext data type is set to "true" and if the (H-)SMF and PSA UPF support Notify Start Pause of Charging via user plane feature as specified in clause 5.30 of 3GPP TS 29.244 [29]. When present, it shall be set as follows: - true: Notify Start Pause of Charging via user plane feature is supported. |  |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the PDU session resource in the home SMF or the SMF and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. |  |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | When present, this IE shall indicate the satellite backhaul category information last signalled towards the anchor SMF, if any. |  |
| sscMode | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall indicate the SSC mode applicable to the PDU session. When present, it shall be encoded as one character in hexadecimal representation, taking a value of "0" to "7", representing the 3 bits of the SSC mode value of the SSC mode IE specified in clause 9.11.4.16 of 3GPP TS 24.501 [7]. Pattern: "^[0-7]$" Example: SSC mode 3 shall be encoded as "3". |  |
| dlsetSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the (H-)SMF supports the "DLSET" feature as specified in clause 6.1.8. When present, it shall be set as follows: - true: the (H-)SMF supports the "DLSET" feature. - false: the (H-)SMF does not support the "DLSET" feature |  |
| n9fscSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if the SMF supports the "N9FSC" feature as specified in clause 6.1.8. When present, it shall be set as follows: - true: "N9FSC" feature is supported. |  |
| anchorSmfOauth2Required | boolean | O | 0..1 | This IE may be present when the NF consumer (i.e. new I-SMF or new V-SMF) and the NF producer (i.e. the old I-SMF, V-SMF or SMF) belong to the same PLMN. When present, this IE shall indicate whether the H-SMF or SMF for a PDU session with an I-SMF requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the anchor SMF's Nsmf_PDUSession service. (NOTE 3) |  |
| fullDnaiList | array(Dnai) | O | 1..N | This IE may be present to contain the full list of DNAIs of interest for PDU session, including DNAIs that may not be supported by the (source) I-SMF and excluding the ones supported by the Anchor SMF. | DTSSA-Ext1 |
| hrsboAuthResult | boolean | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, it shall Indicate whether HR-SBO request is authorized - true: authorized. - false: Not authorized. | HR-SBO |
| hDnsAddr | IpAddress | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, this IE shall contain the DNS server address of HPLMN. | HR-SBO |
| hPlmnAddr | IpAddress | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, this IE shall contain the HPLMN address information (e.g. H-UPF IP address on N6). The new V-SMF may configure the new V-EASDF to build EDNS Client Subnet option based on this HPLMN address information for target FQDN of DNS queries which are not authorized for HR-SBO. | HR-SBO |
| vplmnOffloadingInfoList | array(VplmnOffloadingInfo) | C | 1..N | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, it shall contain the list of V-PLMN Offloading policies that apply to the PDU session and whose offload identifiers are not yet known by the target V-SMF. (NOTE 5) | HR-SBO |
| vplmnDlAmbr | VplmnDlAmbr | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. When present, it shall contain the Authorized DL Session AMBR for Offloading for the V-PLMN. | HR-SBO |
| offloadIds | array(OffloadIdentifier) | C | 1...N | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO, and if the offloadIds are part of the storedOffloadIds included in the the Retrieve SM Context Request. This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, this IE shall contain a list of offload identifiers that apply to the PDU session and that are already known by the target V-SMF/I-SMF. (NOTE 5) | HR-SBO, ISMF-LOM |
| easInfoToRefresh | EasInfoToRefresh | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the EAS information to be refreshed for EAS re-discovery. | HR-SBO, ISMF-LOM |
| targetDnai | Dnai | C | 0..1 | This IE shall be present, for a HR PDU session, if it is received from H-SMF in AF triggered EAS re-discovery and edge relocation via interacting with HPLMN (see clause 6.7.3.2 of 3GPP TS 23.548 [74]). When present, this IE shall contain the target DNAI. | HR-SBO |
| trafficInfluInfo | TrafficInfluenceInfo | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the traffic influence information applicable at the VPLMN for the HR-SBO PDU session, or applicable to the PDU session with I-SMF. | HR-SBO, ISMF-LOM |
| pendingUpdateInfoList | array(PendingUpdateInfo) | O | 1..N | This IE should be included by the old V-SMF/I-SMF if received from the (H-)SMF. When present, this IE shall indicate the list of information that are not required to be updated in real-time to the (H-)SMF, i.e. the change of the listed information (e.g. UE location or Timezone) may be piggybacked in a subsequent essential update (e.g. to exchange the N1 message from the UE) to the (H-)SMF. The NF service consumer (i.e. I-SMF/V-SMF) should not trigger an Update to the (H-)SMF including only the change(s) of the listed information. |  |
| localOffloadingMgtAllowedInd | boolean | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall Indicate whether I-SMF based local offloading management was allowed by the SMF - true: allowed - false: not allowed | ISMF-LOM |
| dnsAddr | IpAddress | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, this IE shall contain the DNS server address to be used for DNS requests related with traffic not to be subject to Local Offloading Management. | ISMF-LOM |
| psaUpfAddr | IpAddress | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, this IE shall contain the PSA UPF address information (e.g. PSA UPF IP address on N6) to be used by the I-SMF to configure the EASDF to build EDNS Client Subnet option for target FQDN of DNS queries which are not authorized for Local Offloading Management. | ISMF-LOM |
| localOffloadingInfoList | array(LocalOffloadingManagementInfo) | C | 1..N | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the Local Offloading Information list applicable to the PDU session and whose offload identifiers are not yet known by the target I-SMF. (NOTE 5) | ISMF-LOM |
| priorityUserInd | boolean | C | 0..1 | This IE shall be present during intra VPLMN V-SMF change scenario, if previously received from the AMF or from old V-SMF. Presence of this IE with the value false shall be prohibited. (NOTE 6) |  |
| qosMonitoringInfo | QosMonitoringInfo | C | 0..1 | This IE shall be present if QoS monitoring for packet delay has been activated for at least one QoS flow of the PDU session indicated in the QosFlowSetupItem. See also clause 5.2.2.6.1). |  |
| NOTE 1: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 2: See NOTE 7 of Table 6.1.6.2.10-1. NOTE 3: If the anchorSmfOauth2Required IE was received in SmContextCreateData from the AMF, this IE shall be ignored by the new I-SMF or V-SMF. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE 5: The same offloadId should not appear in both the vplmnOffloadingInfoList/localOffloadingInfoList and offloadIds attributes. Both vplmnOffloadingInfoList/localOffloadingInfoList and offloadIds attributes may be present when multiple offload identifers apply to the PDU session and some of them are already known by the target V-SMF/I-SMF while others are not yet known by the target V-SMF/I-SMF. NOTE 6: For a HR PDU Session, if the VPLMN QoS Constraints is enabled and the VPLMN QoS Constraints for high priority PDU sessions and normal PDU sessions are different, the V-SMF shall determine the VPLMN QoS Constraints applicable for this PDU session based on the priority user indication. |  |  |  |  |  |

# *** Next Change ***

# A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.4.0-alpha.5'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.502 V19.4.0; 5G System; Session Management Services; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.502/

servers:

- url: '{apiRoot}/nsmf-pdusession/v1'

variables:

apiRoot:

default: https://example.com

description: >

apiRoot as defined in clause 4.4 of 3GPP TS 29.501. The sm-contexts and pdu-sessions

resources can be distributed on different processing instances or hosts. Thus the

authority and/or deployment-specific string of the apiRoot of the created individual

sm context and pdu-session resources' URIs may differ from the authority and/or

deployment-specific string of the apiRoot of the sm-contexts and pdu-sessions

collections' URIs.

******skipped for clarity*******

SmContext:

description: Complete SM Context

type: object

properties:

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altHplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

interPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

intraPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

pduSessionSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

pduSessionSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

forwardingInd:

type: boolean

default: false

psaTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

chargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

smfChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmfChargingId'

chargingInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

nefExtBufSupportInd:

type: boolean

default: false

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

ranTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

redRanTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRedRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

nspuSupportInd:

type: boolean

smfBindingInfo:

type: string

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

sscMode:

type: string

pattern: '^[0-7]$'

dlsetSupportInd:

type: boolean

n9fscSupportInd:

type: boolean

anchorSmfOauth2Required:

type: boolean

fullDnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

hrsboAuthReqInd:

type: boolean

hDnsAddr:

$ref: '#/components/schemas/IpAddress'

hPlmnAddr:

$ref: '#/components/schemas/IpAddress'

vplmnOffloadingInfoList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VplmnOffloadingInfo'

minItems: 1

vplmnDlAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VplmnDlAmbr'

offloadIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OffloadIdentifier'

minItems: 1

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

trafficInfluInfo:

$ref: '#/components/schemas/TrafficInfluenceInfo'

pendingUpdateInfoList:

type: array

items:

$ref: '#/components/schemas/PendingUpdateInfo'

minItems: 1

easInfoToRefresh:

$ref: '#/components/schemas/EasInfoToRefresh'

localOffloadingMgtAllowedInd:

type: boolean

dnsAddr:

$ref: '#/components/schemas/IpAddress'

psaUpfAddr:

$ref: '#/components/schemas/IpAddress'

localOffloadingInfoList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LocalOffloadingManagementInfo'

minItems: 1

priorityUserInd:

type: boolean

enum:

- true

qosMonitoringInfo:

$ref: '#/components/schemas/QosMonitoringInfo'

required:

- pduSessionId

- dnn

- sNssai

- pduSessionType

- sessionAmbr

- qosFlowsList

******skipped for clarity*******

# *** End of Changes ***