3GPP TSG-CT WG4 Meeting #132	C4-255302

Dallas, US; 17th – 21st November 2025	was C4-255172

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.564 | CR | 0154 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for a subscription containing AppIds or Traffic filters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  |  | 2025-11-19 |  |
| Category: | F |  |  |  |  |  |  | Release: |  |  |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |
| Reason for change: | It is not clear, for the event types User Data Usage Measures and User Data Usage Trends, when the UPF receives a subscription request containing either Application ID(s) or Traffic filters, whether the UPF needs to validate that these Application ID(s) or Traffic filters are included in the PDR(s) provisioned for the target PDU session/PFCP session, i.e. the corresponding application traffic is authorized to be forwarded and if the traffic can be detected, before the UPF can accept the subscription. This needs to be clarified. |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | Add the following clarifications: When the UPF receives a subscription request containing either Application ID(s) or Traffic filters, the UPF should accept the subscription regardless of if these Application ID(s) or Traffic filters are included in the PDR(s) provisioned for the target PDU session/PFCP session. If the corresponding application traffic is not be detected , the UPF may report a "null" measurement if it can not be skipped (see also clause 5.2.2.3.3) for a subscription with a Periodic Report type. The style of the text "- Reporting suggestion information (i.e. Report urgency, Reporting time information)" is corrected. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Incomplete requirements lead to interoperability problems. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.2.1.3.3, 5.2.1.3.4, 5.2.2.2.2, 5.2.2.3.1, 6.1.6.2.8, A2 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: | This CR introduces backwards compatible correction to the following openAPI file: TS29564_Nupf_EventExposure.yaml |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: | Rev1: Move the proposed note into the procedure description, clarify the null measurement. |  |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 5.2.1.3.3	User Data Usage Measures

Table 5.2.1.3.3-1: User Data Usage Measures event

| Description | This event provides information of user data usage of a PDU session: - Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurements. - Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurements. - Application related information: URL(s) and/or Domain information (domain name and protocol) detected for the target traffic. This Type of Measurement requires that Application Id(s) or Traffic Filtering Information is provided (i.e. this measurement is not possible to be applied for all traffic handled by the UPF). |
| --- | --- |
| Subscription type | Subscription via SMF using Nupf_EventExposure Subscribe, if the target is: - PDU session(s) of a specific UE or a group of UEs; (NOTE 2) or - PDU session(s) of "any UE" and the subscription includes at least one of the following parameters: AoI, BSSID/SSID and DNAI. (NOTE 1) Subscription to the UPF, if the target is PDU session(s) of "any UE" or a specific PDU session identified by a UE IP address (see clause 4.15.4.5.5 of 3GPP TS 23.502 [3]), and the subscription does not need to include any of the following parameters: AoI, BSSID/SSID and DNAI. |
| Subscription inputs to UPF | Required: - UE IP address (for an IP PDU session type), SUPI (for a non-IP PDU session type) or "Any UE" - Type of Measurement (i.e. Volume, Throughput, Application related information) - UPF event consumer notification URI - Notification correlation ID Optional: - DNN - S-NSSAI - either Application ID(s) or Traffic filters - Granularity of Measurement (i.e. required granularity for the information reported, i.e. per PDU session, per data flow or per application) - Reporting suggestion information (i.e. Report urgency, Reporting time information) - Skip Reporting Instruction - inclRatType - ratTypeList |
| Report type | One-Time Report Periodic Report |
| NOTE 1: If the SMF receives a subscription request targeting PDU session(s) of any UE, the SMF shall not invoke Nupf_EventExposure Subscribe towards PDU sessions for which it is acting as I-SMF. NOTE 2: When the SMF receives a subscription targeting a group of UEs, it shall map the target group to all impacted UEs and separately invoke Nupf_EventExposure Subscribe towards PDU sessions for each UE. |  |

# *** Next Change ***

##### 5.2.1.3.4	User Data Usage Trends

Table 5.2.1.3.4-1: User Data Usage Trends event

| Description | This event provides statistics related to user data usage of a PDU session: - Throughput Statistic Measurement (average and/or peak throughput) over the measurement period determined for the requested Granularity of Measurements. |
| --- | --- |
| Subscription type | Subscription via SMF using Nupf_EventExposure Subscribe, if the target is: - PDU session(s) of a specific UE or a group of UEs; (NOTE 2) or - PDU session(s) of "any UE" and the subscription includes at least one of the following parameters: AoI, BSSID/SSID and DNAI. (NOTE 1) Subscription to the UPF, if the target is PDU session(s) of "any UE" or a specific PDU session identified by a UE IP address (see clause 4.15.4.5.5 of 3GPP TS 23.502 [3]), and the subscription does not need to include any of the following parameters: AoI, BSSID/SSID and DNAI. |
| Subscription inputs to UPF | Required: - UE IP address (for an IP PDU session type), SUPI (for a non-IP PDU session type) or "Any UE" - UPF event consumer notification URI - Notification correlation ID Optional: - DNN - S-NSSAI - either Application ID(s) or Traffic filters - Granularity of Measurement (i.e. required granularity for the information reported, i.e per PDU session, per data flow or per application) - Reporting suggestion information (i.e. Report urgency, Reporting time information) - Skip Reporting Instruction - inclRatType - ratTypeList |
| Report type | One-Time Report Periodic Report |
| NOTE 1: If the SMF receives a subscription request targeting PDU session(s) of any UE, the SMF shall not invoke Nupf_EventExposure Subscribe towards PDU sessions for which it is acting as I-SMF. NOTE 2: When the SMF receives a subscription targeting a group of UEs, it shall map the target group to all impacted UEs and separately invoke Nupf_EventExposure Subscribe towards PDU sessions for each UE. |  |

# *** Next Change ***

##### 5.2.2.2.2	Creation of a subscription

An NF Service Consumer shall invoke the Subscribe service operation towards the UPF to create a subscription to monitor at least one UPF event. The NF Service Consumer may subscribe to multiple events in a subscription. A subscription may be associated with one UE's PDU session or with any UE.

The NF Service Consumer shall request to create a new subscription by using the HTTP method POST with the URI of the subscriptions collection, see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Subscription creation

1.	The NF Service Consumer shall send a POST request to create a subscription resource in the UPF. The content of the POST request shall contain a representation of the individual subscription resource to be created.

The NF Service Consumer shall include the following information in the HTTP message body:

-	NF ID, indicating the identity of the network function instance creating the subscription;

-	Target of Event Reporting, indicating the target(s) to be monitored, i.e.

-	a specific PDU Session of a UE identified with a (5GC assigned) UE IP address for an IP PDU session type;

-	a specific PDU Session of a UE identified with a SUPI (and S-NSSAI/DNN) for a non-IP PDU session type if the SMF allows the UPF to receive the SUPI associated with a N4 session based on the local configuration; or

-	any UE (identified by the "anyUE" flag);

-	List of UPF events requested to be subscribed;

-	Type of measurement, for UPF events supporting multiple types of measurement, e.g. for a subscription to the UserDataUsageMeasures event;

-	Event Reporting Mode, indicating how the events shall be reported (One-time Report, Periodic Report or Continuous Report);

-	UPF event consumer notification URI, indicating the address where to send the event notifications generated by the subscription;

-	Public IPv4 or IPv6 address of the remote end, for a subscription to "UE_NAT_MAPPING_INFO" event; and

-	the bundledEventNotifyUri set to the URI to be used by the UPF for sending notification requests including bundled event reports of multiple UPF event subscriptions (see clause 5.2.2.3.4), if the bundlingAllowed IE is set to true and the NF service consumer provides a distinct eventNotifyUri per UPF event subscription.

The NF Service Consumer may include the following information in the HTTP message body:

-	The S-NSSAI and/or the DNN of PDU sessions to which the subscription applies;

-	either one or more Application ID(s) or traffic filters identifying the traffic to be monitored by the subscription;

-	Granularity of Measurement, indicating that the granularity of the required measurements is per PDU Session, per data flow or per application;

-	Reporting period, defining the period for periodic reporting;

-	Skip Reporting Instruction, instructing the UPF to skip sending event reports when certain conditions are met as described in clause 5.2.2.3.3;

-	Maximum number of reports, defining the maximum number of reports after which the event subscription ceases to exist;

-	Expiry time, suggested by the NF Service Consumer representing the time up to which the subscription is desired to be kept active and the time after which the subscribed event(s) shall stop generating reports;

-	Reporting suggestion information, i.e. Report urgency indicating whether the event report can be delayed (i.e. it is delay-tolerant) and if so, the Reporting time information defining the last valid reporting time for the UPF to report the detected event;

-	Deactivate notification flag, indicating that the notification of the available events shall be muted until the event consumer NF (e.g. NWDAF or DCCF) provides the retrieval notification flag to retrieve the stored events;

-	Immediate Report Flag per event, indicating an immediate report to be generated with the current event status;

-	Notification Correlation ID, indicating the correlation identity to be signalled in the event notifications generated by the subscription;

-	Sampling ratio, defining the random subset of PDU sessions among target PDU sessions, in which case the UPF shall only report the event(s) related to the selected subset of PDU sessions;

-	partitioningCriteria, defining the criteria for partitioning PDU sessions before applying the sampling ratio;

-	Muting Exception Instructions, which specify instructions to apply to the subscription and the stored events when an exception occurs at the UPF while the event is muted (e.g., the buffer of stored event reports is full, or the number of stored event reports exceeds a certain number), if the EEMM feature is supported (see clause 6.1.8);

-	Port number of the remote end, for a subscription to "UE_NAT_MAPPING_INFO" event;

-	IP domain, for a subscription to "UE_NAT_MAPPING_INFO" event;

-	the subTerminationReportInd IE set to true to indicate a request to send a notification when the subscription is terminated;

-	the remainingDataReports IE per subscribed event requesting to discard or to send the remaining data for the related event to the NF service consumer, if the subscription is terminated at the UPF (e.g. when the PFCP session of the PDU session that is the target of the subscription is released) and the UPF has not yet reported the collected data for the related event to the consumer;

-	the bundlingAllowed IE set to true to allow the UPF to bundle event reports generated for this subscription with event reports generated for other subscriptions in a same Nupf_EventExposure_Notify request message (see clause 5.2.2.3.4), if the UPF supports the BERMS feature (see clause 6.1.8); and/or

-	the bundleId IE containing a Bundle Identifier identifying the subscriptions whose event reports may be bundled together (see clause 5.2.2.3.4), if the bundlingAllowed IE is set to true and if the NF service consumer only allows to bundle event reports of specific subscriptions, e.g. if the NWDAF wishes to only bundle event reports generated for a same analytic service request.

-	the inclRatType IE per subscribed event requesting the UPF to include the RAT type in the corresponding event reports (i.e. in NotificationItem).

-	the RatTypeList, which contains a list of RAT type for which the NF service consumer require to receive the event reports from the PDU session(s) of the UE(s) camping on these RAT type(s), and if received, the UPF may skip sending measurement reports for PDU session(s) of UE(s) not camping on these RAT types.

NOTE:	If the UPF does not skip sending measurement reports for UEs not camping on the RAT type(s) requested in the subscription, the NF service consumer can use the RatType IE included in the event reports to filter measurements reported for different RAT types than those requested in the subscription.

2a.	On success (i.e. if the request is accepted), the UPF shall include a HTTP Location header to provide the location of the newly created resource (subscription) together with the status code 201 in the response message indicating that the requested resource is created.

If the NF Service Consumer has included more than one events in the event subscription and some of the events cannot be subscribed, the UPF shall accept the request and provide the successfully subscribed event(s) in the CreatedEventSubscription.

If the subscription request contains either Application ID(s) or Traffic filters, the UPF should accept the subscription regardless of whether these Application ID(s) or Traffic filters are included in the PDR(s) provisioned for the target PDU session/PFCP session. If no corresponding application traffic is detected, the UPF shall report a "null" measurement if it cannot be skipped (see also clause 5.2.2.3.3) for a subscription with a Periodic Report type.

If the NF Service Consumer has included the Immediate Report Flag with the value true in the event subscription, and if the current status of the events subscribed are available, the UPF shall include the current status of the events subscribed in the response. Otherwise, the UPF shall generate reports for the events and notify the NF service consumer using the Nupf_EventExposure_Notify service operation. If the events with the Immediate Report Flag set to true are subscribed via an SMF, the notification shall be sent to the actual NF service consumer directly, i.e. the current status of the events subscribed shall not be included in the subscription creation response.

If the NF Service Consumer has set the event reporting option to ONE_TIME and if the UPF has included the current status of the events subscribed in the response, then the UPF shall not do any subsequent event notification for the corresponding events.

The response, based on operator policy and taking into account the expiry time included in the request, may contain the expiry time, as determined by the UPF, after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the UPF. The UPF shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

If the sampling ratio ("sampRatio") attribute is included in the subscription without a partitioningCriteria, the UPF shall select a random subset of PDU sessions among target PDU sessions according to the sampling ratio and only report the event(s) related to the selected subset of PDU sessions. If the partitioningCriteria attribute is also included along with sampling ratio, the UPF shall apply the sampling ratio on the group of PDU sessions determined according to the partitioning criteria.

If the "notifFlag" attribute is included and set to "DEACTIVATE" in the request by e.g. the NWDAF or DCCF, the UPF shall mute the event notification and store the available events. Additionally, if the UPF supports the EEMM feature (see clause 6.1.8) and if the NF service consumer includes event muting instructions in the request, the UPF should evaluate the received event muting instructions against to local actions (if configured) and, if the subscription creation request is accepted, the UPF may indicate the following information to the NF service consumer in the response:

-	the maximum number of notifications that the UPF expects to be able to store for the subscription;

-	an estimate of the duration for which notifications can be buffered.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

If the UPF supports the EEMM features (see clause 6.1.8), the NF service consumer sets the "notifFlag" attribute to "DEACTIVATE" and event muting instructions in the request, but the UPF cannot accept the received instructions, the UPF may reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

For a subscription request targeting a PDU session, if the UPF cannot find a unique PDU session due to no DNN and/or S-NSSAI being received in the request, the UPF shall reject the request with a 403 Forbidden response and the application error "REJECTION_DUE_TO_NO_DNN_SNSSAI" (see clause 4.4.1.2 of 3GPP TS 23.502 [3]).

# *** Next Change ***

#### 5.2.2.3	Notify

##### 5.2.2.3.1	General

The Notify service operation shall be invoked by the UPF, to send a notification, towards the notification URI, when certain event included in the subscription has taken place. See Figure 5.2.2.3.2-1.

The Notify service operation shall also be invoked by the UPF if the event exposure subscription is terminated at the UPF (e.g. when the PFCP session of the PDU session that is the target of the subscription is released) and the subscription includes the subTerminationReportInd IE set to true (indicating a request to send a notification when the subscription is terminated).

The Notify service operation may also be invoked if the event exposure subscription is terminated at the UPF (e.g. when the PFCP session of the PDU session that is the target of the subscription is released) to report any data that has been collected but not yet sent to the consumer, if the subscription included the remainingDataReports IE for subscribed events requesting to send the remaining data for the related events to the NF service consumer and/or based on local configuration (see clauses 4.15.4.5.6 and 4.15.4.5.7 of 3GPP TS 23.502 [3]).

NOTE 1:	The Notification request message does not indicate that an event report is sent due to the remainingDataReports set in the subscription.

For the events "USER_DATA_USAGE_MEASURES", "USER_DATA_USAGE_TRENDS" and "UE_NAT_MAPPING_INFO", the UPF shall use the HTTP method POST, using the notification URI received in the subscription creation as specified in clause 5.2.2.2.2, including e.g. the subscription ID, Event ID(s) for which event has happened, notification correlation ID provided by the NF service consumer at the time of event subscription, to send a notification.

If the subscription is targeting PDU sessions of any UE, i.e. the "anyUe" is set to true in the subscription creation request, the UPF shall perform the requested measurements for every PDU session that matches the event filter information (i.e. S-NSSAIs, DNNs, either Application ID(s) or traffic filters) and send notification(s) with multiple NotificationItem IEs within the NotificationData wherein each NotificationItem shall correspond to a report on one subscribed event per PDU session. If the subscription request included a sampling ratio, the notification may include the sampling ratio achieved by the UPF.

For the events "QOS_MONITORING", "TSC_MNGT_INFO" or "HANDLING_OF_PAYLOAD_HEADERS_INFO", the UPF shall use the HTTP method POST, using the notification URI received from the SMF via N4 interface, see clause 5.33.5 of 3GPP TS 29.244 [15].

For the event "USER_DATA_USAGE_MEASURES", the event notification may contain following information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurements;

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurements;

-	Application related Information: URLs and/or Domain information (Domain name and protocol) detected in the target traffic identified by the information included in the subscription request, e.g. an application id.

When the granularity of the measurement is per data flow, the notification shall include the packet filter set and the Applications Identifier if available.

The UPF shall report null measurements, i.e. set the corresponding VolumeMeasurement and/or ThroughputMeasurement to "0", or include a noApplRelatedInfoDet IE (indicating no application related information detected) in the ApplicationRelatedInformation, if the UPF does not detect any corresponding user data traffic for the subscribed Granularity of Measurement and the UPF is requested to send a notification to report the event, e.g. when the report type is set to "periodic".

For the event "USER_DATA_USAGE_TRENDS", the event notification may contain following information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement determined for the requested Granularity of Measurements.

When the granularity of the measurement is per data flow, the notification shall include the packet filter set and the Applications Identifier if available.

The UPF shall report null measurements, i.e. set the corresponding ThroughputStatisticsMeasurement to "0" if the UPF doesn't detect any corresponding user data traffic for the subscribed Granularity of Measurement and the UPF is requested to send a notification to report the event, e.g. when the report type is set to "periodic".

For the event "QOS_MONITORING", this service operation is used by the UPF to send the following types of event notifications:

-	periodic notification on the downlink packet delay, uplink packet delay, and/or the round trip packet delay between the UPF (PSA) and UE;

-	event triggered notification on the downlink packet delay, uplink packet delay, and/or the round trip packet delay between the UPF (PSA) and UE, i.e. when the packet delay exceeds a defined threshold;

-	notification on the downlink packet delay, uplink packet delay, and/or the round trip packet delay between the UPF (PSA) and UE when the PDU session is released.

-	event triggered notification of congestion information of the QoS flow on the UL and/or DL directions received from the NG-RAN, upon a change of the congestion information.

-	event triggered notification of available bitrate of the GBR QoS flow on the UL and/or DL directions received from the NG-RAN.

For the event "TSC_MNGT_INFO", the event notification may contain the following information:

-	Port Management Information Container(s) for one or more NW-TT ports and/or

-	a User Plane Node Management Information Container.

The event notification shall also contain the following information:

-	the related NW-TT port number(s), if Port Management Information Container(s) is present; and

-	the notification correlation ID received from the SMF, if any.

For the event " UE_NAT_MAPPING_INFO ", this service operation is used by the UPF to send the following types of event notifications:

-	one-time immediate report, when in the subscription the event reporting mode is set to "ONE_TIME" and the indication for immediate reporting is set to true;

-	event triggered notification on the change of the information for (NATed) Public UE IP addresses and/or port(s) assigned to the PDU Session;

-	if no UE NAT mapping exists yet, the UPF sends an empty array to the consumer as part of the UPF event subscription response if the immediate reporting flag is included, otherwise the empty array is sent in the event subscription notify.

For the event "HANDLING_OF_PAYLOAD_HEADERS_INFO", this service operation is used by the UPF to send the following types of event notifications:

-	event triggered notification:

-	when in the subscription the OTRI (One Time Report Indication) bit is set to "1" in the Header Handling Reporting Indication the UPF reports the applied action only for the first occurrence within a packet flow;

-	when in the subscription the OTRI bit is not present the UPF reports the applied actions for each occurrence within a packet flow.

The event notification for the event "HANDLING_OF_PAYLOAD_HEADERS_INFO" shall also contain the following information:

-	the detected header for which an action was applied;

-	the header value prior to the action, when the action is an detection, replacement, or deletion, this is to record the header's initial state before any modifications;

-	the action applied on the header, specifying the operation conducted, i.e. detection, insertion, replacement, or deletion; and

-	the header value after the action is applied, this is to capture the final state of the header. This includes any new values introduced during an insertion, the updated values resulting from a replacement. The headerValueAfter shall not apply for a deletion or a detection.

NOTE 2:	When the action applied is set to "insert", "replace" or "remove", the "detect" action will be implicitly always required therefore it needs not be reported.

For the event "SUBSCRIPTION_TERMINATION", the event notification may contain following information:

-	cause indicating the reason for the subscription being terminated (e.g. N4 session release).

# *** Next Change ***

##### 6.1.6.2.8	Type: ApplicationRelatedInformation

Table 6.1.6.2.8-1: Definition of type ApplicationRelatedInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| urls | array(Uri) | O | 1..N | This IE may be present if available. When present, it shall contain a list of URLs detected in the traffic identified by the information included in the subscription request, e.g. an application id. |
| domainInfoList | array(DomainInformation) | O | 1..N | This IE may be present if available. When present, it shall contain a list of Domain information detected in the traffic identified by the information included in the subscription request, e.g. an application id. |
| noApplRelatedInfoDet | boolean | C | 0..1 | This IE shall be present and set to true if no application related information is detected, i.e. neither urls nor domainInfoList is present. - true: no application related information is detected Presence of this IE with the value false shall be prohibited. |

# *** Next Change ***

# A.2	Nupf_EventExposure API

openapi: 3.0.0

info:

title: 'UPF Event Exposure Service'

version: 1.2.0-alpha.4

description: |

UPF Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.564 V19.4.0; 5G System; User Plane Function Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.564/

******Skipped for clarity******

ApplicationRelatedInformation:

description: Application Related Information

type: object

properties:

urls:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

domainInfoList:

type: array

items:

$ref: '#/components/schemas/DomainInformation'

minItems: 1

noApplRelatedInfoDet:

type: boolean

enum:

- true

******Skipped for clarity******

# *** End of Changes ***