3GPP TSG-CT WG4 Meeting #131	C4-254268

# Sophia Antipolis, France; 13th – 17th October 2025						revision of C4-254057

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.527 | CR | 0101 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing reference due to incomplete implementation of an agreed CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, MCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reference in the last step of the procedure in Figure 8.3.4-1 is missing. The refernce was added in the agreed CR C4-241365 (TS 23.527 CR#0077), but the CR was not compeletely implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing reference in the last step of the procedure in Figure 8.3.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. The reference in the figure that are depicting a procedure are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: The category changed to A. The change is implemented from Rel-18, because the original agreed CR was a Rel-18 CR. |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

### 8.3.4	Multicast MBS session restoration Procedure for NG-RAN failure with or without restart

The procedure specified in this clause may be supported to restore a Multicast MBS session affected by an NG-RAN failure with or without restart.

When a UPF detects the NG-RAN failure with or without restart using Echo Request and Echo Response message which is sent towards the IP address in the DL F-TEID allocated by the failed NG-RAN, or upon receiving GTP-U Error Indication, it shall report such event to the SMF to enable the SMF to derive UEs who have joined the MBS sessions which are affected by the NG-RAN failure and initiates the restoration procedure for each of Multicast MBS session which were activated in the failed NG-RAN as described in Figure 8.3.4-1.

When unicast transport is used over N3mb interface for 5GC Shared MBS traffic delivery, the MB-UPF detects the NG-RAN failure with or without restart using Echo Request and Echo Response message which is sent towards the IP address in the DL F-TEID allocated by the failed NG-RAN, or upon receiving GTP-U Error Indication, it shall report such event to the MB-SMF, so that the MB-SMF removes the DL-TEID allocated by the failed NG-RAN.

Figure 8.3.4-1: Multicast MBS session restoration upon NG-RAN failure with or without restart

1.	The NG-RAN has failed with or without restart.

2.	The UPF receive GTP-U Error Indication when the NG-RAN recovers from its restart for 5GC Individual MBS traffic delivery.

2a. The MB-UPF receives GTP-U Error Indication when the NG-RAN recovers from its restart for unicast transport over N3mb for 5GC Shared MBS traffic delivery.

3.	The UPF sends Echo Request message towards the NG-RAN using the IP address included in the DL F-TEID, thus the UPF detects the NG-RAN has failed with or without restart.

3a.  The MB-UPF sends Echo Request message towards the NG-RAN using the IP address included in the DL F-TEID for unicast transport over N3mb, thus the MB-UPF detects the NG-RAN has failed with or without restart.

4.	The UPF sends PFCP Session Report Request messages to the SMF to report receiving of GTP-U Error Indication message, or sends a PFCP Node Report Request message to the SMF to report the GTP-U path failure towards the NG-RAN or the remote GTP-U entity has restarted.

5.	The SMF sends PFCP Session Modification Request message to request the UPF to remove DL-FTEID allocated by the failed NG-RAN and buffer the MBS session data for 5GC Individual MBS traffic delivery, except for an NG-RAN restart if the UPF has already removed the DL F-TEID on its own as specified in clause 5.5.

6.	The MB-UPF sends PFCP Session Report Request messages to the MB-SMF to report receiving of GTP-U Error Indication message, or sends a PFCP Node Report Request message to the MB-SMF to report the GTP-U path failure towards the NG-RAN or the remote GTP-U entity has restarted, for unicast transport for N3mb.

7.	The MB-SMF sends PFCP Session Modification Request message to request the MB-UPF to remove DL-FTEID allocated by the failed NG-RAN, except for an NG-RAN restart if the MB-UPF has already removed the DL F-TEID on its own as specified in clause 5.5.

8a.	The SMF may wait for UEs who have joined the MBS session(s) which are affected by the NG-RAN failure to trigger a Service Request procedure to re-establish the user plane resource and then restore the Multicast MBS session in NG-RAN.

8b.	Alternatively, the SMF may derive UEs who have joined the MBS session(s) which are affected by the NG-RAN failure (i.e. with a DL F-TEID matching the IP address of the NG-RAN node's to restore the MBS service and perform step 9.

9.	When step 8b applies, the SMF continues with the MBS session activation procedure as specified in clause 7.2.5.2 of 3GPP TS 23.247 [12] starting from step 3 to restore the MBS sessions for affected UEs for each MBS session.

In the case of an NG-RAN restart, if the MB-SMF receives a Distribution Setup Request from the NG-RAN while it has not yet removed the corresponding N3mb DL F-TEID in the MB-UPF (e.g. because step 6 has not taken place yet), the MB-SMF shall replace the N3mb DL F-TEID currently provisioned in the MB-UPF (for the corresponding MBS session and NG-RAN) by the new N3mb DL F-TEID received in the Distribution Setup Request.

* * * End of Changes * * * *